3GPP TSG-RAN WG4 Meeting # 115                                                 R4-2506010

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | Draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.104 to introduce conducted out-of-band blocking for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conducted out-of-band blocking requirements for SBFD capable BS need to be introduced in TS 38.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the conducted out-of-band blocking requirements of SBFD capable BS, the general requirements are introduced in clause 12.3.5.1, the minimum requirements are introduced in clause 12.3.5.2, and the minimum requirements for co-location are introduced in cause 12.3.5.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conducted out-of-band blocking requirements for SBFD capable BS are missing in the TS 38.104. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

12.3.5	Out-of-band blocking for SBFD capable BS

12.3.5.1 General

The out-of-band blocking characteristics for SBFD capable BS is a measure of the receiver ability to receive a wanted signal at its assigned channel at the TAB connector for SBFD capable BS type 1-H in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

12.3.5.2 Minimum requirement for SBFD capable BS type 1-H

Except for band n46, n96, n102 and n104, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to SBFD capable BS type 1-H TAB connector using the parameters in table 12.3.5.2-1.

The reference measurement channel for the wanted signal is identified in clause 12.3.2 for each SBFD capable BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal are further specified in annex D.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for SBFD capable BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the TAB connector for SBFD capable BS type 1-H.

Table 12.3.5.2-1: Out-of-band blocking performance requirement for SBFD

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -15 | CW carrier |
| NOTE 1: PREFSENS depends on the RAT. For SBFD, PREFSENS depends also on the SBFD capable BS channel bandwidth as specified in Table 12.3.2-1, 12.3.2-2, and 12.3.2-3. |  |  |

12.3.5.3 Co-location minimum requirements for SBFD capable BS type 1-H

This additional blocking requirement may be applied for the protection of SBFD capable BS receivers when GSM, CDMA, UTRA, E-UTRA, NR [or SBFD capable BS] operating in a different frequency band are co-located with a SBFD capable BS. The requirement is applicable to all BS channel bandwidths supported by the SBFD capable BS.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and SBFD capable BS receiver and are based on co-location with base stations of the same class.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to SBFD capable BS type 1-H TAB connector input using the parameters in table 12.3.5.3-1 for all the SBFD capable BS classes. The reference measurement channel for the wanted signal is identified in tables 12.3.2-1, 12.3.2-2 and 12.3.2-3 for each SBFD capable BS channel bandwidth and further specified in annex A.1.

The blocking requirement for co-location with BS in other bands is applied for all operating bands for which co-location protection is provided.

Minimum conducted requirement is defined at the TAB connector for SBFD capable BS type 1-H.

Table 12.3.5.3-1: Blocking performance requirement for SBFD capable BS when co-located with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the SBFD capable BS channel bandwidth as specified in Table 12.3.2-1, 12.3.2-2, and 12.3.2-3. NOTE 2: x = -7 dBm for NR BS co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR BS co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR BS co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). NOTE 4: For unsynchronized base stations (except in band n46, n96 and n102), special co-location requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |  |

<End of Change>