3GPP TSG-RAN WG4 Meeting # 115                                                              R4-2506014

Malta, MT, 19th – 23rd May, 2025

Title: 	                 Discussion on system parameters for A-IoT

Source: 	CATT

Agenda item:	7.26.2

Document for:	Discussion

- Introduction

This contribution provides our further analysis for R2D and D2R system parameters according to the WF [1].

- Discussion

2.1 System parameter for R2D

- 2.1.1 R2D channel bandwidth

R2D channel bandwidth can reuse the channel bandwidth of NB-IoT standalone mode, i.e. 200kHz/1RB, 400kHz/2RB, 600kHz/3RB, 800kHz/4RB. The channel bandwidth, transmission bandwidth configuration, and guardband for different number of RBs are shown in figure 1 and table2.

- 1RB

- 2RB

- 3RB

- 4RB

Figure 1: Definition of Channel Bandwidth and Transmission Bandwidth Configuration for R2D

Table 1: Channel bandwidth and Transmission bandwidth configuration for R2D

| Transmission bandwidth configuration NRB | 1RB | 2RB | 3RB | 4RB |
| --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) | 200 | 400 | 600 | 800 |
| Transmission bandwidth configuration Ntone 15kHz | 12 | 24 | 36 | 48 |
| Total guardband (kHz) | 20 | 40 | 60 | 80 |

Proposal 1: The channel bandwidth, transmission bandwidth configuration and total guardband for different number of RBs are shown in figure 1 and table 1.

If the guardbands are asymmetric, for 1RB, the guardband adjacent to the low channel edge is 2.5kHz and the guardband adjacent to the high channel edge is 17.5kHz, as shown in figure 2. If the guardbands are symmetric, both guardbands are 10kHz. The asymmetric guardbands and symmetric guardbands for different number of RBs are shown in figure 2 and table2.

- NRB mode 2=1, 1RB

- NRBmode2=0, 2RB

Figure 2: Asymmetric guardbands and symmetric guardbands for R2D channel bandwidth

Table 2: Asymmetric guardbands and symmetric guardbands for R2D channel bandwidth

| Transmission bandwidth configuration NRB |  | 1RB | 2RB | 3RB | 4RB |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) |  | 200 | 400 | 600 | 800 |
| Transmission bandwidth configuration Ntone 15kHz |  | 12 | 24 | 36 | 48 |
| Asymmetric guardbands | Guardbandlow (kHz) | 2.5 | 12.5 | 22.5 | 32.5 |
|  | Guardbandhigh (kHz) | 17.5 | 27.5 | 37.5 | 47.5 |
| Symmetric guardbands | Guardbandlow and Guardbandhigh (kHz) | 10 | 20 | 30 | 40 |

It can be seen from table 1 that if the two guardbands are asymmetric, guardbands adjacent to the high channel edge are 15kHz larger than the guardbands adjacent to the low channel edge. The design of digital filters is generally symmetrical, asymmetric will degrade filter performance, and this is more obvious when the number of RB is small. Therefore, it is preferable to have symmetric guardbands.

Proposal 2: The guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

Proposal 3: The guardbands for different number of RBs are shown in table 2.

- 2.1.2 R2D channel raster

The channel raster design should guarantee flexible placement of the A-IoT RB, and the A-IoT RB should be aligned with the NR RB. To do A-IoT in NR bands with 100 kHz channel raster, the channel raster of R2D should be 10kHz.

Proposal 4: The channel raster of R2D should be 10kHz.

Table 3: R2D channel raster to RE mapping

| Transmission bandwidth configuration NRB |  | 1RB | 2RB | 3RB | 4RB |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) |  | 200 | 400 | 600 | 800 |
| Transmission bandwidth configuration Ntone 15kHz |  | 12 | 24 | 36 | 48 |
| Channel raster to RE mapping | Resource element index | subcarrier #6 | subcarrier #0 | subcarrier #6 | subcarrier #0 |
|  | Physical resource block index | RB#0 | RB#0 | RB#0 | RB#0 |
| Note 1: If NRBmod2=0, resource element index=0, if NRBmod2=1, resource element index=6. Note 2: Physical resource block index=NRB2. Note 3: Subcarrier index for each RB is subcarrier #0~ subcarrier #11. |  |  |  |  |  |

Proposal 5: RAN4 to specify R2D channel raster to RE mapping are shown in table 3.

2.2 System parameter for D2R

Define D2R channel bandwidth for A-IoT device need to consider 2SB, D2R maximum data rate, SFO, FDMA and filter performance. We can decide the channel bandwidth after RAN1 has conclusions on D2R maximum data rate and SFO.

If the small frequency shift is controllable and determinable, then the D2R channel bandwidth is a fixed value,  If the small frequency shift is within a range, then the D2R channel bandwidth needs to consider the range of the frequency shift on the basis of a fixed value.

If the small frequency shift is controllable and determinable, the D2R channel bandwidth remains a fixed value, independent of whether the frequency shift occurs. However, if the small frequency shift varies within a certain range, the D2R channel bandwidth must also account for the range of the shift.

Proposal 6: We can decide the channel bandwidth of D2R after RAN1 has conclusions on D2R maximum data rate and SFO.

Proposal 7: If the small frequency shift is controllable and deterministic, the D2R channel bandwidth remains a fixed value. However, if the small frequency shift varies within a certain range, the D2R channel bandwidth must also account for the range of the shift.

2.3 Other issue

The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified.

Proposal 8: The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified.

- Summary

This contribution provides our further analysis for A-IoT system parameters. We have the following observation and proposals.

Proposal 1: The channel bandwidth, transmission bandwidth configuration and total guardband for different number of RBs are shown in figure 1 and table 1.

Proposal 2: The guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

Proposal 3: The guardbands for different number of RBs are shown in table 2.

Proposal 4: The channel raster of R2D should be 10kHz.

Proposal 5: RAN4 to specify R2D channel raster to RE mapping are shown in table 3.

Proposal 6: We can decide the channel bandwidth of D2R after RAN1 has conclusions on D2R maximum data rate and SFO.

Proposal 7: If the small frequency shift is controllable and deterministic, the D2R channel bandwidth remains a fixed value. However, if the small frequency shift varies within a certain range, the D2R channel bandwidth must also account for the range of the shift.

Proposal 8: The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified.

Reference

[1] R4-2505230, “WF on A-IoT device requirements”, CMCC, RAN4#114bis

[2] TR 38.769