3GPP TSG-RAN WG4 Meeting #115                                                                                                      R4-2506022

Malta, MT, 19th –23rd May, 2025

Source: 	CATT

Title: 	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Agenda item:	7.11.6

Document for:	Approval

- Introduction

In RAN4#114bis meeting, A WF [1] on Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF was agreed, and remaining open issues in WF require further discussion.

In this contribution, we provide our views on SAN RF core requirements for Ku-band for NR NTN.

- Discussion

Requirements for FR1-NTN Ku-band and FR2-NTN Ku-band

As per WF [1], the remaining issue concerning requirements for FR1-NTN Ku-band and FR2-NTN Ku-band is copied as below:

#### Issue 2-1-1: General issues for Ku band SAN RF requirements Agreement: - FFS on whether to introduce a new SAN type for FR1-NTN Ku bands which is based on SAN type 2-O RF requirements.

The SAN type 1-O in TS 38.108 is defined as below:

SAN type 1-O: 	Satellite Access Node operating at FR1-NTN with a requirement set consisting only of OTA requirements defined at the RIB.

The definition of SAN type 2-O in TS 38.108 is missing, but it can be defined as below:

SAN type 2-O: 	Satellite Access Node operating at FR2-NTN with a requirement set consisting only of OTA requirements defined at the RIB.

Based on above definition, the straightforward method is that SAN type 1-O is for FR1-NTN Ku-band and SAN type 2-O is for FR2-NTN Ku-band, but from perspective of SAN type, the SAN type for Ku-band is not straightforward, at same time, the requirement for Ku-band NTN is not apparent. So we define SAN type 1-2-O as below:

SAN type 1-2-O: Satellite Access Node operating at overlapping frequency range of FR1-NTN and FR2-NTN supporting FR1-NTN and FR2-NTN numerologies with a requirement set consisting only of OTA requirements defined at the RIB.

So if we define the SAN type 1-2-O, the requirements for Ku-band NTN in TS spec become clearer and existing requirement will not be impacted.

Based on above, we propose,

Proposal 1: Introduce SAN type 1-2-O for Ku-band NTN as below:

SAN type 1-2-O: Satellite Access Node operating at overlapping frequency range of FR1-NTN and FR2-NTN supporting FR1-NTN and FR2-NTN numerologies with a requirement set consisting only of OTA requirements defined at the RIB.

As above proposal, if we introduce SAN type 1-2-O for Ku-band, the requirement for SAN type 1-2-O for Ku-band can be defined independently.

Proposal 2: If introducing SAN type 1-2-O for Ku-band, the requirement for SAN type 1-2-O for Ku-band can be defined independently.

EIRP accuracy and TRP accuracy

As per WF[1], the agreement and remaining issues concerning EIRP accuracy and TRP accuracy for Ku-band is as below:

#### Issue 2-2-1: The tolerance for the radiated transmit power and SAN output power Agreement: - For the tolerance of the radiated transmit power and SAN output power for both the FR1-NTN and FR2-NTN Ku bands - [+-3.4] dB EIRP accuracy, [+-3] dB TRP accuracy (re-using FR2-NTN) as baseline - Other options are not precluded - Other NTN bands will not be impacted

If we introduce SAN type 1-2-O for Ku-band, we think that the EIRP accuracy and TRP accuracy for Ku-band can be stricter than that for Ka-band, we can analyse them as following.

The existing EIRP accuracy and TRP accuracy in normal conditions for FR1-NTN and FR2-NTN are summarized in the following table.

| Frequency range designation | Corresponding frequency range (MHz) | EIRP accuracy(dB) | TRP accuracy(dB) |
| --- | --- | --- | --- |
| FR1-NTN | 410 – 7125 | 2.2 | 2 |
| FR2-NTN | 17300 – 30000 | 3.4 | 3 |

Referring to TR 37.842 and TR 37.843, the factors [2] which influence EIRP and TRP accuracy are but not limited to the following:

1) Transceiver unit accuracy;

2) Array accuracy, including sub-array gain accuracy, mismatch errors and insertion loss variations;

3) Steer accuracy, including phase shift accuracy among transceiver units, and phase shifter accuracy.

So, EIRP accuracy and TRP accuracy can be expressed as equation 1 and equation 2 respectively.

[dB]                                                                                                       (Equation 1)

[dB]                                                                                                             (Equation 2)

Using Equation 1, the Steer accuracy for FR1-NTN and FR2-NTN can be calculated as 0.9dB and 1.6dB, respectively. From our understanding, it is reasonable to define the Array accuracy for FR1-NTN and FR2-NTN as 0.5dB and 0.8dB respectively. Using Equation 2 with TRP accuracy 2dB for FR1-NTN and 3dB for FR2-NTN in above table, the transceiver unit accuracyfor FR1-NTN and FR2-NTN can be calculated as 1.8dB and 2.9dB respectively. So the transceiver unit accuracy, Array accuracy, and Steer accuracy for FR1-NTN and FR2-NTN are listed in the following table.

| Frequency range designation | Array accuracy(dB) | Transceiver unit accuracy(dB) | Steer accuracy(dB) |
| --- | --- | --- | --- |
| FR1-NTN | 0.5 | 1.8 | 0.9 |
| FR2-NTN | 0.8 | 2.9 | 1.6 |

The frequency range for Ku-band is 10700 MHz – 14500MHz, the DL frequency is about 11GHz, and 11GHz is between higher frequency of FR1-NTN and lower frequency of FR2-NTN, so comparing to FR1-NTN, it is reasonable to improve transceiver unit accuracy, Array accuracy, and Steer accuracy for Ku-band to make Ku-band implementation to unrestrictive. From our understanding, the transceiver unit accuracy, Array accuracy, and Steer accuracy in the following table is applicable for Ku-band.

| Frequency range | Array accuracy(dB) | Transceiver unit accuracy(dB) | Steer accuracy(dB) |
| --- | --- | --- | --- |
| Ku-band 10700 MHz – 14500MHz | [0.7] | [2.1] | [1.2] |

So, using Equation 2 with above transceiver unit accuracy, Array accuracy, the TRP accuracy for Ku-band can be calculated as sqrt(0.7*0.7+2.1*2.1)= 2.2dB. At same time, using Equation 1 with 2.2dB TRP accuracy and 1.2dB steer accuracy, the EIRP accuracy for Ku-band can be calculated as sqrt(2.2*2.2+1.2*1.2)=2.5dB. So for frequency in Ku-band, [2.5]dB EIRP accuracy and [2.2]dB TRP accuracy is more applicable.

Proposal 3:  Define [2.5]dB EIRP accuracy and [2.2]dB TRP accuracy for Ku-band if introducing SAN type 1-2-O for Ku-band.

- Conclusion

This contribution provides analysis on SAN RF core requirements for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Introduce SAN type 1-2-O for Ku-band NTN as below:

SAN type 1-2-O: Satellite Access Node operating at overlapping frequency range of FR1-NTN and FR2-NTN supporting FR1-NTN and FR2-NTN numerologies with a requirement set consisting only of OTA requirements defined at the RIB.

Proposal 2: If introducing SAN type 1-2-O for Ku-band, the requirement for SAN type 1-2-O for Ku-band can be defined independently.

Proposal 3:  Define [2.5]dB EIRP accuracy and [2.2]dB TRP accuracy for Ku-band if introducing SAN type 1-2-O for Ku-band.

- References

- R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#114bis

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#114

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#114

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107