3GPP TSG-RAN WG4 Meeting #115		R4-2506064

St. Julian, Malta, May. 19th – 23rd, 2025

Title:	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Response to:	R2-2401572

Release:	Release 19

Work Item:	LTE_NR_R19_Simult_RxTx-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:

Name:	Aijun CAO

E-mail Address:	ajcao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 thanks RAN2 for the reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx and sharing RAN2 agreements and questions on for the example band the 2UL/2DL inter-band FDD-FDD CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL.

For carrier aggregation involving a low-low band combination where the UL of one band partially overlaps with the DL of the other band, such as CA_n5-n8, RAN4 conducted studies in Rel-18 on enhancements for 700/800/900MHz band combinations. The studies concluded that three options are feasible for implementing the CA_n5-n8 band combination, as detailed in TR 38.872.:

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

- Option 1: Only support 1UL/2DL CA. Single UL in n5

- Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

2) Dedicated RF filters implementation with partial frequency range

- Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL without any scheduling restriction on both bands.

In particular, Option 1 has already been specified in Rel-18, while RAN4 has agreed in RAN4#112bis to specify only Option 2 to support full band frequency range of each band in Rel-19 (See WF R4-2417125) with Option 3 excluded in Rel-19 which enables simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions.

Moreover, standardizing Option 2 in Release 19 may necessitate a future-proof signalling design to accommodate the introduction of simultaneous operation capabilities for additional NR FDD-FDD inter-band CA combinations. These combinations may involve partial UL and DL overlap between different FDD bands or face duplexer-related challenges, as requested in the original liaison statement R4-2420410.

Additionally, RAN4 has continued the discussion with regards to reply to RAN2 where several potential solutions for this band combination with new issues identified for Option 2, e.g., interrupt needed for RF front end configuration change, and PA ON/OFF etc, therefore, several solutions are discussed, each representing a different level of improvement.

- Solution #1 – switching by activation and deactivation:

  - If UE reports support of 2UL for CA_n5-n8, network only activate either n5DL or n8UL at a time

  - Change of cases is done in such a way that to no extra interrupts/switching time is needed

    - Staying in Case 1 (n5DL+n8DL/n5UL) longer than Case 2 (n5DL/n5UL+n8UL)

  - RAN2 capability signaling is required

- Solution #2 - New capability: introduce a UE capability of switching gap (i.e., switching period and location) to cover necessary switching period including RFFE configuration switching, PA On/Off etc.,

  - Can be the same for both directions (n5DL -> n8UL, n8UL->n5DL), or one for each direction

  - Network accommodates the switching gap by scheduling

    - Cross-carrier scheduling might be needed (e.g., from Case 2 with only 1 DL n8DL to Case 1 with two DL)

  - No RAN1 spec change is required

- Solution #3 - Further improvement, e.g., under L-L CA via switching framework

  - Not considered in Rel-19

  - Not precluded in Rel-20 or later

RAN4 has decided to support Solution #1 in Rel-19, where the required RAN2 capability signaling aligns with the original request. The RAN2 capability indicates two independent concurrent capabilities for a band combination consisting of two FDD bands, referred to as band nA and band nB:

- Capability #1: A value of “1” indicates concurrent nA uplink and nB downlink operation; a value of “0” indicates that nA-UL and nB-DL are non-concurrent.

- Capability #2: A value of “1” indicates concurrent nA downlink and nB uplink operation; a value of “0” indicates that nA-DL and nB-UL are non-concurrent.

If 2UL is supported for the band combination CA_n5-n8, the two capabilities should be set to (1,0).

Regarding the questions raised, in-line answers could be found below:

- Whether it’s possible for a UE to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restriction, or whether there may be a case where a UE can support simultaneous Rx/Tx in some part(s) of the overlapping FDD bands? If possible, from which release version?

Answer 1:      If only Option 2 is assumed, it becomes impossible to implement simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without introducing scheduling restrictions. Furthermore, a UE would not be able to support simultaneous Rx/Tx in any portion of the overlapping FDD bands.

However, as noted above, a UE implemented with Option 3 can support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 without any scheduling restrictions, or enable simultaneous Rx/Tx in specific portions of the overlapping FDD bands. Nevertheless, RAN4 has excluded Option 3 in Rel-19.

- If a UE may be able to support simultaneous Rx/Tx in inter-band FDD-FDD CA_n5-n8 (or some other inter-band FDD-FDD CA combinations) in some form, whether it requires additional UE capabilities for the support/requirement of other features, e.g., full-duplex, switching time, or some frequency gap/guard band?

Answer 2:     This can be further discussed when deciding to specify Option 3 in Rel-20 or later.

# 2	Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take the above information into account and introduce the requested capability signalling design for supporting CA_n5-n8 with 2UL/2DL.

# 3	Dates of next TSG-RAN WG4 meetings

3GPP TSG RAN WG4#116	August 25th – 29th, 2025			Bangalore, India