3GPP TSG RAN WG4 Meeting #115		R4-2506066

St Julian, Malta, May 19th – 23rd , 2025

Title: 	Further discussion on UE RF requirements for low band CA via switching

Agenda Item:	7.7.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#114bis, a good progress has been made regarding the UE RF requirements for low band CA via switching with a WF approved [1]:

# Way Forward Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded Issue 5: Test methodology - Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

In this contribution, we discuss how to specify the Tx on-off time mask requirements in RAN4 specs and potentially how to test the requirements accordingly.

# 2 Discussion

Tx on-off time mask

As shown in Fig. 1, uplink transmission at the FDD band only happens in Case 1 while downlink of the FDD band is received at the same time, and in Case 2, both Tx and Rx of the FDD band is switched off, and only Rx of the SDL band is switched on, hence, three types of time masks are involved:

- Tx on-off time mask at the FDD band

- Rx on-off time mask at the FDD band: since Tx and Rx of the FDD band are tied together in time domain, Rx on-off time mask at the FDD band shall be identical to Tx on-off time mast at the same FDD band.

- Rx on-off time mask at the SDL band

Fig. 1, RFE configuration for carrier switching [2]

Since no cross-carrier scheduling is assumed for low band CA via switching, the first OFDM symbol of the switch-to carrier should not be impacted by the switching since it could be part of CORESET#0, the switching period should always be placed at the switch-from carrier/band.

Proposal 1: Switching period is always located at the switch-from band.

Under this proposal, the Tx on-off time mask for the FDD band can be shown as Fig. 2.

Fig. 2, Tx on-off time mask for band n5

In addition, the Rx on-off of the FDD band should be exactly identical to the Tx on-off of the FDD band, and the Rx on-off of the SDL band should be exactly revert to the Tx on-off of the FDD band. In short words, these three types of on-off time masks should be defined clearly for the operation of low band CA via switching.

Proposal 2: RAN4 to define the following three types of on-off time masks for low band CA via switching: Tx on-off time mask for the FDD band, Rx on-off time mask for the FDD band and Rx on-off time mask for the SDL band.

Test methodology for low band CA via switching

According to the above discussion, testing of low band CA via switching involves testing the following three types of time masks:

- Test of Tx on-off mask at the FDD band: ON to OFF, and OFF to ON

- Test of Rx on-off mask at the FDD band:

  - when Tx of the FDD band is off, then Rx of the FDD band is off

  - when Tx of the FDD band is on, then Rx of the FDD band is on

- Test of Rx on-off mask at the SDL band:

For 1), principles or procedures of testing transient period could be referenced, and no new test methodology is required. However, for 2), Tx and Rx of the FDD band are coupled together in time domain, testing Rx operation to be linked to Tx at the same band is a new issue. For 3), similar to 2), but in a reverse way and in another band, i.e., if Tx of the FDD band is ON, then Rx of the SDL band is OFF, else Rx of the SDL band is ON.

Proposal 3: Studies on new test methodologies are required to test the Rx on-off mask to be identical to Tx on-off mask at the same band, or the Rx on-off mask is reverse to Tx on-off mask at a different band in time domain.

# 3 Conclusion

In this contribution, we have the following proposals on UE RF requirements for low band CA via switching:

Proposal 1: Switching period is always located at the switch-from band.

Proposal 2: RAN4 to define the following three types of on-off time masks for low band CA via switching: Tx on-off time mask for the FDD band, Rx on-off time mask for the FDD band and Rx on-off time mask for the SDL band.

Proposal 3: Studies on new test methodologies are required to test the Rx on-off mask to be identical to Tx on-off mask at the same band, or the Rx on-off mask is reverse to Tx on-off mask at a different band in time domain.

# Reference

[1] R4-2505223, “WF on UE RF requirements for LB CA via switching”, Apple

[2] RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, et., al.