3GPP TSG-RAN WG4 Meeting #115	R4-2506104

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.29.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for event triggered L1 measurement reporting were discussed. The conclusions were captured in the approved WF [1]. In this paper, we further discuss the RRM impact for event triggered L1 measurement for LTM and give our views.

- Discussion

- Whether to introduce/update requirements for reporting criteria per measurement category

- Whether to define a separate capability for supported number of events triggered L1 measurement reporting

For whether to introduce/update requirements for reporting criteria per measurement category and whether to define a separate capability for supported number of events triggered L1 measurement reporting, the candidate options are copied as follows [1]:

Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple) Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

In our view, the existing reporting criteria per measurement category did not consider LTM L1 events. Considering that sharing the same UE capabilities regarding support of event triggering between LTM events and L3 events is complicated, so it is better to introduce new UE capability for LTM. In addition, the existing Ecat was defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. However, whether to define intra-frequency and inter-frequency CSI-RS based L1 measurement are still FFS, so a separate UE capability is more conducive to defining supported number of events triggered L1 measurement reporting.

Proposal 1: Updating requirements for reporting criteria per measurement category is needed.

Proposal 2: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM.

- Conclusion

This document discussed the issues for event triggered L1 measurement for LTM and the following proposals are presented:

Proposal 1: Updating requirements for reporting criteria per measurement category is needed.

Proposal 2: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM.

- Reference

[1]     R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple