3GPP TSG-RAN WG4 Meeting #115	R4-2506111

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.31.5.2

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part2 was approved. This document will discuss other RRM requirements including downlink coverage enhancement for NR NTN phase 3 and give our understanding and proposals.

- Discussion

- RAN2 LS

In RAN2 #129bis meeting, for downlink coverage enhancement, an LS about multiple SMTC for downlink coverage enhancement was sent to RAN4, and there were two agreements from RAN2 that need confirmation from RAN4. The content of LS is copied as follows [1]:

For downlink coverage enhancement, RAN2 assumes that it will be possible to have different SSB periodicities among neighbour cells in the same frequency layer, and for the case of intra-frequency neighbouring cells, the SSBs of surrounding neighbour cells may be transmitted at different times. Thus, RAN2 made the following agreements, given that at any time the UE will not use more SMTCs/Gaps in parallel than in previous releases: - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. RAN2 would like RAN4 to provide feedback on whether it is feasible to support these agreements in Rel-19 timeframe.

In our view, for the case of configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, we believe that it just has impact on configuration and there is no additional RAN4 RRM impact. In existing requirements, if the UE only support to measure 2 SMTCs in parallel, when the network configure 4 SMTCs, the UE will choose 2 SMTCs to measure [2]. Therefore, if the network configures more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, and the UE capability of supporting multiple SMTCs in parallel doesn’t change, then the UE also choose 2/4 SMTCs to measure, and there is no need to update the related RRM requirement.

Observation 1: In existing requirements, if the UE only support to measure 2 SMTCs in parallel, when the network configure 4 SMTCs, the UE will choose 2 SMTCs to measure.

Proposal 1: For the network configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, if the UE capability of supporting multiple SMTCs in parallel doesn’t change, then the UE also choose 2/4 SMTCs to measure, and there is no need to update the related RRM requirement.

Proposal 2: Reply RAN2 that it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs in Rel-19 timeframe, and there is no additional RAN4 RRM impact.

For configuring two different SMTC periodicities (with different offsets) in one frequency layer, at least the calculation for Kmulti_SMTC  for idle, inactive and connected mode neighbor cell measurements need to be discussed, especially for the case of SMTCs partially overlapping with each other, the intra-f and inter-f measurement requirement in different SMTCs configured with different periodicities will be different. In addition, we also think that RAN4 also need to discuss whether there are impacts for L1-RSRP measurement requirements.

From RAN4 perspective, the first step is to confirm which requirements are affected by the above agreement case by case, and then discuss how to define update the requirements. Considering CR drafting, we believe that RAN4 may not have enough time to discuss relevant impacts in Rel-19 timeframe.

Proposal 3: For configuring two different SMTC periodicities (with different offsets) in one frequency layer, at least the calculation for Kmulti_SMTC  for idle, inactive and connected mode neighbor cell measurements need to be discussed, especially for the case of SMTCs partially overlapping with each other.

- The intra-f and inter-f measurement requirement in different SMTCs configured with different periodicities will be different.

Proposal 4: For configuring two different SMTC periodicities (with different offsets) in one frequency layer, RAN4 also need to discuss whether there are impacts for L1-RSRP measurement requirements.

Observation 2: Considering CR drafting, we believe that RAN4 may not have enough time to discuss relevant impacts in Rel-19 timeframe.

From our point of view, considering the time point of this WI completing, if RAN4 doesn’t have detailed conclusion on how to update RAN4 spec in this meeting, then RAN4 need to reply RAN2 that it is not feasible to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements in Rel-19 timeframe.

Proposal 5: If RAN4 doesn’t have detailed conclusion on how to update RAN4 spec in this meeting, then RAN4 need to reply RAN2 that it is not feasible to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements in Rel-19 timeframe.

- Conclusion

This document discussed the issues about SMTC enhancement in RAN2 LS and the following observations and proposals are presented:

Observation 1: In existing requirements, if the UE only support to measure 2 SMTCs in parallel, when the network configure 4 SMTCs, the UE will choose 2 SMTCs to measure.

Proposal 1: For the network configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, if the UE capability of supporting multiple SMTCs in parallel doesn’t change, then the UE also choose 2/4 SMTCs to measure, and there is no need to update the related RRM requirement.

Proposal 2: Reply RAN2 that it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs in Rel-19 timeframe, and there is no additional RAN4 RRM impact.

Proposal 3: For configuring two different SMTC periodicities (with different offsets) in one frequency layer, at least the calculation for Kmulti_SMTC  for idle, inactive and connected mode neighbor cell measurements need to be discussed, especially for the case of SMTCs partially overlapping with each other.

- The intra-f and inter-f measurement requirement in different SMTCs configured with different periodicities will be different.

Proposal 4: For configuring two different SMTC periodicities (with different offsets) in one frequency layer, RAN4 also need to discuss whether there are impacts for L1-RSRP measurement requirements.

Observation 2: Considering CR drafting, we believe that RAN4 may not have enough time to discuss relevant impacts in Rel-19 timeframe.

Proposal 5: If RAN4 doesn’t have detailed conclusion on how to update RAN4 spec in this meeting, then RAN4 need to reply RAN2 that it is not feasible to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements in Rel-19 timeframe.

- Reference

- R2-2503068, LS on SMTC enhancements, RAN2

- 3GPP TS 38.133, NR Requirements for support of radio resource management

# 5 Annex

3GPP TSG-RAN WG4 Meeting # 115                                                             R4-22XXXXX

# Malta , MT, 19th – 23rd May, 2025

Title:	Reply LS on SMTC enhancements

Response to:	R2-2503068

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG 4

To:	RAN WG 2

Cc:

Contact person:

Name:	Lingyu Gao

E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1 Overall description

RAN4 thanks RAN2 for the LS sent in R2-2503068 asking about SMTC enhancements for R19 NTN. In this meeting, RAN4 has discussed RAN4 impact for supporting the following agreements:

- RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements. - RAN2 agrees to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

Based on the discussion in RAN4, RAN4 would like to provide the following feedback:

RAN4 feedback:

It is not feasible to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements in Rel-19 timeframe.

It is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs in Rel-19 timeframe, and there is no additional RAN4 RRM impact.

2 Actions

To RAN WG2 group

ACTION: 	RAN4 sincerely asks RAN2 to take into account the above feedback into consideration.

3 Dates of Next TSG RAN WG4 Meetings

TSG-RAN WG4 Meeting #116	25 Aug – 29 Aug 2025	     India(TBC) , IN

TSG-RAN WG4 Meeting #116-bis	13 Oct  – 17 Oct 2025	     Prague, CZ