3GPP TSG-RAN WG4 Meeting #115	R4-2506112

# Malta , MT, 19th – 23rd May, 2025

Title: 	Discussion on Rel-19 NR NTN phase3 RRM performance requirements

Source: 	CATT

Agenda item:	7.31.6

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and CRs split work has been done [1]. In this contribution, we discuss and provide our initial views on the performance requirements based on the agreed core requirements.

- Discussion

  - 2.1 (e)RedCap UE with FR1-NTN

- 2.1.1 Testing configuration

For duplex mode in test configuration, considering that NTN only considered FDD duplex mode and RedCap UE has introduced HD-FDD mode, so the duplex mode in test configuration for (e)RedCap UE with FR1-NTN should include FDD and HD-FDD duplex mode.

Proposal 1: Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

For SCS and bandwidth in test configuration, we believe that the legacy configurations in NTN can be reused, as RAN4 didn’t define any new test cases for eRedCap UE in Rel-18, so 15 kHz SCS and 10MHz bandwidth can be used for RedCap UE with FR1-NTN.

Considering that both GSO and NGSO are configured in NTN test cases, so the test cases defined RedCap UE with FR1-NTN also need to consider GSO and NGSO.

Observation 1: RAN4 didn’t define any new test cases for eRedCap UE in Rel-18.

Proposal 2: 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

Proposal 3: Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

According to the above, for example, the following testing configuration can be supported for RedCap UE with FR1-NTN for cell reselection requirements:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

- 2.1.2 Test scope

- Test cases to verify the core requirements reusing NTN

Firstly, for the following core requirements, the existing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands, and there is also no need to differentiate 1Rx and 2Rx (e)RedCap UEs.

- UE transmit timing

- UE timer accuracy

- Timing Advance

- Active BWP switch delay

- Active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

However, since HD-FDD was introduced for RedCap UE, and the supported test configuration in legacy NTN test cases didn’t have this duplex mode, so new test cases for HD-FDD RedCap UE also need to be defined for the above core requirements.

Proposal 4: RAN4 to define the new test cases for HD-FDD RedCap UE for the following requirements reusing NTN:

- UE transmit timing

- UE timer accuracy

- Timing Advance

- Active BWP switch delay

- Active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

- Test cases to verify the new core requirements

For the following core requirements, we believe that the new test cases need to be added for (e)RedCap UEs with FR1-NTN bands, and the legacy test cases defined for NTN can be a baseline.

- Cell Re-selection

- Configured Grant based Small Data Transmissions (CG-SDT)

- Random access based Small Data Transmissions (RA-SDT)

- Handover/CHO/ satellite switching with re-synchronization

- RRC Connection Mobility Control

- Radio Link Monitoring

- Link Recovery Procedures

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

- NR measurements for positioning

In addition, since RAN4 agreed to defined the requirements for SDT including CG-SDT and RA-SDT, so the new test cases for SDT need to be defined for (e)RedCap UEs with FR1-NTN compared with NTN. However, there were no test cases defined for RA-SDT requirements in RedCap WI, so we also think that only CG-SDT test case need to be defined for (e)RedCap UEs with FR1-NTN.

Proposal 5: RAN4 to define new test cases for the following requirements:

- Cell Re-selection

- Configured Grant based Small Data Transmissions (CG-SDT)

- Handover/CHO/ satellite switching with re-synchronization

- RRC Connection Mobility Control

- Radio Link Monitoring

- Link Recovery Procedures

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

- NR measurements for positioning

For 1Rx and 2Rx (e)RedCap UEs, which test cases need to respectively introduced for both 1Rx and 2Rx will follow the principles in test cases for TN RedcCap UE.

Proposal 6: RAN4 to define the test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

- Test cases list

To be more specific, we give the following test cases list, which are based on existing NTN measurement cases, and CG-SDT test cases have been added. It is worth noting that 1Rx and 2Rx need to be defined separately. In order to reduce the redundancy of test cases, RAN4 needs to further discuss which test cases can be deleted.

| RRC_IDLE state mobility | - Cell reselection to FR1 intra-frequency NR case - Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements - Time-based measurement initiation to FR1 intra-frequency NR cell reselection - Location-based measurement initiation to FR1 intra-frequency NR cell reselection - Cell reselection to FR1 inter-frequency NR case - Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - Time-based measurement initiation to FR1 inter-frequency cell reselection - Location-based measurement initiation to FR1 inter-frequency NR cell reselection - Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion - Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion - Cell reselection to FR1 inter-RAT for NR NTN carrier - Cell re-selection to FR1 inter-frequency NR case with TN carrier |
| --- | --- |
| RRC_INACTIVE state mobility | - NR UE CG-SDT Test in FR1 for 1 Rx RedCap UE - NR UE CG-SDT Test in FR1 for 2 Rx RedCap UE |
| Handover | - Intra-frequency SAN Handover from FR1 to FR1 - Inter-frequency SAN Handover from FR1 to FR1 - Intra-frequency SAN time-based conditional Handover from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover from FR1 to FR1 - Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 - Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 - Intra-frequency SAN RACH-less Handover from FR1 to FR1 - Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 |
| RRC Connection Mobility Control | SA: RRC Re-establishment for SAN - Intra-frequency RRC Re-establishment in FR1 - Inter-frequency RRC Re-establishment in FR1 Random Access - 4-step RA type contention based random access test in FR1 for NR standalone - 4-step RA type non-contention based random access test in FR1 for NR standalone RRC Connection Release with Redirection - Redirection from NR in FR1 to NR in FR1 - RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 - RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 |
| Timing for Satellite Access | - NR UE Transmit Timing Test for FR1 - SA FR1 timing advance adjustment accuracy |
| Signalling characteristics Radio link Monitoring | - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode |
| Beam Failure Detection and Link recovery procedures for satellite access | - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode |
| Active BWP switch for satellite access | - DCI-based and Timer-based Active BWP Switch - RRC-based Active BWP Switch |
| UE specific CBW change for satellite access | - UE specific CBW change on PCell in FR1 in non-DRX |
| Pathloss reference signal switching delay | - MAC-CE based pathloss reference signal switch delay |
| Intra-frequency Measurements | - SA event triggered reporting tests without gap under non-DRX - SA event triggered reporting tests without gap under DRX - SA event triggered reporting tests without gap under non-DRX with SSB index reading - SA event triggered reporting tests with single measurement gap under non-DRX for satellite access - SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access - SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access |
| Inter-frequency Measurements | - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access - SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access - Event triggered reporting test without gap under non-DRX - Event triggered reporting tests without gap under DRX |
| L1-RSRP measurement for beam reporting for satellite access | - SSB based L1-RSRP measurement for satellite access when DRX is not used - SSB based L1-RSRP measurement for satellite access when DRX is used - CSI-RS based L1-RSRP measurement for satellite access when DRX is not used - CSI-RS based L1-RSRP measurement for satellite access when DRX is used |
| Measurement Performance requirements | - SS-RSRP for SAN(intra/inter) - SS-RSRQ(intra/inter) - SS-SINR(intra/inter) - L1-RSRP measurement for beam reporting (SSB based/CSI-RS based) |

Proposal 7: RAN4 needs to further discuss which test cases can be deleted from the above tset case list.

  - 2.2 Other parts for NR NTN

For this part, only SSB periodicity larger than 20 ms had some RRM impact in core requiremnets, so RAN4 only need to consider the related test cases. In addition, for SSB configuration, RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE.

According to the agreements in core part, RAN4 need to consider defining the test cases at least for RRC Re-establishment. Other test cases need to wait for the progress in core part.

Proposal 8: RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE.

Proposal 9: RAN4 to define the test cases at least for RRC Re-establishment, and other test cases need to wait for the progress in core part.

- Conclusion

This document discussed the issues for R19 NR NTN RRM performance requirements and presented following observation and proposals:

Proposal 1: Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

Observation 1: RAN4 didn’t define any new test cases for eRedCap UE in Rel-18.

Proposal 2: 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

Proposal 3: Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

Proposal 4: RAN4 to define the new test cases for HD-FDD RedCap UE for the following requirements reusing NTN:

- UE transmit timing

- UE timer accuracy

- Timing Advance

- Active BWP switch delay

- Active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

Proposal 5: RAN4 to define new test cases for the following requirements:

- Cell Re-selection

- Configured Grant based Small Data Transmissions (CG-SDT)

- Handover/CHO/ satellite switching with re-synchronization

- RRC Connection Mobility Control

- Radio Link Monitoring

- Link Recovery Procedures

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

- NR measurements for positioning

Proposal 6: RAN4 to define the test cases for 1Rx and 2Rx (e)RedCap UE with NTN.

Proposal 7: RAN4 needs to further discuss which test cases can be deleted from the above tset case list.

Proposal 8: RAN4 need to discuss whether to introduced new SSB patterns for DL enhancement NTN UE.

Proposal 9: RAN4 to define the test cases at least for RRC Re-establishment, and other test cases need to wait for the progress in core part.

- Reference

[1]	R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT