3GPP TSG-RAN WG4 Meeting #115	R4-2506114

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	7.28.3.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

# Topic #1: General and system parameters ### Sub-topic 1-1 General and system parameters Issue 1-1-1: Any RF impacts in RAN2 LS R2-2501394 Agreement: - No RF impact corresponding to RAN2 LS R2-2501394 Issue 1-1-3: Operation bands for LP-WUS in TR 38.774 Agreement: - n47, n46 ,n96 and n102 should be removed for LP-WUS.

Discussion

Transmission bandwidth configuration for LP-WUS

In previous meeting [1], it was agreed that transmission bandwidth configuration for LP-WUS will be specified under  section 5.3 with LP-WUS specific suffix of M. Below is a draft wording proposed for the section 5.3M

The transmission bandwidth configuration for LP-WUS NRB,LP-WUS shall be of length 11 RBs for both 15 KHz and 30 KHz SCS. For a [Type 1 LR], the LP-WUS RBs shall be additionally surrounded by 1 blanked RBs on both sides. LP-WUS RBs shall be flexibly allocated anywhere within the UE channel bandwidth by taking into account the additional guard RBs required, if any, specified in the side conditions for the ACS requirement.

Agree to the following wording for transmission bandwidth configuration specification for LP-WUS “The transmission bandwidth configuration for LP-WUS NRB,LP-WUS shall be of length 11 RBs for both 15 KHz and 30 KHz SCS. For a [Type 1 LR], the LP-WUS RBs shall be additionally surrounded by 1 blanked RBs on both sides. LP-WUS RBs shall be flexibly allocated anywhere within the UE channel bandwidth by taking into account the additional guard RBs required, if any, specified in the side conditions for the ACS requirement”.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Agree to the following wording for transmission bandwidth configuration specification for LP-WUS “The transmission bandwidth configuration for LP-WUS NRB,LP-WUS shall be of length 11 RBs for both 15 KHz and 30 KHz SCS. For a [Type 1 LR], the LP-WUS RBs shall be additionally surrounded by 1 blanked RBs on both sides. LP-WUS RBs shall be flexibly allocated anywhere within the UE channel bandwidth by taking into account the additional guard RBs required, if any, specified in the side conditions for the ACS requirement”.

- References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, Hubei, China, 07th – 11th April, 2025.