3GPP RAN WG4 Meeting #115																	R4-2506123

Malta, Malta, 19th – 23rd May 2025

Agenda item:	7.33.5

Source:	Apple

Title:	On NS flags and A-MPR values for the IOT TDD band

WI/SI:	IoT_NTN_TDD-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. After that additional NTN bands were defined including a new spectrum WID [4], which will add the IOT TDD band operating in the same frequency range as the IOT band 254.

During the previous meetings RAN WG4 discussed which regulations should be followed for this IOT TDD band, which NS flags to adopt and, as a result, whether any A-MPR would be needed. Thus, in this discussion paper we provide our general view on the aforementioned aspects.

# 2	Regulations, NS flags and A-MPR values

Frequency range of 1610-1626.5MHz and its usage for the MSS services has well established international and local regulations, captured in the corresponding WRC provisions and local standards. This was already discussed by 3GPP in Rel-18 during standardisation of the NTN band 254/n254. And since existing regulations and requirements do not differentiate between the FDD and TDD modes, our understanding is that the same regulatory frameworks can be applied to a new IOT TDD band 249 operating in 1616-1626.5MHz frequency range. To be more precise, in case of FCC it will be covered by Title 47 CFR § 25.202 and § 25.216; and in case of ETSI it will be ETSI EN 301441. For countries and regions following neither FCC nor ETSI rules, generic 3GPP requirements would apply which are usually based on the international regulations.

Proposal 1:	Since a new IOT TDD band 249 falls into the same frequency range as band 254 UL, same international and local regulations can be followed (i.e. ETSI EN 301 441, FCC Title 47 CFR § 25.202 and § 25.216).

Following the same line of considerations as above, a new IOT TDD band can leverage existing NS flags, which were already defined for this frequency range:

-	NS_03N: This NS flag will be used to cover channels configured under the FCC regulatory domain

-	NS_04N: This NS flag can be used to cover channels configured in ETSI 1616-1618.25MHz range

-	NS_05N: This NS flag can be used to cover channels configured in ETSI 1618.25-1626.5MHz range or channels overlapping between this and previous range.

Proposal 2:	Same NS flags, as for band 254, can be also applied to a new IOT TDD band 249 (i.e. NS_03N for FCC and NS_04N/NS_05N for ETSI)

Band 254 NS flags define local regulatory requirements and, in case of the ETSI regulatory domain, also define A-MPR values needed to comply with the ETSI in-band emission masks. There is an ongoing discussion to finalise band 254 A-MPR values (see Annex A for 12, 6 and 3-tone power back-off results), outcome of which can be also applicable to band 249. Table 2-1 below presents a summary of our power back-off simulations for the concerned frequency range.

Table 2-1: Summary of A-MPR values

| Number of tones | FCC 1610-1626.5 MHz | ETSI |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | 1610-1618.25 MHz | 1618.25-1626.5 MHz |  |
| 12 | 0 | 0 | 0 | Full RB 12-tone allocation |
| 6 | 0 | 0 | 1 | 0-5 and 6-11 tones |
| 3 | 0 | 0.5 | 1.5 | 0-2 and 9-11 tones |
| 1 | 0 | 2 | 2 | 2dB A-MPR is applied to ETSI because of the existing -15dBW/4kHz limit, i.e. 21.3dBm/15kHz |

Proposal 3:	Same A-MPR values, as for band 254, can be also adopted for a new IOT TDD band 249.

# 3	Conclusions

In this discussion paper we have presented our considerations on the IOT TDD band regulations, NS flags and A-MPR values. Our understanding is that since this band falls into the same frequency range as band 254 UL, the same international and local regulations would apply meaning that we can leverage existing NS flags with associated A-MPR values.

Proposal 1:	Since a new IOT TDD band 249 falls into the same frequency range as band 254 UL, same international and local regulations can be followed (i.e. ETSI EN 301 441, FCC Title 47 CFR § 25.202 and § 25.216).

Proposal 2:	Same NS flags, as for band 254, can be also applied to a new IOT TDD band 249 (i.e. NS_03N for FCC and NS_04N/NS_05N for ETSI)

Proposal 3:	Same A-MPR values, as for band 254, can be also adopted for a new IOT TDD band 249.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-230792, "New WID: FDD LTE band (L+S band) for IoT NTN operation", MediaTek

RP-242415, "New WID for introduction of IoT-NTN TDD mode", Iridium Satellite LLC

R4-2505407, "Clarification for the PSD limits for a UE operating in 1610-1626.5 MHz frequency range", Apple, Globalstar

Annex A: Power back-off results for 12, 6 and 3-tone allocations

Figure A-1: 12-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure A-2: 6-tone spectrum plots (FCC, ETSI sub-range 1 and 2)

Figure A-3: 3-tone spectrum plots (FCC, ETSI sub-range 1 and 2)