3GPP TSG-RAN WG4 Meeting #115	R4-2506129

Saint Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on remaining issues on Rel-18 Mobility enhancements

Agenda item:	5.19.1

Document for:	Discussion

Introduction

This contribution discusses about the remaining issues of Release 18 mobility enhancements.

Issue 1-2-1: whether/how to handle the collision between an SSB and a valid RO for LTM Agreement on reply LS - RACH shall be prioritized over SSB, if the collision is outside MG. - If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion. < Agreement > - Further discuss the definition of RACH occasion colliding with the MG occasion.

Discussion

PRACH transmission and MG collision

In RAN4#114 it has been agreed in RAN4 that when the network transmits PDCCH order to trigger RACH, and in relation to the measurement gap, RACH transmission shall be prioritized over DL SSB measurement, if the collision is outside of MG. If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion collides with the MG occasion.

By design, LTM does not have automatic retransmission of preamble. If the UE drops the RACH transmission attempt, currently it is not clear whether the UE should transmit on a next occasion or not. From the network point of view, the network has transmitted a PDCCH ordered RACH, and the network is expecting UE to perform PRACH transmission. Hence, the UE should always transmit RACH preamble at the next valid PRACH occasion (e.g., one not colliding with MG).

After receiving PDCCH ordered RACH command, the UE shall eventually transmit RACH preamble without a new PDCCH ordered RACH command from the network.

When the PRACH transmission is performed outside of the active BWP (configured or outside of the configured) there is RF / BB retuning and interruption defined by RAN4. Currently it is not clear if when the MG overlaps with RF / BB retuning time or interruption time when the UL resource is outside of the active BWP. It should be clarified what is the UE behaviour if the RF/BB preparation or interruption time overlaps with MG. RF / BB retuning can be up to 10ms, hence it’s quite a long duration together with PRACH transmission and interruption.

If MG is colliding with any of the components PDCCH ordered RACH, i.e., BB/RF preparation time, interruption, or PRACH transmission, and the UE is not capable / able to prioritise the RACH transmission over MG, the UE shall transmit preamble at the next available / valid RACH occasion.

Interruptions due to PDCCH ordered RACH on target LTM cell

RAN4 replied to an LS from RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18 in the last RAN4#114bis meeting [1], [2].

RAN4 would like to thank RAN2 for the LS on granularity definition of pdcch-RACH-AffectedBandsList-r18. RAN4 discussed the question from RAN2 and reached the following agreement: - "Understanding 2" in the LS (R2-2501392 / R4-2503115) is what RAN4 intended. And the band in which the PDCCH order is received has no impact on whether and on which band the interruption may occur.

We think that the requirements should be still clarified on the interruption / noInterruption side in the following way:

##### 8.2.2.2.20 Interruptions due to PDCCH ordered RACH on target LTM cell When PRACH transmission is triggered by PDCCH order on target LTM cell, - During RACH transmission, the UE is allowed an interruption on activated serving cell’s DL slot(s) overlapped with RACH occasion depending on UE capability pdcch-RACH-AffectedBandsList-r18. - If UE indicates NoInterruption for an activated serving band in the pdcch-RACH-AffectedBandsList-r18, the UE shall not cause interruption on DL slot(s) overlapped with RACH occasion for the serving band indicated in the band pair. - If UE indicates Interruption for an activated serving band in the pdcch-RACH-AffectedBandsList-r18, the UE may cause interruption on DL slot(s) overlapped with RACH occasion for the serving band indicated in the band pair.

This would also help to remove any remaining ambiguities in the use of the capability.

Clarify in TS 38.133 the use of “Interruption” / “NoInterruption” of pdcch-RACH-AffectedBandsList-r18 flag

Conclusion

The following Observations and Proposals were made:

Proposal 1: After receiving PDCCH ordered RACH command, the UE shall eventually transmit RACH preamble without a new PDCCH ordered RACH command from the network.

Proposal 2: If MG is colliding with any of the components PDCCH ordered RACH, i.e., BB/RF preparation time, interruption, or PRACH transmission, and the UE is not capable / able to prioritise the RACH transmission over MG, the UE shall transmit preamble at the next available / valid RACH occasion.

Proposal 3: Clarify in TS 38.133 the use of “Interruption” / “NoInterruption” of pdcch-RACH-AffectedBandsList-r18 flag

# References

- R2-2501392 LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18 3GPP TSG RAN WG2 Meeting #129 Athens, Greece, 17th – 21st Feb. 2025

- R4-2504940 Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18 3GPP TSG-RAN WG4 Meeting #114bis Wuhan, China, 7th – 11th April 2025

- 3GPP TS 38.306 User Equipment (UE) radio access capabilities, V18.5.0 (2024-03)