3GPP TSG-RAN WG4 Meeting #115	R4-2506136

St Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on LP-WUS-WUR RRM core requirements

Agenda item:	7.28.5.2

Document for:	Discussion

Introduction

In this contribution we discuss open issues related to LP-WUS-WUR work item core requirements.

Discussion

## LP-WUS status at legacy state

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Currently we have captured this in the CR:

The requirements in this clause apply when: - The LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation mode and after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement.

Our understanding is that the requirements apply when the LR is in “on” state. However, it is up to UE implementation when to turn the LR to “on” state. We don’t see a problem with this approach hence we propose to specify:

The requirements in this clause apply when the LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation mode and after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement

Higher priority frequency layer and MR neighbouring cell measurements

Issue 1-1-10: Considerations on higher priority frequency layer

It was agreed that relaxed higher priority frequency layer search is supported at case 1. As in case 3 MR is anyways on, it is natural to support high priority case in this case.

In the last meeting case 1 was discussed and tentative agreement was reached. We are ok to confirm the agreement.

For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. Note: K2 is 60

Relaxing higher priority carriers and relying solely on LR measurements will cause the UE not to measure the neighbouring cells for a long period of time, for instance 1h in the case above.

When we consider the current idle-mode measurement requirements, it is highly likely that when the UE exits LP-WUS relaxation, MR neighbouring cell measurements need to be restarted. When the UE exits LP-WUS mode because serving cell has been getting worse (e.g. exit threshold is reached), the UE should be able to check relatively quickly which neighbouring cell is a good one in order to reselect if needed.  Ideally, in this case the MR should not start the measurements from idle-mode detection as this would take a very long time and impacts how the network need to configure thresholds for offloading or relaxation. Network in this case needs to take account also the long neighbouring cell measurement time after the LR has decided to exit LP-WUS mode. How this is done after 1h of relaxations is not very clear to us.

Long measurement time after LP-WUR offloading / relaxation exit will impact the network threshold configurations, hence reducing area where the feature can be used.

After UE exits offload / relaxation mode based on LR measurements, the MR should not restart neighbouring cell measurements from cell detection.

Lower / Upper bound on LP-SS periodicity

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

This issue is about mandating lower bound to LP-SS. So far RAN1 has agreed LP-SS periodicity of 320ms. From the network point of view this means that the network does not need to transmit LP-SS more than every 320ms. Before RAN1 agrees other periodicities, the LP-SS periodicity assumption should be 320ms. As far as the network is concerned, when UEs with LP-SS are configured in the network, the LP-SS is only transmitted every 320ms if no further agreements made in RAN1.

RAN1 Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

- No need for lower bound in requirements. Use the agreed 320/160ms periodicity.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

Similarly to lower bound, there is no need to restrict any configurations on upper bound, the requirements should assume reasonable configuration from the network side.

- Assume that the network configuration is reasonable. No need to define upper bound for SSB-based LP-WUR measurement periodicity

LR evaluation requirements

Issue 1-2-4-2-3: On how to define LR evaluation requirements

The following text is capture in the CR R4-2506137 LP-WUR serving cell measurement and evaluation requirements.

The UE shall be capable of evaluating that the following conditions defined in TS 38.304 [1] are met within Tevaluate-LP-WUR-LP-SS - [entry condition for LP-WUS monitoring] - [entry condition for RRM relaxation] - [entry condition for RRM offloading] - exit condition for LP-WUS monitoring - exit condition for RRM offloading

Currently it seems that we don’t have an agreement that the entry condition shall be met with LR.Our understanding is that it is up to UE implementation when to turn the LR on, but if the LR entry thresholds have been configured, then UE shall evaluate those before entering the LP-WUS monitoring, RRM relaxation or RRM offloading. This is implemented in our current CR, and we think it’s ok to remove the brackets around entry conditions.

When entry condition is configured for LP-WUR, the UE shall evaluate the corresponding entry condition before entering LP-WUS monitoring, RRM relaxation or RRM offloading within evaluation period.

What is also not clear is how often the LP-SS / PSS/SSS evaluation is performed. This is needed at least for the test cases.

Discuss how often LP-SS / PSS/SSS evaluation is performed.

MR and LR evaluation thresholds

Issue 1-2-5-2: On MR evaluation requirements

According to agreements in previous RAN4 meeting, it is up to UE implementation when and how to turn on the LR for serving cell measurements. As mentioned, this is ok behaviour from UE power saving perspective as it allows UE to have control over power saving on LR (even if RAN1 design suggests the power consumption is minimal). Furthermore, it was agreed that when both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately. One thing to clarify here is that when MR and LR thresholds are both configured, the UE shall enter relaxations / serving cell offloading only when both of the criteria are met, when configured. We believe this is the thinking from companies but we also think it is good clarify.

- If both MR and LR entry thresholds are configured (e.g. for LP-WUS monitoring entry or LP-WUS RRM relaxation entry), the UE shall evaluate both LR and MR thresholds before MR relaxations or MR offloading requirements apply. I.e. LR threshold evaluation is not optional when it is configured.

Number of SSBs and MR sync time

Issue 1-2-6-1: On the number of SSBs

The number of SSB required by MR cannot be determined without determining side condition first. RAN1 design on LP-WUS is that LP-WUs is not used at the cell edge. Therefore, we can use existing side condition and existing number of samples in the requirements.

Currently in RAN4 requirements. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = Trs ms. Therefore 1 sample for MR in very good radio conditions should be enough. In RAN1 discussions the assumption has been that the UE can reuse the timing from LR, so one sample should be enough. RAN1 assumption has been that MR and LR share the timing information

MR synchronization shall be associated with specific target side condition, e.g., ≥-2dB, or -3dB

- 1 SSB samples is enough for sync in good radio conditions where LR is assumed to be used.  1 sample can be used for AGC. Max 2 samples.

RAN1 proposed that the number of SSBs is different depending on the sync state. From RAN4 point of view there is no need to differentiate different cases. Hence same number can be used regardless of the wake up mode.

Use same number of samples, (e.g. [2,2,2] (-2 dB))

RAN4 has not yet concluded whether to use SSB or SMTC periodicity after the MR wake up. RAN1 sent and LS to RAN4 to confirm that SMTC periodicity is not used. We think that this approach is ok. If the serving cell quality is reduced and MR needs to be woken up, then it is important that SSB periodicity is used in this case to gain quick serving cell synchronisation, and measure neighbouring cells.

- After MR wake up, MR follows SSB periodicity for serving and neighbouring cell evaluation.

MR accuracy

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

Legacy accuracy requirements are used for MR.

LP-WUR in connected mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Currently it is not clear if RAN4 should be defining any requirements for the connected mode. For the scenario where UE MR is part of MR it is highly likely that the receiver is also capable of performing SSB measurements. Therefore, there may not be need to add any special handing for connected mode in this scenario, at least in Rel-19 scope.

If companies would like to pursue connected mode requirements in this release, we are ok to start the discussion about LR PSS/SSS and LP-WUS monitoring requirements and measurement procedures.

- Prioritise idle-mode requirements over connected mode requirements. If needed, start the connected mode requirements from LR PSS/SSS and LP-WUS monitoring requirements.

Support of FR2 scenario

Issue 1-1-11: RRM requirements for FR2

Currently there are various open issues around the LP-WUS framework. The current WID has support for FR2, but when considering the RAN4 workload, the maturity of LP-SS design, and the complexity of defining FR2 requirements wit beam sweeping. RAN4 can prioritise the FR1 requirement work over FR2-1 and FR2-2 requirements.

- Proposal 6: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Idle-mode measurements and LR

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

In the previous meetings, companies have brought up issues related to early measurement reporting. The rel-18 work on fast CA/DC setup expanded the idle-mode procedures to cover reselection measurements also. RAN4 should discuss whether the relaxation factor applies to all EMR or whether the validity of the measurement results can be guaranteed when the UE has relaxing MR measurements.

- RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact

Some companies have already proposed that the UE shall keep the MR measurements on while there is EMR timer running, this should also work when UE is in idle-mode less than the duration of the timer.

The configuration for both EMR and LP-WUS is coming from system information, which means that the UE may not be configured by the same cell as the UE left. Therefore, if the UE is configured simultaneously with EMR and LP-WUS the UE should not use LR.

- Discuss about two alternatives to handle the situation

  - When T331 running UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

  - UE shall not enter relaxation / offloading if configured simultaneously with EMR.

Conclusion

The following Observations and Proposals were made:

Proposal 1: The requirements in this clause apply when the LP-WUR is in ON state. Before entering LP-WUS, RRM offloading and RRM relaxation mode and after exiting LP-WUS, RRM offloading and RRM relaxation mode, it is up to LP-WUS UE implementation when and how to turn the LP-WUR to ON state for serving cell measurement

Proposal 2: For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. Note: K2 is 60

Observation 1: Long measurement time after LP-WUR offloading / relaxation exit will impact the network threshold configurations, hence reducing area where the feature can be used.

Proposal 3: After UE exits offload / relaxation mode based on LR measurements, the MR should not restart neighbouring cell measurements from cell detection.

Proposal 1: No need for lower bound in requirements. Use the agreed 320/160ms periodicity.

Proposal 2: Assume that the network configuration is reasonable. No need to define upper bound for SSB-based LP-WUR measurement periodicity

Proposal 3: When entry condition is configured for LP-WUR, the UE shall evaluate the corresponding entry condition before entering LP-WUS monitoring, RRM relaxation or RRM offloading within evaluation period.

Proposal 4: Discuss how often LP-SS / PSS/SSS evaluation is performed.

Proposal 5: If both MR and LR entry thresholds are configured (e.g. for LP-WUS monitoring entry or LP-WUS RRM relaxation entry), the UE shall evaluate both LR and MR thresholds before MR relaxations or MR offloading requirements apply. I.e. LR threshold evaluation is not optional when it is configured.

Proposal 6: MR synchronization shall be associated with specific target side condition, e.g., ≥-2dB, or -3dB

Proposal 7: 1 SSB samples is enough for sync in good radio conditions where LR is assumed to be used.  1 sample can be used for AGC. Max 2 samples.

Proposal 8: Use same number of samples, (e.g. [2,2,2] (-2 dB))

Proposal 9: After MR wake up, MR follows SSB periodicity for serving and neighbouring cell evaluation.

Proposal 10: Legacy accuracy requirements are used for MR.

Proposal 11: Prioritise idle-mode requirements over connected mode requirements. If needed, start the connected mode requirements from LR PSS/SSS and LP-WUS monitoring requirements.

Proposal 12: Proposal 6: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Proposal 13: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact

Proposal 14: Discuss about two alternatives to handle the situation

a.	When T331 running UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

b.	UE shall not enter relaxation / offloading if configured simultaneously with EMR.