3GPP TSG-RAN WG4 Meeting #115	R4-2506162

# St Julian’s, Malta, 19th – 25th May 2025

Title:	REFSENS procedure, SFO and timing aspects

Source:	Qualcomm Incorporated

Agenda item:	7.26.3.2

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4#115 WF for Ambient IoT device [8] was agreed. In this paper we continue discussion on general aspecst, bandwidth, REFSENS process, SFO and timing aspects.

# 2. 	Terminology

Terminology alignment to help the discussion:

Frequency shift by R=1 – in the WF [8], the figure, there seems to be conflicting use of terminology for the R=1. It would be good to align what does it mean that there is or there is no frequency shift.

Proposal 1: R=1 is frequency shift by 1 in RAN4 terminology.

Agreement can also come via other agreement such as definition of bandwidth  with frequency shift of R=1,2,4…

## 2.1 	Device bandwidth considerations.

In the WF, the D2R channel bandwidth with frequency shift is defined. RAN1 has agreed [A] a set of chip rates and bit lengths that can be used to calculate all possible channel BWs. RAN1 is in the process of downselecting the possible values and how to indicate which one to be used. RAN4 can use this table as a starting point to define channel BW for the D2R that can be used for deployment purposes of allocating bandwidth.

One issue to be discussed is how to accommodate odd harmonics of the device. In the Figure 2 focusing on the green spectrum, the hump on frequencies below 500 kHz is the 3rd harmonic of the intended spectrum around 160 kHz. As it can be seen, the 3rd harmonic is substantially high level. This is due to ASK modulation nature.

Observation 1: Odd harmonics of the D2R signal are high level for harmonics compared to the fundamental.

Our understanding is that they are 9 dB lower than the fundamental.

Figure 2. Ambient IoT device spectrum with four different small frequency shifts.

If the harmonics are essential for the reception of the signal is not clear. Without receiving the harmonics, the signal shape is rounded but the transition timing is still the same and therefore RAN4 could exclude the harmonics from the channel BW of the D2R signal.

Proposal 2: Do not include baseband harmonics in the channel bandwidth calculation

The RAN1 agreed table then becomes as shown in Table 1 as a function of occupied RBs for double sided spectrum (there maybe typos in the table).

Table 1. Number of occupied RBs as a function of bitlength (T_b) and chip length (T_c) (DSB)

From this table it can be derived that the largest channel bandwidth is 24 RBs, this is including 10 % SFO adder. This need to be increased by the size of the guardband. Our understanding is that RAN1 is in the process off downselecting values in the table so we are not making any proposals this time.

Observation 2: Largest channel bandwidth in number of RBs with current agreements is 24 RB + guardbands

## 2.2	Process for REFSENS

RAN4 has agreed a process for determining REFSENS in WF [8] that contains the following key step:

Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min

WF also has this aspect among many others to consider:

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

How that step 5 is exactly done is not clear since we can not specify BLER based REFSENS criteria and how to accommodate the miss detection rate (MDR) and the success rate. Firstly, these terms are for a population of devices and therefore as such are not suitable for device requirements. It also appears that 90 % success rate is same as 10 % miss detection rate.

RAN4 should consider also ensuring that the device false alarm rate (FAR) of 10 % is not exceeded.

False Alarm happens when a command or inventory request is sent by the reader but this request or command was not addressed the device in question. For example, if a specific device is addressed but this device receives that incorrectly and responds, then it can be determined that the device did not receive the R2D message correctly and falsely responded.  Process should rule out the situation where the device responds just upon reception of something and does not correctly decode the message.

Proposal 3: Ensure with the process for testing REFSENS that device decodes the R2D correctly

To accommodate the agreed miss detection rate of 10 %, the reader should repeat the request for response some number of times and allow device to fail 10% of the receptions. The number of repeats and how many are needed to ensure device is within limits should be left for RAN5 discussion.

Proposal 4: 10% false alarm rate and 10% miss detection rate should be included in the REFSENS test procedure

To test the device properly, the device should be commended to do something specific to that device and the observable success criteria for correctly decoding the R2D message is a correct device response in D2R. This can be done in two ways within the existing RAN1 and RAN2 agreements:

- Use contention-free random access (CFRA) procedure with the device pagingID to respond (Msg1) in one of the usable resources (T_b and T_c pair) in correct time window TR->D

- Use contention based or contention based random access (CBRA) procedure and assign AS ID to the device in msg2 and then use that assigned AS ID to command the device to do something in defined resources

The problem with the option 2 is that reader must go through CBRA and during this device may run out of energy, since energy aspects are not defined and if device has poor energy capabilities, then the CBRA procedure maybe need to be repeated for every measurement if device forgets the AS ID between harvesting periods and then it creates further problems since AS ID may need to be re-assigned..

Observation 3: Test procedures should support devices with low energy capabilities and using CBRA may lead to very long test times

Proposal 5: Assume the device under test in a known device in REFSENS testing process

Proposal 6: Use known pagingID and CFRA in REFSENS test procedure

With this process, the MDR and FAR can be accommodated.

# 3 Discussion on SFO

Sample Frequency Offset is the offset from the defined sample frequency of 1.92 MHz. The offset can be observed in two ways, 1) in time domain as longer than expected symbol time and e.g. time interval between repeated transmissions or 2) frequency offset The device creates a small frequency offset and RAN1 is discussing how many different offset the device will support. In Figure 3 we show spectrum of the device output with four different offsets.

RAN1 is currently assuming 10 % initial device SFO. This would mean that the 16 x frequency shifted spectrum in Figure 3 is shifted more than the one full sample frequency, 1.6 times more. It maybe challenging to detect the signal this way but it also makes the test easy: set the device to maximum possible small frequency shift and then measure center of the transmitted spectrum.

Alternative way, by using the time domain measurements is also easy if device is set to repeat the message 2 multiple times. Observing the preamble from the last repeated message multiplies the error.

Observation 4: By using maximum frequency shift or time domain repetition it is easy to observe the SFO.

Before defining the test or even the requirement, Ran4 needs to wait for RAN1 conclusion on the max SFO, max repetitions and maximum frequency shifts.

One method to identify the SFO of the device is to observe the D2R spectrum in frequency domain, e.g. one in Figure 1.

Proposal 7: Define spectral confinement based on +/- 10 % maximum error in sample clock

An other method is to observe the D2R message in time domain. RAN1 has agreed that the message response time is a parameter to be given to the device. RAN1 defines nominal time and RAN4 should define limits for that time which can be derived from SFO, e.g. +/- 10 %.

Proposal 8: Define TR->D minimum and maximum as +/- 10 % of the nominal value.

It might be useful to define both time domain and frequency domain confinement limits including SFO

# 4 CW input level

RAN4 should define CW power range for the device including the maximum and minimum. Then for REFSENS and maximum output power and emissions tests, a suitable worst case value can be chosen. If this will be written to RAN4 or RAN5 specifications can be discussed.

Proposal 9: Define a range for CW input power for the device similarly as max input level.

Our preference is to say in Ran4 that the device will meet all the requirements in the full CW range but something RAN4 could also discuss if REFSENS must be met wirth maximum CW level. Also, if max output power should be met with minimum CW level seems unlikely, thus we propose:

Proposal 10: CW level related proposals as follows:

10-1: REFSENS is met with lowest CW power level

10-2: Maximum output power is met with maximum CW power level. Output power is assumed to scale linearly with CW level until minimum CW level

10-3: Emission are met with maximum CW input level

# Conclusion

We discussed various device requirements and made the following observations:

Observation 1: Odd harmonics of the D2R signal are high level for harmonics compared to the fundamental

Observation 2: Largest channel bandwidth in number of RBs with current agreements is 24 RB + guardbands

Observation 3: Test procedures should support devices with low energy capabilities and using CBRA may lead to very long test times

Observation 4: By using maximum frequency shift or time domain repetition it is easy to observe the SFO.

And we proposed:

Proposal 1: R=1 is frequency shift by 1 in RAN4 terminology.

Proposal 2: Do not include baseband harmonics in the channel bandwidth calculation

Proposal 3: Ensure with the process for testing REFSENS that device decodes the R2D correctly.

Proposal 4: 10% false alarm rate and 10% miss detection rate should be included in the REFSENS test procedure

Proposal 5: Assume the device under test in a known device in REFSENS testing process

Proposal 6: Use known pagingID and CFRA in REFSENS test procedure

Proposal 7: Define spectral confinement based on +/- 10 % maximum error in sample clock

Proposal 8: Define TR->D minimum and maximum as +/- 10 % of the nominal value.

Proposal 9: Define a range for CW input power for the device similarly as max input level.

Proposal 10: CW level related proposals as follows:

10-1: REFSENS is met with lowest CW power level

10-2: Maximum output power is met with maximum CW power level. Output power is assumed to scale linearly with CW level until minimum CW level

10-3: Emission are met with maximum CW input level

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500490, “Ambient IoT device requirements”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[7]	R4-2502858, “WF on requirements for system parameters and A-IOT device”,  CMCC, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[8]	R4-2505230, “WF on A-IOT device requirements”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[9]	R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[10]	R4-2505227, “(new) TS 38.191 Ambient IoT device radio transmission and reception”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[11]	R4-2505228, “3GPP_TS 38.194 skeleton”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[12]	R4-2503560, “Relevance of phase noise and LLS for SNR for REFSENS”, Qualcomm, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[13]	R4-2503559, “Ambient IoT device backscattering and SFO error”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

# Appendix: Relevant RAN1 agreements

R1-2503023	Summary #4 for coding aspects of physical channel design	Moderator (CMCC)

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
|  | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |