3GPP TSG-RAN WG4 Meeting #115	R4-2506163

# St Julian’s, Malta, 19th – 25th May 2025

Title:	Phase noise specification for CW

Source:	Qualcomm Incorporated

Agenda item:	7.26.3.3

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4#114bis approved a WF [11] for BS and CW requirements. WF agrees in Issue 3-4 to FFS if phase noise can be specified. What makes it complicated is that some companies plan to use shared reference for the CW source despote the topology in the WID. In this paper we propose a specification for the phase noise for the CW.

# 2. 	Discussion on CW phase noise

In [14] we proposed to adopt the phase noise of -90 dBc/15 kHz. However there was an inbuild compromise in that proposal and in this paper we discuss phase noise specification for the two cases with the agreed D2R bandwidth.

The agreed BW in the WF [11] is 15 kHz for the DSB case. This equals to 7.5 kHz on each side of the CW. Further away from the CW the phase noise become better thus less important.

Observation 1: Phase noise need to be specified only for the closest D2R bandwidth.

It is also possible to specify either single sided or double sides phase noise.

Some small offset should be set from where the measured phase noise is integrated. It is not practical to measure the noise from 0 Hz offset. Observing the D2R spectrum, the energy at the edge of the spectrum is very low and likely the 1/f noise is very high at the CW source.

Proposal 1: Ignore first 500 Hz of the phase noise.

Similarly the SFO of 10 % should be included i.e. the phase noise should be integrated up to 7.5 kHz + 10% i.e. 8.25 kHz.

From system simulations in the TR we can conclude that the 90 dBc is a good value for the requirement regardless of the integration bandwidth.

Proposal 2: For truly CW outside topology, the phase noise of CW is 90 dBc/8.25 kHz DSB.

If a shared reference is used and it provides the receiver CW cancellation circuitry a CW reference with correlated phase noise, the value itself is less relevant. Looking at the phase noise profile in Figure 6.7.5.2.2.4-2 of TR 38.769, achievable phase noise is between -100 to -110 dBc/Hz. Integrated over 8.25 kHz it comes to 71 dBc/7.5 kHz SB or -68 dBc.8.25 kHz DSB phase noise.

Proposal 3: For shared reference between CW and reader receiver deployment phase noise requirement is -68 dBc/8.25 kHz DSB.

In Appendix we provide specification language for this requirement.

# Conclusion

We discussed, observed and proposed:

Observation 1: Phase noise need to be specified only for the closest D2R bandwidth.

Proposal 1: Ignore first 500 Hz of the phase noise.

Proposal 2: For truly CW outside topology, the phase noise of CW is 90 dBc/8.25 kHz DSB.

Proposal 3: For shared reference between CW and reader receiver deployment phase noise requirement is -68 dBc/8.25 kHz DSB.

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500490, “Ambient IoT device requirements”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[7]	R4-2502858, “WF on requirements for system parameters and A-IOT device”,  CMCC, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[9]	R4-2502859, “WF on requirements for A-IoT BS and CW”, Huawei, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[10]	R4-2505230, “WF on A-IOT device requirements”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[11]	R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[12]	R4-2505227, “(new) TS 38.191 Ambient IoT device radio transmission and reception”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[13]	R4-2505228, “3GPP_TS 38.194 skeleton”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[14]	R4-2503560, “Relevance of phase noise and LLS for SNR for REFSENS”, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[15]	R4-2503559, “Ambient IoT device backscattering and SFO error”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

# Appendix: Specification for CW phase noise

## x.1 Phase noise requirement for the CW source

CW phase noise is measured on both sides of the CW frequency and integrated over 500 Hz to 8.25 kHz. The integrated phase noise is less than the values in table x.1-1. In Table x.1-1, the shared reference unit is a CW source with external reference clock that is used by the reader node in the same topology. Which requirement applies is based on manufacturer declaration. The reference power is the power of the CW at the desired frequency.

Table x.1-1 Phase noise requirement for CW source

|  | IPn | Unit |
| --- | --- | --- |
| Standalone CW unit | -90 | dBc |
| Shared reference unit | -68 | dBc |