3GPP TSG-RAN WG4 Meeting #115	R4-2506171

Malta, May 19 – May 23, 2025

Source:	Tejas Networks

Title:	Study of RAN4 impacts for RRM measurement prediction for AIML mobility in NR

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

The discussions on AI/ML mobility in NR were continued in RAN4#114bis in the scope of Rel-19 SI. Agreements achieved during this meeting are summarized in the WF [1].

In this paper, we provide our views on issues related to factors that potentially impact RAN4 requirements.

Discussion

During the RAN4 #112bis meeting, RAN4 agreed to study aspects related to NR AIML mobility requirements and prioritized the following scenarios and use cases.

Relative RSRP accuracy definition

Agreement: - For intra-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 and cell 2 are on the same frequency, or - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of beam 1 – reported predicted RSRP of beam 2) – (ground truth of RSRP of beam 1 – ground truth of RSRP of beam 2), where beam 1 and beam 2 are from the same cell. - FFS if this definition will be needed. - For inter-frequency L3-RSRP, - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported predicted RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, if cell 1 and cell 2 are the target cells for AI/ML-based L3-RSRP inference. - Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2]. - Depending on the agreement on ground truth definition, we may revisit relative RSRP accuracy definition. - FFS: whether the reported RSRP of cell2 can be predicted or measured or a combination of both predicted and measured samples. - FFS, if there is any impact on this definition for different scenarios/different frequency ranges.

RAN4 #114bis meeting agreed on the definition of relative RSRP accuracy for the inter-frequency case, where Cell 1 is the target cell for AI/ML-based L3-RSRP inference based on measurements from Cell 2, as follows:

Accuracy of predicted relative L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), where cell 1 is on a different frequency than cell 2 but in the same FR as cell 2, [if cell 1 is the target cell for AI/ML-based L3-RSRP inference based on the measurements on cell 2].

The definition includes both the reported RSRP of Cell 2 and the ground truth RSRP of Cell 2. RAN4 should clarify the distinction between ground truth RSRP and reported RSRP for Cell 2.

Proposal 1: RAN4 to clarify the difference between ground truth RSRP and reported RSRP

As per the definition, Cell 1 is the target cell for AI/ML-based L3-RSRP inference using measurements from Cell 2. This means the AI/ML model inputs measurements from Cell 2 to predict the RSRP for Cell 1. The accuracy of Cell 2’s measurements is crucial, as any errors will reduce the reliability of the prediction. However, if the prediction accuracy for Cell 2 is sufficiently high, the RSRP of Cell 2 can be a combination of both predicted and measured samples.

- Below figure illustrates RRM measurements prediction in the frequency domain. Predicting measurements for unmeasured frequencies can be challenging due to varying channel conditions across frequency bands. Interference patterns also differ, which can lead to inaccurate predictions if not properly accounted for. Additionally, in scenarios with carrier aggregation or frequency hopping, managing multiple carriers or changing patterns can further reduce the reliability of predictions.

Figure 3: Inter frequency measurement prediction

Observation 1: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies

In scenarios where measurements are limited to the serving frequency, it can be beneficial to consider monitoring neighbouring frequencies for short durations. By incorporating measurements from inter-frequency neighbours and combining them with predictions based on temporal trends, the accuracy of these predictions can be improved. Temporal domain prediction, which relies on past measurement trends over time, can be enhanced by the additional context provided by inter-frequency measurements. This integration could provide a more comprehensive understanding of the network, potentially leading to more accurate predictions without significantly increasing measurement overhead.

Proposal 2: RAN4 should explore potential enhancements in inter-frequency measurement prediction by combining with temporal domain prediction.

Inter-frequency prediction scenario

Agreement: - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

As per the RAN2 study, using a group of cells in one carrier frequency as input for prediction can improve RRM measurement accuracy compared to using a single cell. However, defining separate performance requirements for cluster-based and non-cluster-based approaches may not be practical or beneficial. Standardizing the number of cells in a cluster is challenging, as intra-frequency neighbour cells may not always be available due to varying network deployments, UE mobility, and real-time channel conditions.

Additionally, the decision to use a cluster-based or non-cluster-based prediction model should be left to the implementation. RAN4 should focus on defining the minimum requirements necessary to ensure accurate mobility event predictions.

Proposal 3: It is proposed that RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios.

Proposal 4: No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

Measurement event prediction use-cases

Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

- In Sub-use Case 2, L3 cell-level measurement results are predicted based on actual L3 cell level measurement results, primarily supporting Goal 2 by improving handover performance through better predictions. However, since real-time L3 measurements are still required for prediction, this approach does not inherently reduce the number of measurements (Goal 1). Similarly, in Sub-use Case 3, L3 cell-level results are predicted from actual L1 beam-level measurements, which also relies on real-time L1 measurements. While this approach enhances handover performance (Goal 2) by leveraging actual measurements, it does not reduce the number of measurements, as real-time L1 measurements collection remains necessary.

- When the AI/ML model forecasts a potential handover based on predicted L3 cell-level measurements, the network can take proactive actions, such as preparing for the handover in advance or deciding whether to proceed with the handover. However, both sub-use cases 2 and 3 with reliable prediction models enhances the performance of mobility management, but do not directly contribute to measurement reduction without additional strategies.

- RAN4 should explore the possibility of relaxing the L1 beam-level measurement requirement based on the confidence level of the L3 cell-level measurement prediction.

- Proposal 5: RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: Predicting multiple inter-frequency measurements can be challenging due to varying channel conditions across different frequencies

Proposal 1: RAN4 to clarify the difference between ground truth RSRP and reported RSRP

Proposal 2: RAN4 should explore potential enhancements in inter-frequency measurement prediction by combining with temporal domain prediction.

Proposal 3: It is proposed that RAN4 not study the impact of cluster-based and non-cluster-based inter-frequency prediction scenarios.

Proposal 4: No need to define separate requirement for clustered and non-clustered approach in inter frequency prediction scenario.

- Proposal 5: RAN4 should study the mobility enhancements based on L3 cell-level measurement predictions in conjunction with the relaxation of L1 beam-level measurements.

- References

- R4-2504901, WF on AI/ML Mobility, Moderator (Nokia), Wuhan, China, April 7th – April 11th, 2025.

- RP-250168, Status Report to TSG, Incheon, South Korea, March. 12-14, 2025

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Nokia, OPPO, MediaTek, Hefei, China, Oct 14 – Oct 18, 2024

- R4-2504917, Adhoc minutes for FS_NR_AIML_Mob, Nokia, Wuhan, China, April 7th – April 11th, 2025