3GPP TSG-RAN WG4 Meeting #115	R4-2506205

Malta, MT, 19 May 2025 - 23 May 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for on-demand SSB SCell operation

Agenda item:	7.27.3.1

Document for:	Discussion

- Introduction

At RAN4#114bis meeting, the R19 WI on network energy saving was further discussed and the agreements were captured in the WF [1]. In this contribution, we continue the discussion on the RRM impact from on-demand SSB SCell operation.

- OD-SSB based deactivated SCell measurement (Case 1)

In previous meetings, the UE measurement behaviour on the deactivated OD-SSB SCell was concluded. In general, a UE is expected to perform a fast measurement based on OD-SSB periodicity within a fast mode window (FMW) when OD-SSB is activated. Afterwards, UE would fall back to legacy deactivated SCell measurement for UE power saving. We will continue discussing the leftover issues in this section.

  - OD-SSB based deactivated SCell measurement in FMW

Regarding to the measurement requirement in FMW, it was agreed to introduce a lower bound in case some UEs are not able to measure each of the OD-SSBs when the OD-SSB periodicity is very short e.g. 5ms. But the value of the lower bound is still FFS as below.

Sub-issue 1: Lower bound for measurement in FMW Agreement Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value Sub-issue 3: OD-SSB in FMW collided with MG Wayforward - FFS UE is allowed to skip the OD-SSB measurement within FMW if the OD-SSB occasion in FMW collides with MG occasion.

In our view, UE should be able to handle the 5ms SSB periodicity as this has been assumed in SCell activation procedure. Even if some UEs may have legacy problems with L3 measurement, it should not be too far from the minimum OD-SSB periodicity. The lower bound can be derived based on 10ms OD-SSB periodicity and no further relaxation is allowed.

- If a UE indicates a lower bound is needed, the lower bound is derived based on the OD-SSB periodicity of 10ms and no further relaxation is allowed.

Another issue to be discussed is how to handle the collision between OD-SSBs in FMW and measurement gaps. In the discussion of FMW, the intention was to allow fast measurement so that network can deactivate the OD-SSB transmission for network energy saving. It was agreed to prioritize the OD-SSB based measurement in FWM over the measurement on legacy carriers i.e. CSSF=1. While measurement gaps are considered, the measurements in gaps are with even lower priority than the measurement outside gaps. Hence the OD-SSB based measurement in FWM shall also be prioritized over measurement gaps. UE is allowed to skip the measurement gaps if OD-SSB in FMW collides with MG occasions.

Observation 1: The measurements in gaps are considered with even lower priority than the measurement outside gaps.

- The OD-SSB based measurement in FMW is prioritized over measurement gaps. UE is allowed to skip the measurement gaps if OD-SSB in FMW collides with MG occasions.

  - OD-SSB based deactivated SCell measurement in FMW

On top of the measurement framework, companies further raised the issue of reactivation of OD-SSB and proposed to optimize the measurement based on a time period T2. There was no consensus on where T2 starts and ends so far.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure - Background - The deactivated SCell measurement requirement in FR1 is as follow. - DRX cycle; - T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. - The OD-SSB based deactivated SCell measurement procedure is shown as follow. Sub-issue 1: The start point of T2 Way forward The start point of T2 is - Option 1: - The time point of OD-SSB deactivation - Option 2: - The time point of Tend, i.e. the time point of OD-SSB activation plus Tuncertain +Tidentify. - Option 2a: - The time point of OD-SSB activation Sub-issue 2: The minimum interval(T2) between multiple FMWs Way forward - Option 1: 5* measCycleSCell - Option 1a: 5*(max(measCycleSCell, DRX cycle) – OD-SSB periodicity) - Option 1b: 5*max (measCycleSCell, DRX cycles) - Option 2: Minimum measurement period defined in existing spec. - Option 3: The time period of known Scell condition but with OD-SSB periodicity for FR1 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Way forward - Option 1: The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: The UE is assumed to perform fast mode measurement without cell identification period. - Option 3: No requirement for UE within T2.

Based on the agreements so far, when OD-SSB is activated, UE will perform fast measurement in FMW and fall back to the legacy deactivated SCell measurement. As long as the OD-SSB is not deactivated (explicitly or implicitly), UE will keep the legacy measurement based on measCycleSCell on the SCell. What remains open is the UE behaviour when OD-SSB is deactivated and again (re)activated. Hence the start point of T2 should be the time instance of OD-SSB deactivation.

Observation 2: As long as the OD-SSB is not deactivated (explicitly or implicitly), UE will keep the legacy deactivated SCell measurement based on measCycleSCell on the SCell.

- The start point of T2 is the time instance of OD-SSB deactivation (explicitly or implicitly).

When network decides to activate OD-SSB transmission, network is expecting UE to perform fast measurement on the SCell. This can be used for obtaining the first measurement reporting or updating the measurement result in a fast manner. If the OD-SSB has been activated recently i.e. OD-SSB is reactivated within a time period T2, UE may have just measured the SCell hence UE can perform fast measurement for a shorter time period for UE power saving i.e. a shorter fast mode window may apply for OD-SSB reactivation. And the shorter fast mode window can be set to the OD-SSB based measurement period i.e. not counting the cell identification time period.

Observation 3: Network is expecting UE to perform fast measurement on the SCell every time OD-SSB is activated.

- If the OD-SSB is reactivated within a time period T2, UE performs fast measurement for a shorter time period for power saving i.e. a shorter fast mode window may apply for OD-SSB reactivation.

- When OD-SSB is reactivated within a time period T2, the shorter fast mode window is the OD-SSB based measurement period i.e. not counting the cell identification time period.

Given above expected UE measurement behaviour, the introduction of T2 is to allow the UE to leverage the measurement results UE has obtained when OD-SSB was previously activated. As the UE will fall back to legacy deactivated SCell measurement after the fast measurement window, the value of T2 can be the same as the value used for SCell activation i.e. max(5*measCycleSCell,  5*DRX cycles).

- The value of T2 is max(5*measCycleSCell,  5*DRX cycles).

- OD-SSB based deactivated SCell measurement (Case 2)

In last meeting, the expected UE measurement behaviour was agreed when AO-SSB and OD-SSB are transmitted on the same frequency and with the same spatial relation i.e. Alt Time-C1. RAN1 further sent another LS R1-2501633 informing more scenarios to be supported. Referring to Fig.1, RAN1 further agreed to support Alt Time-C2 and scenario 3 in addition to Alt Time-C1. It is still open whether and how to define the measurement requirements for these scenarios.

Fig. 1 OD-SSB SCell operation in Case 2 and Scenario #2/#2A

In last meeting, the following was agreed on the measurement behaviour in scenario Alt Time-C1. With this principle, UE is not required to perform fast measurement in FWM if UE has sent a valid measurement reporting before OD-SSB activation. This leverages the measurement based on AO-SSB and minimizes the fast measurement for UE power saving.

Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreement - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue.

For Alt Time-C2 and scenario 3, despite the difference on time/frequency location of AO-SSB and OD-SSB, we believe the same measurement principle as agreed above can be well applied. The only difference is the measurement requirement in FMW in respective scenarios.

- For Alt Time-C2 and scenario 3, The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation within the time window as follow. Otherwise, the same approach in Case 1 can be reused.

      - max(5*measCycleSCell,  5*DRX cycles) for FR1, or

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

Measurement requirement in FMW for scenario Alt Time – C2

Regarding to the scenario Alt Time-C2, there were different options as cited below. In our view, it differs from Alt Time-C1 in that the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern. During the discussion, some UE vendors argued they may not be able to handle the non-periodic time distance hence not all the SSBs are measured. In this case, we can consider relaxing the measurement requirement in FMW by using the OD-SSB periodicity instead of the effective measurement periodicity considering both AO-SSB and OD-SSB. It is up to UE to determine which SSB to be measured but the measurement requirement is defined based on OD-SSB periodicity.

Issue 1-2-3: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: Define the requirement based on OD-SSB periodicity. - Option 2: Not to define requirement.

- For the measurement in FMW in scenario Alt Time-C2, the measurement requirement is defined based on OD-SSB periodicity.

Measurement requirement in FMW for Scenario 3 – AO-SSB and OD-SSB on different frequencies

RAN1 reply LS (R1-2501633): RAN1 discussed the relation in terms of time location between the always-on SSB and on-demand SSB and agreed the following in RAN1#120. Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted RAN2 LS R2-2503019: In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption: Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. RAN1#120 Agreement: Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SSB

Scenario 3 was also agreed in RAN1 where AO-SSB and OD-SSB are located in different frequencies within the same BWP. In last meeting, RAN2 agreed to introduce a new servingCellMO for OD-SSB in addition to the MO for AO-SSB. Since the measurement is performed per MeasId which links one MO with one report configuration, the measurement on AO-SSB and OD-SSB are separately handled hence the combination or union of AO-SSB and OD-SSB shall not be considered. Besides, RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB when OD-SSB is activated. To simplify the discussion, we can assume UE is required to measure either AO-SSB or OD-SSB at one time. For the measurement in FMW in scenario 3, the measurement requirement is defined based on OD-SSB periodicity.

Observation 4: According to RAN2, the measurement on AO-SSB and OD-SSB are separately handled hence the combination or union of AO-SSB and OD-SSB shall not be considered in scenario 3.

- For the measurement in FMW in scenario 3, the measurement requirement is defined based on OD-SSB periodicity.

- OD-SSB based SCell activation

Known/unknown condition

On SCell activation delay, the following was captured in the way forward.

RAN4#114bis: Issue 1-3-2: The known cell condition – period of known condition - Wayforward Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2. RAN4#113: Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

Another issue being raised is to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure. As discussed before, for the scenario where AO-SSB and OD-SSB are on the same frequency i.e. Alt Time-C1 and Alt Time-C2, as the known condition is based on the measurement reporting for the SCell, it does not matter the known condition is determined based on always-on SSB and/or OD-SSB and the SCell shall be activated as a known SCell.

Observation 5: If AO-SSB and OD-SSB are on the same frequency i.e. Alt Time-C1 and Alt Time-C2, the known condition is based on the measurement reporting for the SCell on the servingMO, it does not matter the known condition is determined based on always-on SSB and/or OD-SSB.

Even if the AO-SSB and OD-SSB are on different frequencies i.e. scenario 3, UE is not required to measure both SSBs according to RAN1. Since they are within the same BWP, we can assume there is not much difference between the measurements based on AO-SSBs and OD-SSBs. Although separate measurement reportings are configured for AO-SSB and OD-SSB, both of the measurement reports can be used to determine the known condition to facilitate the SCell activation. Hence the known condition can be determined based on either AO-SSB based measurement reporting or OD-SSB based measurement reporting.

Observation 6: If the AO-SSB and OD-SSB are on different frequencies i.e. scenario 3, separate measurement reportings may be triggered for AO-SSB and OD-SSB, but both of the measurement reports can be used to determine the known condition to facilitate the SCell activation.

In summary, for the OD-SSB based SCell activation in all the Case 2 scenarios, the known condition is determined based on the measurement reporting for the SCell irrespective of it is measured on AO-SSB or OD-SSB.

- For OD-SSB based SCell activation in all the Case 2 scenarios, the known condition is determined based on the measurement reporting for the SCell irrespective of it is measured on AO-SSB or OD-SSB.

OD-SSB based multiple SCell activation

Issue 2-1-1: OD-SSB based multiple SCells activation Wayforward - Option 1: - RAN4 to define OD-SSB based multiple SCells activation delay requirements. - The limitation is needed - It doesn’t impact the WI’s completion. - Option 2: - Not define multiple OD-SSB carriers requirement in this release.

According to existing specification, multiple SCell activation is specified in TS 38.133 clause 8.3.7 and it addresses the scenario where UE receives one single MAC command for multiple SCell activation. Following the same terminology, OD-SSB based multiple SCell activation means more than one OD-SSB SCell are to be activated via one single MAC command.

Observation 7: OD-SSB based multiple SCell activation addresses the scenario where more than one OD-SSB SCell are to be activated via one single MAC command.

If OD-SSB transmission are activated on more than one OD-SSB SCell, the UE may start the FMW in some of the SCells. If there are multiple OD-SSB SCells to be measured within the fast mode window the fast mode window, the number of OD-SSB SCells needs to be counted when deriving delay. However, it is a matter how many OD-SSB SCells are being measured in fast window and the fast windows may not always align in time. To account the workload of the discussion and finalize the work item, we suggest focusing on single OD-SSB SCell in this release, and only consider multiple simultaneously OD-SSB carriers if time allows or otherwise in a later release.

- Focus on single OD-SSB SCell and do not consider OD-SSB based multiple SCell activation in this release.

- General OD-SSB requirements

  - Processing time

Applicable scenarios

In last meeting, there were different proposals on the scenarios where additional processing time is needed but this was not discussed due to time limit.

Issue 1-4-1: Additional Processing time for OD-SSB reception - scenario - Proposal - Option 1: The processing/preparation time for OD-SSB reception shall not apply at least in the following cases: - If OD-SSB has been activated recently, or - If OD-SSB is transmitted on the same frequency with always-on SSB, or - OD-SSB for SCell activation - Option 2: The processing/preparation time for OD-SSB reception shall only apply in the following cases: - When OD-SSB carrier is always prioritized. - When OD-SSB carrier has same priority as legacy SSB carriers and no AO-SSB on the SCell (Case 1)

In our understanding, the UE processing time was raised because UE needs additional time to get prepared for receiving the first OD-SSB. It only applies to OD-SSB reception but not regular SSBs as OD-SSB may be activated at any unexpected time. But this also means that in some cases if the UE has been prepared when receiving OD-SSB activation command, the UE processing time may not be needed.

For Case 1, only OD-SSB may be transmitted in an SCell. The UE would likely turn off the receiver while OD-SSB is not activated. It may take time to prepare for the first OD-SSB reception after receiving activation indication. But if OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time.

For Case 2, always-on SSB are being transmitted in addition to OD-SSB in an SCell. The UE is required to keep monitoring always-on SSBs which may provide the reference for OD-SSB reception. For example, if OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

Observation 8: If OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time when it is activated again.

Observation 9: If OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

Hence, we would like RAN4 to discuss the applicability of the UE processing time. The processing/preparation time is needed only when necessary and it shall not apply at least in above cases.

- The processing/preparation time at OD-SSB activation is needed only when necessary and it shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB.

Value of processing time

Regarding to the length of the additional processing time, the following was agreed. We would like to note that we already relaxed the fast measurement window by one OD-SSB periodicity considering the UE implementation efforts. This additional time can also be used for UE to process the OD-SSB activation command and get ready to receive the first OD-SSB. Hence we would prefer a shorter additional processing time to avoid unnecessary relaxation.

Issue 1-4-2: Additional Processing time for OD-SSB reception - value Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

Observation 10: We already relaxed the fast measurement window by one OD-SSB periodicity considering the UE implementation efforts.

- A shorter additional processing time e.g. 2ms is preferred to avoid unnecessary relaxation.

Processing time for OD-SSB deactivation

In last meeting, RAN1 sent LS to RAN4 asking for confirmation on their working assumption regarding to the timeline when MAC-based OD-SSB deactivation is received. This issue was also raised by other companies in RAN4.

Issue 1-4-3: Additional Processing time for OD-SSB deactivation Wayforward - FFS RAN4 to consider defining the same timeline for activating and deactivating OD-SSB

In our view, the working assumption in the LS is the same as the timeline for receiving OD-SSB activation command. They are the same as it is relevant to the time to send HARQ and the time for processing the MAC command. RAN4 can send reply LS confirming their working assumption.

Different from the OD-SSB activation, we don’t think additional processing time is needed upon OD-SSB deactivation. The reason why additional processing time was introduced is the time for UE to get ready to receive the OD-SSB which are activated irregularly. However, this is not required when receiving OD-SSB deactivation. UE would be expected to release the resources for monitoring OD-SSB which should not count additional time. Therefore, the additional processing time is not needed upon OD-SSB deactivation.

- RAN4 to send reply LS to RAN1 confirming their working assumption.

- Additional processing time is not needed upon OD-SSB deactivation.

  - Interruption

In last meeting, it was agreed to introduce new interruption at OD-SSB activation/deactivation. Another aspect to be discussed is impact on existing interruption requirement due to OD-SSB.

In existing specification, interruption is allowed on active serving cells due to RF tuning during some RRM procedures e.g.

- 8.2.2.2.1 Interruptions at SCell addition/release: This defines the maximum allowed interruption during the RRC reconfiguration procedure for SCell addition/release. As UE needs to monitor SSB

- 8.2.2.2.3 Interruptions during measurements on deactivated SCC:  This defines the maximum allowed interruption due to measurements when an SCell is deactivated

In both clauses above, UE needs to tune on its RF in order to monitor SSBs for identifying or measuring the SCell. However, if the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted. Hence no interruption is allowed before OD-SSB is indicated when UE is commanded to add/release an OD-SSB SCell.

If the SCell is in OD-SSB operation in Case 2 i.e. with always-on SSB, UE needs to tune the RF in order to receive at least always-on SSB. Existing interruption requirement can still apply. In any case, the interruption requirements during SCell addition/release need to be updated at least for OD-SSB SCell in Case 1.

Observation 11: If the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted.

- No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

Similarly, interruption shall not be allowed during the measurements on deactivated SCC before OD-SSB is indicated. This applies if the SCell is in OD-SSB operation in Case 1.

- No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

  - L1 measurement in OD-SSB SCell

One issue brought into discussion in last meeting is the scheduling restriction and measurement restriction for L1 measurement in OD-SSB SCell in case 2.

Issue 1-4-6: Scheduling restriction for L1 measurement after SCell activation Wayforward - FFS In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction are expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

We understand this depends on how the UE is expected to perform L1 measurement after SCell activation. RAN1 has made the following agreement regarding to CSI report.

RAN1#120bis Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

If AO-SSB and OD-SSB are on the same frequency, RAN1 agreed which SSB to measure is up to UE implementation as long as RAN4 requirements are fulfilled without differentiating Alt Time-C1 from Alt Time-C2. As we agreed to follow legacy requirement for the L1 measurement in Case 1, the same principle can apply to Case 2. The question in Case 2 is which SSB periodicity is used to determine the L1 measurement period in addition to DRX cycle. In our view, we can define the L1 measurement period in the similar way as L3 measurement in respective scenarios. That is, if AO-SSB and OD-SSB are in the same frequency,

- Alt Time-C1: define the L1 measurement requirements based on union of AO-SSB and OD-SSB and DRX cycle.

- Alt Time-C2: define the L1 measurement requirements based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- Scenario 3: define the L1 measurement requirements based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- If AO-SSB and OD-SSB are on the same frequency, the L1 measurement requirement is defined as the following:

- Alt Time-C1: based on union of AO-SSB and OD-SSB and DRX cycle.

- Alt Time-C2: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- Scenario 3: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

With the measurement requirement above, no scheduling restriction or measurement restriction is allowed for AO-SSB occasions if UE only measures OD-SSB based L1-RSRP measurement when OD-SSB is activated.

- In Case 2, if AO-SSB and OD-SSB are on the same frequency, there is no scheduling restriction or measurement restriction for AO-SSB occasions if UE only measures OD-SSB based L1-RSRP measurement when OD-SSB is activated.

If AO-SSB and OD-SSB are on different frequencies, RAN1 didn’t reach any conclusion. We can wait for the RAN1 progress before we define the measurement requirement.

- If AO-SSB and OD-SSB are on different frequencies, RAN4 to wait for RAN1 progress on the CSI report discussion.

OD-SSB impact on neighbor cell measurement

About neighbor cell measurement, it was in general concluded to reuse the legacy measurement requirements if there is OD-SSB SCell or neighbor cell in the MO. But for the scenarios where OD-SSB is an SCell, the conclusion only applies to FR1 and further discussion is needed for FR2.

Issue 7-2: OD-SSB based Neighbor cell measurement - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbor cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbor cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbor cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

In previous meetings, it was agreed a UE is not required to measure OD-SSB before OD-SSB is activated in the SCell. Hence the measurement requirement for the deactivated OD-SSB SCell (being discussed in section 3 and 4) is applicable only if OD-SSB is activated. But this shall not impact the measurement on the neighbor cells in the MO. The UE is required to perform the intra-frequency measurement on the serving MO and existing measurement requirement for neighbor cells on the MO still apply. The same principle shall be applied to both FR1 and FR2.

- If there is an OD-SSB SCell on the serving CC, the legacy requirement for neighbour cell measurement applies for both FR1 and FR2.

- Conclusion

In this contribution, we discussed the RRM requirements for OD-SSB SCell operation based on RAN1/2 progress and presented our views on the potential RAN4 impact. The following observations and proposals are made:

Observation 1: The measurements in gaps are considered with even lower priority than the measurement outside gaps.

- If a UE indicates a lower bound is needed, the lower bound is derived based on the OD-SSB periodicity of 10ms and no further relaxation is allowed.

- The OD-SSB based measurement in FMW is prioritized over measurement gaps. UE is allowed to skip the measurement gaps if OD-SSB in FMW collides with MG occasions.

Observation 2: As long as the OD-SSB is not deactivated (explicitly or implicitly), UE will keep the legacy deactivated SCell measurement based on measCycleSCell on the SCell.

- The start point of T2 is the time instance of OD-SSB deactivation (explicitly or implicitly).

Observation 3: Network is expecting UE to perform fast measurement on the SCell every time OD-SSB is activated.

- If the OD-SSB is reactivated within a time period T2, UE performs fast measurement for a shorter time period for power saving i.e. a shorter fast mode window may apply for OD-SSB reactivation.

- When OD-SSB is reactivated within a time period T2, the shorter fast mode window is the OD-SSB based measurement period i.e. not counting the cell identification time period.

- The value of T2 is max(5*measCycleSCell,  5*DRX cycles).

- For Alt Time-C2 and scenario 3, The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation within the time window as follow. Otherwise, the same approach in Case 1 can be reused.

- max(5*measCycleSCell,  5*DRX cycles) for FR1, or

- 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

- For the measurement in FMW in scenario Alt Time-C2, the measurement requirement is defined based on OD-SSB periodicity.

Observation 4: According to RAN2, the measurement on AO-SSB and OD-SSB are separately handled hence the combination or union of AO-SSB and OD-SSB shall not be considered in scenario 3.

- For the measurement in FMW in scenario 3, the measurement requirement is defined based on OD-SSB periodicity.

Observation 5: If AO-SSB and OD-SSB are on the same frequency i.e. Alt Time-C1 and Alt Time-C2, the known condition is based on the measurement reporting for the SCell on the servingMO, it does not matter the known condition is determined based on always-on SSB and/or OD-SSB.

Observation 6: If the AO-SSB and OD-SSB are on different frequencies i.e. scenario 3, separate measurement reportings may be triggered for AO-SSB and OD-SSB, but both of the measurement reports can be used to determine the known condition to facilitate the SCell activation.

- For OD-SSB based SCell activation in all the Case 2 scenarios, the known condition is determined based on the measurement reporting for the SCell irrespective of it is measured on AO-SSB or OD-SSB.

Observation 7: OD-SSB based multiple SCell activation addresses the scenario where more than one OD-SSB SCell are to be activated via one single MAC command.

- Focus on single OD-SSB SCell and do not consider OD-SSB based multiple SCell activation in this release.

Observation 8: If OD-SSB has been activated recently, the UE may be able to receive the OD-SSB without additional processing time when it is activated again.

Observation 9: If OD-SSB is transmitted on the same frequency with always-on SSB, the UE receiver setting used for always-on SSB can be well applied to OD-SSB reception hence the additional processing time may be also saved.

- The processing/preparation time at OD-SSB activation is needed only when necessary and it shall not apply at least in the following cases:

- If OD-SSB has been activated recently, or

- If OD-SSB is transmitted on the same frequency with always-on SSB.

Observation 10: We already relaxed the fast measurement window by one OD-SSB periodicity considering the UE implementation efforts.

- A shorter additional processing time e.g. 2ms is preferred to avoid unnecessary relaxation.

- RAN4 to send reply LS to RAN1 confirming their working assumption.

- Additional processing time is not needed upon OD-SSB deactivation.

Observation 11: If the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted.

- No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

- No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

- If AO-SSB and OD-SSB are on the same frequency, the L1 measurement requirement is defined as the following:

- Alt Time-C1: based on union of AO-SSB and OD-SSB and DRX cycle.

- Alt Time-C2: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- Scenario 3: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- In Case 2, if AO-SSB and OD-SSB are on the same frequency, there is no scheduling restriction or measurement restriction for AO-SSB occasions if UE only measures OD-SSB based L1-RSRP measurement when OD-SSB is activated.

- If AO-SSB and OD-SSB are on different frequencies, RAN4 to wait for RAN1 progress on the CSI report discussion.

- If there is an OD-SSB SCell on the serving CC, the legacy requirement for neighbour cell measurement applies for both FR1 and FR2.

- References

[1]	R4-2504906, “WF on Netw_Energy_NR_enh”, Ericsson