3GPP TSG-RAN WG4 Meeting #115	R4-R4-2506214

Malta, MT, May 19 – 23, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Agenda item:	7.18.3.2

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. At RAN4#114 meeting, the optimized CSSF solutions were mostly concluded but some details are still FFS in [1]. In this contribution we will continue discussing the leftover issues.

Solutions 1: measuring one serving CC per band

Revision on solution 1

In last meeting, there was a proposal to remove one condition where UE shall follow the network indicated SCC to be measured. It was argued the measurement shall be always configured with reporting hence the condition “if none of SCCs is configured with SSB based measurement reporting” is not a valid case.

Issue 1-2-1: revision on solution 1 (measure one serving CC per band) Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618. - Proposal 1 (OPPO): For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

According to existing measurement requirements in FR2, when neither PCC nor PSCC is in the same band, the UE will select one of SCCs on which UE is configured to report SSB based measurements if such SCC exists. Otherwise, which means there is no SCC on which UE is configured to report SSB based measurements, the selected SCC is up to UE implementation. This means UE is still required to select and measure one SCC on the band but it is not testable as no measurement reporting is configured.

Observation #1: In legacy requirements, when there is no SCC on which UE is configured to report SSB based measurement, UE is still required to select and measure one SCC on the band.

TS 38.133 clause 9.2.3.2:

9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

Similarly, when optimized measurement is enabled and network indicates which SCC to measure on the band, UE is required to measure the indicated SCC. This should be independent from if UE is configured with SSB-based measurement reporting. It may not be testable if UE is not configured with SSB-based measurement reporting on the indicated SCC, but the rule for determining the selected SCC remains valid. Hence, the above case where UE follows network indication shall not be removed. UE shall follow network indication to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band.

- UE shall follow network indication to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band.

Solution 1 in FR1 scenarios

At RAN4#113 meeting, it was agreed to further discuss if the CSSF optimization solutions can be extended to FR1 after it is concluded in FR2. In last meeting, there was concern from companies about the feasibility of such extension.

RAN4#113 meeting: Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable RAN4#114bis meeting: Issue 1-2-2: Expand solution 1 to FR1 case [Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

In our understanding, collocation deployment has been assumed for FR1 intra-band contiguous CA. In the discussion of SCell activation, we have been assuming the same timing and channel status among contiguous carriers in one FR1 band. Taking SSB-less SCell activation as an example, the SSB-less SCell can be activated immediately (i.e. 3ms) if there is at least one contiguous active serving cell in the same band. This shows the very good synergy among the intra-band contiguous carriers in FR1. Hence, we believe the same principle of solution 1 can be applied at least to FR1 collocated intra-band contiguous CA. For other FR1 collocated scenarios, we may leave it out from this WI for now considering the timeline.

Observation #2: The SSB-less SCell can be activated immediately (i.e. 3ms) if there is at least one contiguous active serving cell in the same band. This shows the very good synergy among the intra-band contiguous carriers in FR1

- Extend solution 1 to FR1 collocated intra-band contiguous CA.

In addition, to allow flexible control on the optimized measurement, network may indicate to enable or disable this CSSF enhancement for FR1 and FR2 separately.

- Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

In existing specifications, UE is already allowed to skip the neighbor cell measurement on some SCCs in one FR2 band as specified in TS 9.2.3.2. But this does not apply to the measurement on FR1 i.e. UE is required to measure both serving and neighbor cells in one FR1 carrier in the same way. When solution 1 is enabled for FR1, UE only needs to measure one serving CC per FR1 contiguous band as these serving cells are collocated. But it is open how and if the UE is allowed to skip the neighbor cells measurement. In our view, it would be simplest if the UE performs both serving and neighbor cell measurements on single SCC on one band. We are open to discussing this issue.

Observation #3: In existing specifications, UE is not allowed to skip the neighbor cell measurement on some SCCs in one FR1 band.

- RAN4 to discuss how to handle the neighbor cell measurement when network indicates to enable solution 1 for FR1 bands.

Solution 3: 3-searcher solution

On the 3-searcher solution, it was further proposed to restrict the applicability of 3-searcher if the per-FR gap in one FR is colliding with the SMTC occasions in the other FR. It was argued this would lead to 4 searchers in practice hence require higher measurement processing capability.

In our view, the same issue exists in legacy measurement requirements with 2 searcher assumption. According to TS 38.133 clause 9.1.5, CSSFoutside_gap,i and CSSFwithin_gap,I are specified separately without any exception regarding to the  SMTC and per-FR gap configuration.

Observation #4: In current specification, CSSFoutside_gap,i and CSSFwithin_gap,i are specified separately without any exception regarding to the SMTC and per-FR gap configuration.

In addition, the following applicability is defined for the CSSF outside gaps. It clearly states it is applied when none/part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap. The measurement gap can be also understood as the per-FR measurement gap. Hence, the concerned scenario “when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR” has been excluded by the existing applicability condition.

Therefore, we don’t see any additional applicability is needed for 3-searcher solution. If the UE indicates the capability of supporting 3-searcher solution, the new CSSF values shall be applied with existing applicability conditions.

TS 38.133 clause 9.1.5.1:

Otherwise, the carrier-specific scaling factor CSSFoutside_gap,i for measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement without measurement gap as defined in clauses 9.2.1 and 9.2A.1, when none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based intra-frequency measurement without measurement gap as defined in clauses 9.2.1 and 9.2A.1, when part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

Observation #5: The concerned scenario “when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR” has been excluded by the existing applicability condition.

- Do not introduce additional applicability about per-FR gaps for 3-searcher solution.

Conclusion

The following observations and proposals were made in this paper:

Observation #1: In legacy requirements, when there is no SCC on which UE is configured to report SSB based measurement, UE is still required to select and measure one SCC on the band.

- UE shall follow network indication to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band.

Observation #2: The SSB-less SCell can be activated immediately (i.e. 3ms) if there is at least one contiguous active serving cell in the same band. This shows the very good synergy among the intra-band contiguous carriers in FR1.

- Extend solution 1 to FR1 collocated intra-band contiguous CA.

- Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

Observation #3: In existing specifications, UE is not allowed to skip the neighbor cell measurement on some SCCs in one FR1 band.

- RAN4 to discuss how to handle the neighbor cell measurement when network indicates to enable solution 1 for FR1 bands.

- Observation #4: In current specification, CSSFoutside_gap,i and CSSFwithin_gap,i are specified separately without any exception regarding to the SMTC and per-FR gap configuration.

- Observation #5: The concerned scenario “when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR” has been excluded by the existing applicability condition.

- Do not introduce additional applicability about per-FR gaps for 3-searcher solution.

- References

- R4-2503874, Way Forward for NR_RRM_Ph5_Part1, Apple