3GPP TSG-RAN WG4 Meeting #114bis				             R4-2506225

Malta, MT, 19th – 23rd May, 2025

Source: 	China Telecom

Title: 	Views on Power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Document for:	Discussion

Introduction

In meeting RAN4#114-bis, we made an agreement on default MPR reduction method and had some progress on the corresponding requirements about RB extension [1].

Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting

So in this contribution we bring our perspective on Fixed or flexible extension, and the method of converting inner RB allocation to outer RB allocation.

Discussion

Compromised solution for flexible RB extension with capability

As we provided in our last paper, from operator’s perspective, not all scenarios can satisfy 0.5* UE CBW extension and there should be a solution to reduce MPR when the extension space of spectrum is limited while not sacrifice transmit bandwidth. According to the agreements of the last meeting, setting a limited number of extension ratio less than 0.5 is helpful for application of this feature.

Observation1: Setting a limited number of extension ratio less than 0.5 is helpful for application of this feature.

As a compromised solution, here we provide two modes to utilize the limited space of spectrum when UE support the flexible RB extension capability.

- Fixed extension range

RB extension range should be one of bandwidth in table 5.3.2-1in TS 38101-1 and the selectable extension value should be less than space of spectrum for relaxation on each side and less than 0.5*NRB.

- Fixed extension ratio

Fixed extension ratio (such as 1/4 1/6) with NW indication.

Proposal1: Considering two modes of compromised solution for flexible extension.

- Fixed extension range

RB extension range should be one of bandwidth in table 5.3.2-1in TS 38101-1 and the selectable extension value should be less than space of spectrum for relaxation on each side and less than 0.5*NRB.

- Fixed extension ratio

Fixed extension ratio (such as 1/4 1/6) with NW indication.

Guard band for extended CBW

Generally, we support option 1 of GB for extended CBW part in WF [1].

According to the agreements in the last meetings, the IBE is used in extend CBW from initial CBW edge while OOBE is used outside the extend CBW. The extended CBW is only for MPR calculating and this feature can be treated as utilizing OOBE relaxation by shifting ACLR/SEM application range to reduce MPR. So the extended CBW together with GB of initial CBW act as a global extended GB where any UE cannot be allocated and we also should add IBE in this range as compensation.

Proposal2: The extended CBW together with GB of initial CBW act as a global extended GB where any UE cannot be allocated and we also should add IBE in this range as compensation.

Derivation of new inner RB region in extended CBW

As defined in 38.101-1, the formula of determining RB allocation can be treated as a function that NRB and LCRB are input values and RB allocation region is the output value. Any effort about converting inner to outer will concentrate on this formula eventually since the RB allocation region only depends on it. So if we want to reflect the new inner RB allocation in spec, we can introduce new input value to this formula.

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = max(1, floor(LCRB/2)) where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x. RBStart,High = NRB – RBStart,Low – LCRB The RB allocation is an Inner RB allocation if the following conditions are met RBStart,Low ≤ RBStart ≤ RBStart,High, and LCRB ≤ ceil(NRB/2) where ceil(x) is the smallest integer greater than or equal to x.

The extended UE CBW actually influence the NRB in the above formula, so the RB shift values on each side should be reflected in any part of the formula contains NRB. For two side extension, the UE will have two values: RBshift_low and RBshift_high, which denote the numbers of RB extension on the left and right side of UE CBW. And the new RB number can be expressed as:

NRB_new = RBshift_low + NRB + RBshift_high

The details of formula of determining RB allocation may be different when the coordinate system origin of RB_start is different. The coordinate system origin of RB_start can base on extended NRB_new or original NRB so here we provide two ways of determining RB allocation.

- NRB based RB_start coordinate system origin

The NRB based RB_start coordinate system origin can be expressed in Fig.1.

Fig.1. NRB based RB_start coordinate system origin

The formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

The max() and min() at the outermost layer in RBStart,Low_new and RBStart,Low_new are to make sure the new inner region is the intersection of the inner MPR region of the extended UE CBW and the original MPR region based on the UE CBW.

Proposal3: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

- RBshift_low, RBshift_high ≤ NRB/2

- NRB_new based coordinate system origin

The NRB_bew based RB_start coordinate system origin can be expressed in Fig.1.

Fig.2. NRB_new based RB_start coordinate system origin

The inner RB allocation region that we expect is the intersection of red dotted line triangle and yellow triangle. So the formula of new inner region can be changed as below:

- RBStart,Low_new = max(RBshift_low, RBStart,Low);

- RBStart,High_new = min(NRB + RBshift_low – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

Proposal4: For NRB_new based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(RBshift_low, RBStart,Low);

- RBStart,High_new = min(NRB + RBshift_low – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

Besides, here we offer a feasible solution to rewrite the formula and we also do not exclude other options and we are open to discuss the details.

Proposal5:

Signalling

Technically speaking, there is no difference between MPR reduction approach for scenario 1 and scenario 2. From UE side different scenarios can only influence the space of spectrum for extension. So no need to distinguish UE capabilities for scenario 1 and scenario 2 but we should indicate whether UE supports one fixed value (0.5*NRB) capability or multiple values of fixed extension capability.

Proposal5: No need to distinguish UE capabilities for scenario 1 and scenario 2. Indicate whether UE supports one fixed value (0.5*NRB) capability or multiple values of fixed extension capability.

Conclusion

Observation1: Setting a limited number of extension ratio less than 0.5 is helpful for application of this feature.

Proposal1: Considering two modes of compromised solution for flexible extension.

- Fixed extension range

RB extension range should be one of bandwidth in table 5.3.2-1in TS 38101-1 and the selectable extension value should be less than space of spectrum for relaxation on each side and less than 0.5*NRB.

- Fixed extension ratio

Fixed extension ratio (such as 1/4 1/6) with NW indication.

Proposal2: The extended CBW together with GB of initial CBW act as a global extended GB where any UE cannot be allocated and we also should add IBE in this range as compensation.

Proposal3: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

- RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

- RBshift_low, RBshift_high ≤ NRB/2

Proposal4: For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

- RBStart,Low_new = max(RBshift_low, RBStart,Low);

- RBStart,High_new = min(NRB + RBshift_low – LCRB, NRB + RBshift_low + RBshift_high – RBStart,Low_new – LCRB);

- RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2)；

- RBshift_low, RBshift_high ≤ NRB/2

Proposal5: No need to distinguish UE capabilities for scenario 1 and scenario 2. Indicate whether UE supports one fixed value (0.5*NRB) capability or multiple values of fixed extension capability.

References

- R4-2505099 WF on power domain enhancement, RAN4#114-bis