3GPP TSG-RAN WG4 Meeting #115				             R4-2506226

Malta, MT, 19th – 23rd May, 2025

Source: 	China Telecom

Title: 	Discussion on remaining issues on SRS lL imbalance

Agenda Item:	7.1.2.3.4

Document for:	Discussion

Introduction

In RAN4#114-bis, we have reached some further agreements on SRS IL imbalance issue and the main works needed to be done is listed as follows in WF [1].

- Sub-topic 4-2: SRS IL imbalance issue solutions Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

So in this paper we reiterate our view and also try to provide some compromised options.

Discussion

As the two solutions for SRS IL issue (UE capability indication and Type 3 PHR extension for PUSCH) were mentioned in last WF [1], we should consider utilizing them together to get better performance. Pre-compensation at UE side and post-compensation at BS side could both benefit to SRS power balance.

Proposal1: Consider introducing UE indication and Type 3 PHR together for SRS IL imbalance improvement.

For UE self-compensation, we should consider specifying the trigger condition at UE side and it’s not reasonable to depend entirely on the implementation of UE. The output power of SRS transmission is the minimum value of PCMAX,f,c and the actually calculated SRS power (denoted by PSRS_cal) as defined in 38.213.

While the insertion loss, i.e. the ∆TRxSRS can only influence the PCMAX,f,c part so the self-compensation should not be triggered unless the following equation is satisfied:

PCMAX,f,c - PSRS_cal ≤ ∆TRxSRS

Where PCMAX,f,c is not considering IL. In another word, the compensation will execute on PCMAX,f,c and only when the gap between the original PCMAX,f,c and PSRS_cal is small enough that the behavior is needed. So if the self-compensation is totally determined at UE side, it should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Proposal2: If the self-compensation is totally determined at UE side, it should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power.

For indication on whether UE performs SRS IL self-compensation, only timely information is meaningful for NW doing post-compensation so the indication should be dynamic and triggered when the UE performing self-compensation. To convey the indication to NW we should introduce a new IE which only need 1bit to carry a 0/1 flag, and this behavior also align with the agreement that no RAN1/4 impact.

Proposal3: The indication should be dynamic and triggered when the UE performing self-compensation.

Proposal4: Introduce a new IE to convey the indication from UE to NW.

For Type 3 PHR, from RAN2 side, the impact of specification would be quiet slight that we only need to care about the reporting granularity and trigger condition.

However, type 3 PHR was only introduced in multiple entry PHR, i.e. the CA/EN-DC scenarios and single cell can only report type 1 PHR as specified in TS 38.321 as below. So if this approach is adopted, we need to inform RAN2 to add Type 3 PHR to single cell scenario.

Single entry PHR MAC CE

Multiple entry PHR MAC CE

Proposal5: Inform RAN2 to add Type 3 PHR to single cell scenario.

At last but not the least, we still need to make a little change to Type 3 PHR at RAN1 to carry PUSCH information. The enhanced Type 3 PHR can be controlled by UE capability or configuration from NW and the change is small it just need to add a few wording. So although we have the agreement that no RAN1 impact, we should still ask RAN1 whether it can be done and minimum the impact. So we should send LS to RAN1 and ask whether Type 3 PHR could be enhanced for PUSCH cell while controlled by signaling.

Proposal6: send LS to RAN1 and ask whether Type 3 PHR could be enhanced for PUSCH cell in this feature while controlled by signaling.

Conclusion

Proposal1: Consider introducing UE indication and Type 3 PHR together for SRS IL imbalance improvement.

Proposal2: If the self-compensation is totally determined at UE side, it should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power.

Proposal3: The indication should be dynamic and triggered when the UE performing self-compensation.

Proposal4: Introduce a new IE to convey the indication from UE to NW.

Proposal5: Inform RAN2 to add Type 3 PHR to single cell scenario.

Proposal6: send LS to RAN1 and ask whether Type 3 PHR could be enhanced for PUSCH cell in this feature while controlled by signaling.

References

- R4-2505101, WF on for 6Rx UE requirements, RAN4#114-bis, Intel