# 3GPP TSG-RAN WG4 Meeting # 115		  R4-2506233

# Malta, MT, 19th – 23rd May, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	Discussion on SAN RF requirements for supporting new SDO band for LTE-based 5G Broadcast

# Agenda Item:	7.9.5

# Document for:	Approval

- Introduction

In RAN#107 meeting, a new Work Item [1] was approved on introduction of NTN broadcast band with the following RAN4 objective. In this contribution, we want to share some further views about NTN broadcast band from SAN RF perspective.

Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

- Discussion

Issue 4-1-3: Total power dynamic range - Agreements - FFS on whether total power dynamic range requirement applies for LTE based 5G broadcast over Satellite

After RAN4#114bis, a good progress has been made for SAN RF requirements, and there is one issue about total power dynamic range left to be further studied. The total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition. For Rel-18 LTE based 5G terrestrial broadcast base stations, it is stated that total power dynamic requirements is not needed as the transmit power for a broadcast BS can always keep a high level. So in our opinion, this principle is also applied for NTN broadcast.

Proposal 1: Total power dynamic range requirement is not applied for LTE based 5G broadcast over Satellite.

- Conclusion

In this contribution, some views on SAN RF requirements are provided and the proposals is made as follows:

Proposal 1: Total power dynamic range requirement is not applied for LTE based 5G broadcast over Satellite.

- Reference

RP-250788, New band for 5G Broadcast Utilizing Geosynchronous Satellite.

R4-2504725, WF for 114bis318 5G_Broadcast_GSO_NTN_band, Astrum Mobile, Qualcomm, ZTE, Huawei

3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access"

3GPP TS 36.108: “Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception”.

3GPP TS 38.108: “NR; Satellite Access Node radio transmission and reception”.