3GPP TSG-RAN WG4 Meeting #115	R4-2506236

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Low Band CA operation

Agenda item:	7.7.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM issues for low band CA via switching between FDD and SDL CC based on the WF [1] and RAN1 agreements on switching pattern below.

RAN1 agreement Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Discussion

General

SSB assumptions for R19 LB CA via switching

In previous meeting, following agreement has been achieved for assumption for LB CA

Agreement in RAN4#114 - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting.

Regarding SSB-based or SSB-less SCell operation, it is better to specify both requirement. SSB-based SCell operation is default operation, furthermore, deployment assumption (e.g. co-located and synchronized network for both carriers) can be leveraged for SSB-less SCell operation.

- Proposal 1: RAN4 to specify SSB-based SCell operation as baseline, and also specify SSB-less SCell operation.

MRTD assumptions for R19 LB CA via switching

- Option 1: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3: MRTD = CP

The deployment for LB CA via switching is co-located and synchronized network, so the concept of existing MRTD could be applied identically to maximum receive timing difference for LB CA. The maximum receive timing difference for LB CA should be 3us.

- Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Switching delay or interruption requirement

Based on RAN1 agreements in the last meeting, switching pattern and switching gap would be operated based on UE DL reception timing of PCell. The switching gap is located at the “switching from” carrier for switching from SDL to FDD carrier, and in this case, switching gap duration can cover at least the switching period and UL TA. It means that the there would be no interruption of PCell uplink transmission. Therefore, no interruption requireiments is required for PCell UL. For switching from PCell to SDL, RAN1 is still discussing the switching gap location, but since switching pattern and switching gap is based on PCell DL timing, there are no interruption for both DL and UL. Therefore, RAN4 does not need to introduce interruption requirements for LB CA vis switching.

- Proposal 3: RAN4 not to introduce interruption requirements for LB CA via switching oepration

SDL SCell related RRM requirements

Activated SDL SCell measurement requirement

Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied.

For activated SDL SCell, UE does not need to perform L3 measurement since there would be no only SCell change without PCell handover for LB CA via switching scenario. However, RAN4 still needs to consider BFD/CBD measurements for SDL SCell. For BFD/CBD requirements, UE performs the BFD/CBD measurement only during the SDL SCell reception duration by switching pattern. Therefore, the requirements should consider SDL SCell reception duration/periodicity by configured switching pattern.

- Proposal 4: RAN4 not to introduce L3 measurement requirements for activated SDL SCell measurement for mobility

- Proposal 5: RAN4 to introduce BFD/CBD measurement requirements based on SDL SCell reception duration/periodicity by configured switching pattern, 
e.g., introducing scaling factor (PLBCA) to existing requireiments

- PLBCA = Ntotal / Navailable

- For a window W of duration max(TL1, MGRP_MAX, Tswitching_pattern), where TL1 is periodicity of the target x-RS, and Tswitching_pattern is the periodicity of semi-static switching pattern, and MGRP_MAX is maximum measurement gap repetition period across per-UE GAPs or per-FR GAPs

- Ntotal is the number of x-RS resource occasions within the window W, and

- Navailable is the number of x-RS resource occasions that are not overlapped with any non-dropped GAP occasion, , nor PCell duration of semi-static switching pattern within window W.

Deactivated SCell measurement requirement

Agreement: - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement.

RAN4 agreed that the switching pattern is not applied to deactivated SCell measurement. For LB CA via switching, single RF chain is assumed for inter-band CA operation. The behavior for deactivated SCell measurement would differ from that of existing inter-band CA which is based on separate RF chains. Therefore, even if it is deactivated SCell measurement, RAN4 needs to consider measurement gap configuration for deactivated SCell measurement considering RF retuning in inter-band LB CA via switching operation, and NW can configure legacy MG pattern taken into account measCycleSCell for deactivated SDL SCell.

- Proposal 6: Due to inter-band CA considering a single RF chain, deactivated SDL SCell measurement should be performed within the measurement gap configured by NW.

SDL SCell activation/deactivation

Agreement: - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS

RAN4 agreed that the switching pattern is not applied to SCell activation procedure. For SDL activation delay, after SCell activation command is received, the UE has to switch to SDL SCell after the HARQ-ACK feedback + MAC-CE processing to complete SCell activation, and the UE would perform SDL SCell measurements before transmitting valid CSI report in PCell. Therfore, a certain SCell activation window (e.g., Tactivation_window) that can guarantee the UE to complete SCell measurements is required. During the SCell activation window, UE does not require to transmit and receive the signal to/from PCell. The SCell activation delay requirements for SDL SCell in LB CA could resue existing SCell activation delay requirements in FR1.

- Proposal 7: RAN4 to specify SCell activation window which ensures SCell measurements of the deactivated SCell.

- Proposal 8: Existing SCell activation / deactivation delay requirements (based on SSB or SSB-less) can be rused for SDL SCell activation / deactivation delay of low band CA vis switching

A UE would stay on PCell after compeleting SCell activation since the UE needs to report valid CSI on PCell. So, if the switching pattern for SDL SCell is ON (i.e., bitmap of switching pattern set to ‘1’) when UE reports valid CSI for SCell activation, there may be some ambiguity in the behavior of whether the UE should switch to SDL SCell according to the switching pattern. Therefore, after compeleting SCell activation, RAN4 needs to clarify when to apply the semi-static switching pattern.

- Proposal 9: RAN4 needs to clarify when to apply the semi-static switching pattern after compeleting SCell activation, e.g., apply immediately the semi-static switching pattern or apply next cycle semi-static switching pattern

BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

Regarding BFD/CBD, there can be an issue when the UE declare beam failure in SDL SCell. Usually, a UE evaluate DL beam quality of serving cell utilizing RS set in q0. If every the DL beam quality of RS in q0 are below Qout_LR, physical layer sends BFI (beam failure indication) to higher layer. If the certain number of BFI is occurred within cerain time window, the UE can declare beam failure, and try to transmit PRACH or PUCCH (if schedulingRequestIDForBFR is configured). After transmit PRACH/PUCCH, the UE can transmit MAC-CE providing index of BFD-RS and CBD-RS for a corresponding SCell. The problem is when the UE needs to transmit PRACH/PUCCH, there is no UL symbol in SDL SCell.

- Observation 1: When beam failure is declared in SDL SCell, no UL resource exists to transmit PRACH/PUCCH

We think it is natural to switch back to PCell in case of beam failure. For instance, the UE can switch to PCell right after the UE declares beam failure, or before transmitting PRACH/PUCCH considering RF switching delay. Furthermore, in order to perform beam failure recovery process properly, it would be helpful for the UE to pause the semi-static switching pattern during initial access, and resume the pattern when beam failure recovery is completed. Hence, RAN4 need to specify requirement for beam failure recovery procedure in SDL SCell.

- Proposal 10: RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell.

Impact on PCell RRM requirements

In general, NW can configure PCell SMTC, and RS for RLM/L1 irrelevant to SCell. Although semi-static pattern has not been specified yet, there can be a case that PCell SMTC/RS can be overlapped with SCell DL reception as illustrated in Figure 2, where periodic RS in PCell, and PCell SMTC are collided with sem-static switching pattern in SDL SCell. For RAN1 agreements for semi-static switching pattern, it is periodic with periodicity of P slots(ms), and P can be 40ms. Based on this, network could consider the periodicity of SMTC/MG/RS to align the periodicity of switching pattern. But RAN4 needs to wait for RAN1 final agreements for semi-static switching pattern, and RAN4 needs to discuss impact on PCell measurement due to overlapping between SCell reception and SMTC/MG/RS occasion of PCell.

- Proposal 11: RAN4 to wait for RAN1 final agreements for semi-static switching pattern design, then discuss impact on PCell measurement due to overlapping between SCell reception and SMTC/MG/RS occasion of PCell.

Conclusion

In this contribution, we provide our initial views on RRM issues for low band CA via switching between FDD and SDL CC, and we propose

- Proposal 1: RAN4 to specify SSB-based SCell operation as baseline, and also specify SSB-less SCell operation.

- Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

- Proposal 3: RAN4 not to introduce interruption requirements for LB CA via switching oepration

- Proposal 4: RAN4 not to introduce L3 measurement requirements for activated SDL SCell measurement for mobility

- Proposal 5: RAN4 to introduce BFD/CBD measurement requirements based on SDL SCell reception duration/periodicity by configured switching pattern, 
e.g., introducing scaling factor (PLBCA) to existing requireiments

- PLBCA = Ntotal / Navailable

- For a window W of duration max(TL1, MGRP_MAX, Tswitching_pattern), where TL1 is periodicity of the target x-RS, and Tswitching_pattern is the periodicity of semi-static switching pattern, and MGRP_MAX is maximum measurement gap repetition period across per-UE GAPs or per-FR GAPs

- Ntotal is the number of x-RS resource occasions within the window W, and

- Navailable is the number of x-RS resource occasions that are not overlapped with any non-dropped GAP occasion, , nor PCell duration of semi-static switching pattern within window W.

- Proposal 6: Due to inter-band CA considering a single RF chain, deactivated SDL SCell measurement should be performed within the measurement gap configured by NW.

- Proposal 7: RAN4 to specify SCell activation window which ensures SCell measurements of the deactivated SCell.

- Proposal 8: Existing SCell activation / deactivation delay requirements (based on SSB or SSB-less) can be rused for SDL SCell activation / deactivation delay of low band CA vis switching

- Proposal 9: RAN4 needs to clarify when to apply the semi-static switching pattern after compeleting SCell activation, e.g., apply immediately the semi-static switching pattern or apply next cycle semi-static switching pattern

- Proposal 10: RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell.

- Proposal 11: RAN4 to wait for RAN1 final agreements for semi-static switching pattern design, then discuss impact on PCell measurement due to overlapping between SCell reception and SMTC/MG/RS occasion of PCell.

Reference

[1] R4-2504895, “WF on RRM requirements for NR_LBCA_Sw” Apple