3GPP TSG-RAN WG4 Meeting #115	R4-2506238

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM core requirements for ATG CA operation

Agenda item:	7.12.5

Document for:	Discussion

Introduction

In this contribution, we provide our views on remaining issues for ATG with DL CA operation.

Discussion

The Report granularity of UE capability

In the last RAN4 meeting, UE capability whether to support common Rx beam for inter-band CA was agreed, but still FFS for report granularity of the capability. Depending on inter-band CA combination, different Rx beam implementation could be considered. Therefore, the report granularity of the UE capability should be per Band Combination.

- Proposal 1: The Report granularity of UE capability whether to support common Rx beam should be per BC.

Interruption requirements

In the last meeting, one issue was raised how to define interruption requirements caused by SCell addition/release and SCell activation/deactivation for common Rx beam UE in inter-band CA. The exsiting interruption requirements have been defined depending on intra-band CA and inter-band CA. For intra-band CA, additional SMTC duration was considered for AGC settling time. But, RAN4 RRM assumed inter-band CA RF architecture for ATG common Rx beam UE when defining MRTD requirement. Therefore, the existing interruption requriements for inter-band CA can be reused for common Rx beam ATG UE.

- Proposal 2: The existing interruption requirments caused by SCell addition / release and SCell activation / deactivation can be reused for ATG CA depending on intra-band CA and inter-band CA (i.e., no need to consider different requirement for common Rx beam ATG UE in inter-band CA).

Sharing factor between L1 measurement and RRM measurement

Agreement in RAN4#114: Agree on the following Proposal 1 for the intra-frequency measurement and L1-RSRP measurement, and apply the similar principle to other related requirements. - Proposal 1: - The Klayer1_measurement and Psharing factor in CA scenario should be updated as (NOTE: take the intra-frequency measurement and L1-RSRP measurement as an example): For UE with the antenna array, - Klayer1_measurement=1, - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4] or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols [configured] by same serving cell when inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement; Klayer1_measurement=1.5, otherwise.

For Case 4 antenna type UE, RAN4 agreed to define MRTD as 33usec. It means that the reception time difference between PCell and SCell could be 33usec for ATG DL CA using common Rx beam. When the UE sweeps Rx beam for neighbor cell measurement, 33usec difference considering MRTD would be impact on 1 data symbol of one of two CCs since the UE has common Rx beam for two CC. If the UE perform RRM measurement within SMTC, one symbol of one of the CC cannot be received due to Rx beam sweeping. Therefore, this should be considered sharing factor Klayer1_measurement, e.g.,

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

And also, scheduling restriction might be considered.

- Proposal 3: Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Conclusion

In this contribution, we provide views on remaining issues for ATG with CA operation, and we propose

- Proposal 1: The Report granularity of UE capability whether to support common Rx beam should be per BC.

- Proposal 2: The existing interruption requirments caused by SCell addition / release and SCell activation / deactivation can be reused for ATG CA depending on intra-band CA and inter-band CA (i.e., no need to consider different requirement for common Rx beam ATG UE in inter-band CA).

- Proposal 3: Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

- if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Reference

[1] R4-2504898, “WF on RRM requirements for NR_ATG_enh,” CMCC