3GPP TSG-RAN WG4 Meeting #115	R4-2506272

St Julian’s, Malta, 19th - 23th May, 2025

Source:	Samsung, KDDI, Qualcomm

Title:	TP to TR 38.719-03-01 to include CA_n28A-n41A-n74A

Agenda item:			6.3.4

Document for:	Approval

- Introduction

The revised basket WID was approved in RAN#107[1].

All the PC3 fallbacks are already in spec.

- References

[1] RP-250214, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, Mar.,2025, RAN#107

Text Proposal

<Start of Text Proposal>

5.x	CA_n28-n41-n74

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n41 | 2496 MHz – 2690MHz | 2496 MHz – 2690MHz | TDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n41A-n74A | CA_n28A-n41A CA_n41A-n74A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n74 | 5, 10, 15, 20 |  |

5.x.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are given in the tables below.

Table 5.0.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n41-n74 | 0.6 | 0.3 | 0.4 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.0.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n28-n41-n74 | 0.2 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.x.2	Specific for 2 bands UL CA

5.x.2.1	UE co-existence studies

5.x.2.1.1	Co-existence studies for 2UL band with 1CC per band

Based on co-existence calculation,

- No IMD due to UL CA_n28A-n41A falls into 74 DL

- IMD5 due to UL CA_n41A-n74A falls into n28 DL

5.x.2.2	REFSENS requirements

Based on clause 5.x.2.1.1, the MSD values are given as follows.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n74 | n28 | N/A | 5 | N/A | 782 | 3.5 | FDD | IMD5 |
|  | n41 | 2569 | 10 | 50 | 2569 | N/A | TDD | N/A |
|  | n74 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |

<End of Text Proposal>