3GPP TSG-RAN4 #115	R4-2506279

St. Julian’s, Malta, 19th May – 23rd May 2025

Title:	(LTE_NBIoT_eMTC_NTN_req-Perf) Clarification on test requirement applicability to IoT-NTN devices

Source:			Anritsu Corporation

Agenda Item:			5.25.4

Document for:			Others

# 1.	Introduction

At the RAN4 #114 meeting in Athens, we discussed on the applicability rule of IoT-NTN requirements in a case when NTN devices support only NTN bands [1]. And the CR [2] was agreed to modify a note to clarify the applicability rule.

In this contribution we discuss the identified issue with the updated note on the applicability rule of IoT-NTN requirements.

# 2.	Discussion

  - Identified issue with the IoT-NTN performance tests

We studied the updated note by the CR [2] internally after the meeting in February and we concluded that there are multiple interpretations with the following note in the Table 8.2.1.2-1 of TS 36.102.[3]

- Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Observation 1: There are multiple interpretations with the note in Table 8.2.1.2-1 of TS 36.102 per NTN scenario whether we need to conduct CSI reporting tests with NTN configurations.

Before discussing those interpretations, we’d like to list our findings with TS 36.101. [4]

- There is a definition of Category NB2 in sub-clause 9.14 for CSI reporting.

- There are also definitions of UE Category M1 and M2 in sub-clause 9.8 for CSI reporting.

Observation 2: There are definitions of Category NB2 in sub-clause 9.14, and Category M1 and M2 in sub-clause 9.8 of TS 36.101 which are related to CSI reporting tests.

Next, in our understanding, the current Note above requires the tests of clause 8 and clause 9 in TS 36.101 with NTN configurations regardless of the NTN scenario (i.e. GSO and/or NGSO).

Observation 3: Note in Table 8.2.1.2-1 of TS 36.102 requires the tests of clause 8 and clause 9 in TS 36.101 with NTN configurations regardless of the NTN scenario (i.e. GSO and/or NGSO).

Based on the understandings above, we tried summarizing the applicability of NTN configurations to TS 36.101 depending on scenarios of GSO/NGSO and supported bands.

Case 1: Summarize an applicability at NGSO only part based on Alternative 1 in R4-2500133 [1].

Also, we consider the following interpretation for both GSO and NGSO support case.

Interpretation: If UE supporting both GSO and NGSO, we can use “TS36.102 to verify UE with NSGO configurations” and “TS36.101 to verify UE with GSO configurations”.

For our reference, we’ll extract the Alternative 1 of R4-2500133 as follows.

(R4-2500133) Table 2. Specification applies to different ntn scenario support with Alternative 1 changes

| ntn-ScenarioSupport-r17 | gso | ngso | not included |
| --- | --- | --- | --- |
|  | TS36.101 under NTN configuration | TS36.102 and TS36.101 under NTN configuration | TS36.102 and TS36.101 under NTN configuration |

Here we summarize our understanding of the applicability rule for case 1 in Table 2.1-1.

Table 2.1-1: Summary of case 1 based on Alternative 1 in R4-2500133 and additional interpretation

|  | Supported band |  | Identified issue |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 (NTN config (GSO)) | (2) 36.101 (NTN config (GSO)) | No issues identified. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.101 (NTN config (NGSO)) 36.102 (NTN config (NGSO)) | (4) 36.101 (NTN config (NGSO)) 36.102 (NTN config (NGSO)) | The Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will be covered. Instead, the tests in clause 8 are duplicated in TS 36.101 and TS 36.102. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 (NTN config (GSO)) 36.102 (NTN config (NGSO)) | (6) 36.101 (NTN config (GSO)) 36.102 (NTN config (NGSO)) | The Cat-NB2, Cat-M1/M2 tests with GSO configurations in clause 9 of TS 36.101 will be covered. The Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered. |

Observation 4: In case 1 for NGSO only scenario, tests in clause 8 are duplicated in TS 36.101 and TS 36.102. For both GSO and NGSO scenario, the Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered.

Case 2: Apply part of the interpretation of Case 1 above (“TS36.102 to verify UE with NSGO configurations”) to NGSO only part.

The same interpretation as Case 1 above is applied also to both GSO and NGSO supported UE.

Summary of the applicability rule for case 2 is as follows.

Table 2.1-2: Summary of case 2 based on the rule

|  | Supported band |  | Identified issue |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1) 36.101 (TN config) 36.101 (NTN config (GSO)) | (2) 36.101 (NTN config (GSO)) | No issues identified. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.101 (NTN config (NGSO)) 36.102 (NTN config (NGSO)) | (4) 36.101 (NTN config (NGSO)) 36.102 (NTN config (NGSO)) | The Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 (NTN config (GSO)) 36.102 (NTN config (NGSO)) | (6) 36.101 (NTN config (GSO)) 36.102 (NTN config (NGSO)) | The Cat-NB2, Cat-M1/M2 tests with GSO configurations in clause 9 of TS 36.101 will be covered. The Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered. |

Observation 5: In case 2 for NGSO only scenario and both GSO and NGSO scenario, the Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered.

Therefore, based on the above considerations of case 1 and case 2 depending on the interpretation of the note in Table 8.2.1.2-1 of TS 36.102, we may have different coverage issues for NGSO only support UE case and both GSO and NGSO support UE case.

Observation 6: Depending on the interpretation of the note in Table 8.2.1.2-1 of TS 36.102, we may have different coverage issues for NGSO only support UE case and both GSO and NGSO support UE case.

Then one clarification question would be “Are the CSI reporting tests in clause 9 of TS36.101 necessary for NTN devices?”.

Observation 7: Clarification is necessary on the need of CSI reporting tests in clause 9 of TS 36.101 for NTN devices per NTN scenario basis.

If we have already agreed that CSI reporting tests are out of scope for NTN devices, then the description of clause 9 in the existing note is not necessary. If the CSI reporting tests are necessary for NTN devices, then further detailed clarification is necessary per NTN scenario i.e. GSO and/or NGSO.

Proposal 1: Maintain the applicability rule of NTN configuration depending on the need of the CSI reporting tests per NTN scenario (i.e. GSO and/or NGSO). Presumable applicability rules can be listed as follows.

CSI reporting for NTN case 1: If CSI reporting tests are out of scope for NTN devices regardless of GSO or NGSO scenario.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN GSO configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) | (2) 36.101 cl.8 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. No NTN CSI reporting tests will be conducted. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. No NTN CSI reporting tests will be conducted. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. No NTN CSI reporting tests will be conducted. |

CSI reporting for NTN case 2: If CSI reporting tests are necessary for both GSO and NGSO NTN devices.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN configurations (GSO and/or NGSO), e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability. Requirements in TS 36.101 clause 8 with NGSO configuration can be omitted if the requirements in TS 36.102 clause 8 with NGSO configuration are applicable.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 & 9 (NTN config (GSO)) | (2) 36.101 cl.8 &9 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with GSO configurations. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. NTN demodulation tests in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with NGSO configurations. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 &9 (NTN config (GSO)) 36.101 cl.9 (NGN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 & 9 (NTN config (GSO)) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with both GSO and NGSO configurations. |

CSI reporting for NTN case 3: If CSI reporting tests are necessary for only GSO NTN devices.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN GSO configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 &9 (NTN config (GSO)) | (2) 36.101 cl.8 &9 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with GSO configurations. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. No NTN CSI reporting tests will be conducted. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 &9 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 &9 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with GSO configurations. |

CSI reporting for NTN case 4: If CSI reporting tests are necessary for only NGSO NTN devices.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTNGSO configurations and Clause 9 also apply with NGSO configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) | (2) 36.101 cl.8 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. No NTN CSI reporting tests will be conducted. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with NGSO configurations. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 (NTN config (GSO)) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with NGSO configurations. |

- Conclusion

In this contribution we discussed the identified issue with the updated note on the applicability rule of IoT-NTN requirements.

Observation 1: There are multiple interpretations with the note in Table 8.2.1.2-1 of TS 36.102 per NTN scenario whether we need to conduct CSI reporting tests with NTN configurations.

Observation 2: There are definitions of Category NB2 in sub-clause 9.14, and Category M1 and M2 in sub-clause 9.8 of TS 36.101 which are related to CSI reporting tests.

Observation 3: Note in Table 8.2.1.2-1 of TS 36.102 requires the tests of clause 8 and clause 9 in TS 36.101 with NTN configurations regardless of the NTN scenario (i.e. GSO and/or NGSO).

Observation 4: In case 1 for NGSO only scenario, tests in clause 8 are duplicated in TS 36.101 and TS 36.102. For both GSO and NGSO scenario, the Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered.

Observation 5: In case 2 for NGSO only scenario and both GSO and NGSO scenario, the Cat-NB2, Cat-M1/M2 tests with NGSO configurations in clause 9 of TS 36.101 will not be covered.

Observation 6: Depending on the interpretation of the note in Table 8.2.1.2-1 of TS 36.102, we may have different coverage issues for NGSO only support UE case and both GSO and NGSO support UE case.

Observation 7: Clarification is necessary on the need of CSI reporting tests in clause 9 of TS 36.101 for NTN devices per NTN scenario basis.

Proposal 1: Maintain the applicability rule of NTN configuration depending on the need of the CSI reporting tests per NTN scenario (i.e. GSO and/or NGSO). Presumable applicability rules can be listed as follows.

CSI reporting for NTN case 1: If CSI reporting tests are out of scope for NTN devices regardless of GSO or NGSO scenario.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN GSO configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) | (2) 36.101 cl.8 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. No NTN CSI reporting tests will be conducted. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. No NTN CSI reporting tests will be conducted. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. No NTN CSI reporting tests will be conducted. |

CSI reporting for NTN case 2: If CSI reporting tests are necessary for both GSO and NGSO NTN devices.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN configurations (GSO and/or NGSO), e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability. Requirements in TS 36.101 clause 8 with NGSO configuration can be omitted if the requirements in TS 36.102 clause 8 with NGSO configuration are applicable.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 & 9 (NTN config (GSO)) | (2) 36.101 cl.8 &9 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with GSO configurations. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. NTN demodulation tests in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with NGSO configurations. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 &9 (NTN config (GSO)) 36.101 cl.9 (NGN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 & 9 (NTN config (GSO)) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with both GSO and NGSO configurations. |

CSI reporting for NTN case 3: If CSI reporting tests are necessary for only GSO NTN devices.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN GSO configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 &9 (NTN config (GSO)) | (2) 36.101 cl.8 &9 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with GSO configurations. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. No NTN CSI reporting tests will be conducted. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 &9 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 &9 (NTN config (GSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with GSO configurations. |

CSI reporting for NTN case 4: If CSI reporting tests are necessary for only NGSO NTN devices.

Proposal of correction to the existing note in Table 8.2.1.2-1 of TS 36.102 is as follows.

Note:    For UE supporting NTN access (ntn-Connectivity-EPC-r17), the requirements in TS36.101 Clause 8 and Clause 9 also apply with NTN GSO configurations and Clause 9 also apply with NGSO configurations, e.g., including Ephemeris, K_offset and NTN bands, according to the UE category and capability.

Summary of the applicability rules would be as follows.

|  | Supported band |  | Applicability rule to demodulation and CSI tests |
| --- | --- | --- | --- |
| ntn-ScenarioSupport-r17 | Both TN and NTN band | NTN only |  |
| gso (GSO only) | (1)36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) | (2) 36.101 cl.8 (NTN config (GSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations. No NTN CSI reporting tests will be conducted. |
| ngso (NGSO only) | (3) 36.101 (TN config) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (4) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with NGSO configurations. |
| not included (Both GSO and NGSO) | (5) 36.101 (TN config) 36.101 cl.8 (NTN config (GSO)) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | (6) 36.101 cl.8 (NTN config (GSO)) 36.101 cl.9 (NTN config (NGSO)) 36.102 cl.8 (NTN config (NGSO)) | NTN demodulation tests will be conducted by clause 8 of 36.101 with GSO configurations and clause 8 of 36.102 with NGSO configurations. NTN demodulation tests with NGSO in clause 8 of 36.101 can be skipped to avoid duplicating tests with 36.102. NTN CSI reporting tests will be conducted by clause 9 of 36.101 with NGSO configurations. |

An associated CR [5] is also submitted to this meeting to solve this interpretation issue.

# 4.	References

[1] R4-2500133, “Discussion on applicability rules for IoT-NTN requirements”, MediaTek Inc., RAN4 #114, Athens, Greece

[2] R4-2502355, “CR for TS36.102, applicability rules for IoT-NTN requirements”, MediaTek Inc., RAN4 #114, Athens, Greece

[3] TS 36.102, "User Equipment (UE) radio transmission and reception for satellite access", v18.8.0, 2025-03

[4] TS 36.101, "User Equipment (UE) radio transmission and reception", v19.1.0, 2025-03

[5] R4-2506280, “CR on note of applicability rules for IoT-NTN requirements”, Anritsu Corp., RAN4 #115, St. Julian’s, Malta