3GPP TSG-RAN WG4 Meeting # 115	R4-2506296

Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SIB1 for Rel-19 NES

Agenda item:	7.27.3.3

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized … - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SIB1 for Rel-19 NES.

Discussion

RRM requirements impacts for OD-SIB1 – paging interruption

An issue regarding RRM impact for paging interruption was raised due to on-demand SIB1 (OD-SIB1). The motivation is to discuss whether additional paging interruption is necessary, because more delay happens to be occured when UL WUS triggers OD-SIB1. RAN4 discussed the issue in RAN4#114bis meeting, and agreed to have following Wayforward[2] as follows:

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Way forward: - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo) - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE) - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

Meanwhile, RAN2 has drawn following agreement regarding paging interruption

Agreements on OD-SIB1 (RAN2#127bis)

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

According to the agreements from RAN2#127bis, RAN4 needs to decide whether to introduce additional paging interruption time and UE behaviour.

In Wayforward[2], RAN4 agreed to have three options. Option A is to increase paging interruption time and clarify OD-SIB1 acquisition procedure, which means that overlapping between paging occasion and OD-SIB1 qcquisition is allowed. Option B is to capture the behaviour in RAN2, and reflect the result in the performance requirement, however, this option may not be valid due to RAN2 agreement. Option C is that the UE is not expected to proceed OD-SIB1 acquisition when it is overlapped with paging occasion, which means that UL-WUS can be postponed.

Option C with UE behaviour clarificiation would be our preference. Since the paging reception from serving cell is much more critical than acquiring SIB1 of the neighbor cell, the receiving RAR and monitoring OD-SIB1 window on neighbor cell should not interfere with the paging reception. If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, UE should monitor the serving cell during the overlapping period. In conclusion, we believe paging interruption time requirement can be reused as legacy, furthermore the corresponding UE behaviour needs to be captured in the spec to avoid misunderstanding.

Figure 1 Paging reception interruption

Proposal 1: If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period

- Reuse legacy paging interruption time requirement

- Capture corresponding UE behaviour

Conclusion

In this contribution, we provide views on RRM requireiments for OD-SIB1, and we propose

Proposal 1: If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period

- Reuse legacy paging interruption time requirement

- Capture corresponding UE behaviour

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei