3GPP TSG-RAN WG4 Meeting # 115	R4-2506312

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title:	Discussion for UE RF Rx requirements of NTN_NR_HPUE

Agenda item:	7.10.3.2.2

Document for:	Discussion

Introduction

In the RAN4#114bis meeting,  the WF on HPUE requirements for NR-NTN was approved. In this discussion, we share our point of view for the HPUE requirements for NR-NTN based on the approved WF[1].

Discussion

Table 1 Way forward for UE RF requirements for NTN_NR_HPUE [1]

# Open issues Issue 1-10: General discussion of RSD analysis Ad-hoc agreements: - In addition to Tx leakage to Rx, Rx non-linearity needs to be considered in RSD evaluation. Issue 1-11: General discussion of RSD of PC1.5 and PC1 Ad-hoc agreements: - Companies agree to bring more analysis for the RSD values for PC1.5/PC1 non-handheld UE. - Further check these in next meeting. Issue 1-13: RSD requirements for PC2 on n255 Ad-hoc agreements: - For n255 PC2: 5MHz; 10MHz; 15 MHz; 20 MHz. RSD@1Tx; [0.8]; [0.8]; [0.9]; [0.9]. RSD@2Tx; [1.6]; [1.6]; [1.7]; [1.7]. Issue 1-14: RSD requirements for PC1.5 and PC1. Way forward: - Encourage companies to further discuss the following proposals: - Option 1: For PC1.5 2Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; 2.2; 2.2; 2.2; 2.2. n255; 3.2; 3.2; 3.3; 3.3. n254; 2.2; 2.2; 2.2. - Option 2: For PC1 1Tx (MTK) Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. n254; [3]; [3]; [3].

Reference sensitivity degradation for NTN HPUE

Based on the submission made in the last meeting, an RSD analysis was conducted on the PC1.5 and PC1 non-handheld devices.

- For PC1.5 2Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.2 | 0.2 | 0.2 | 0.2 |
| n254 | 0 | 0 | 0 | 0 |

- For PC1.5 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.1 | 0.1 | 0.1 | 0.1 |
| n254 | 0 | 0 | 0 | 0 |

- For PC1 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.2 | 0.2 | 0.2 | 0.2 |
| n254 | 0 | 0 | 0 | 0 |

The duplex isolation factor of non-handheld devices can be approximately 10 to 15 dB greater than that of handheld devices. Additionally, the n256 and n254 bands have a larger Tx-Rx frequency separation compared to n255, allowing us to assume that the Tx leakage noise is smaller than that of n255. Considering this, all RSD values were determined to be less than 0.2 dB.

Observation 1: Considering the non-handheld RF characteristic, RSD values of NH PC1.5 and PC1 were determined to be less than 0.2 dB.

Observation 2: following options can be considered for PC1.5 RSD

- Option 1: Do not specify the RSD for the PC1.5 non-handheld device

- Option 2: Specify the small RSD values for PC1.5 non-handheld device in table below

  - For PC1.5 2Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.2 | 0.2 | 0.2 | 0.2 |
| n254 | 0 | 0 | 0 | 0 |

  - For PC1.5 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.1 | 0.1 | 0.1 | 0.1 |
| n254 | 0 | 0 | 0 | 0 |

Observation 3: following options can be considered for PC1 RSD

- Option 1: Do not specify the RSD for the PC1 non-handheld device

- Option 2: Specify the small RSD values for PC1 non-handheld device in table below

  - For PC1 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.2 | 0.2 | 0.2 | 0.2 |
| n254 | 0 | 0 | 0 | 0 |

Proposal: Consider option 1 both observation 2 and observation 3.

- Option 1: Do not specify the RSD for PC1 and PC1.5 non-handheld devices

Conclusion

Observation 1: Considering the non-handheld RF characteristic, RSD values of NH PC1.5 and PC1 were determined to be less than 0.2 dB.

Observation 2: following options can be considered for PC1.5 RSD

- Option 1: Do not specify the RSD for the PC1.5 non-handheld device

- Option 2: Specify the small RSD values for PC1.5 non-handheld device in table below

  - For PC1.5 2Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.2 | 0.2 | 0.2 | 0.2 |
| n254 | 0 | 0 | 0 | 0 |

  - For PC1.5 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.1 | 0.1 | 0.1 | 0.1 |
| n254 | 0 | 0 | 0 | 0 |

Observation 3: following options can be considered for PC1 RSD

- Option 1: Do not specify the RSD for the PC1 non-handheld device

- Option 2: Specify the small RSD values for PC1 non-handheld device in table below

  - For PC1 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0.1 | 0.1 |
| n255 | 0.2 | 0.2 | 0.2 | 0.2 |
| n254 | 0 | 0 | 0 | 0 |

Proposal: Consider option 1 both observation 2 and observation 3.

- Option 1: Do not specify the RSD for PC1 and PC1.5 non-handheld devices

- References

# [1] R4-2505113, WF on HPUE requirments for NR-NTN (Samsung)

# [2] R4-2503722, Discussion for UE RF Rx requirements of NTN_NR_HPUE(LGE)