# 3GPP TSG-RAN WG4 Meeting #115	                                                           R4-2506325

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.113: Introduction of 7 MHz NR FR1 channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of 7 MHz NR FR1 channel bandwidth requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Required changes to support 7 MHz NR FR1 channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7 MHz NR FR1 channel bandwidth requirements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

6.1	Performance criteria for continuous phenomena for BS

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2. If the test is not performed using one of these bearers (for example, of none of them are supported by the BS), the characteristics of the bearer used shall be recorded in the test report.

The throughput in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2 is stated relative to the maximum throughput of the FRC.

The BS uplink and downlink paths shall each meet the performance criteria defined in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2 during the test. If the uplink and downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in table 6.1-1, table 6.1-1a, table 6.1-1b and table 6.1-2 (i.e. throughput > 90 % instead of throughput > 95 %). After each test case BS shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1-1: FR1 performance criteria for continuous phenomena for BS

| NR channel bandwidth (MHz) as defined in TS 38.104 section 5.3.2-1 [2] | Sub-carrier spacing (kHz) | Bearer information data rate (Note 3) (as in annex A.1 in TS 38.104 [2]) | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 | Throughput > 95 %, no loss of service |
| 5, 7, 10, 15 | 15 | G-FR1-A1-21 for NR with NB-IoT in-band operation |  |
| 10, 15 | 30 | G-FR1-A1-1 |  |
| 10, 15 | 60 | G-FR1-A1-10 for NR with NB-IoT in-band operation |  |
| 20 to 50 | 15 | G-FR1-A1-2 |  |
| 20 to 100 | 30 | G-FR1-A1-3 |  |
| 20 to 100 | 60 | G-FR1-A1-4 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. NOTE 3: These reference measurement channels are not applied for bands n46, n96 and n102. |  |  |  |

Table 6.1-1a: Performance criteria for continuous phenomena for band n46

| NR channel bandwidth (MHz) as defined in TS 38.104 section 5.3.2-1 [2] | Sub-carrier spacing (kHz) | Bearer information data rate (as in annex A.1 in TS 38.104 [2]) | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 10 | 15 | G-FR1-A1-12 | Throughput > 95 % no loss of service |
|  | 30 | G-FR1-A1-13 |  |
|  | 60 | G-FR1-A1-3 |  |
| 20 | 15 | G-FR1-A1-14 |  |
|  | 30 | G-FR1-A1-15 |  |
|  | 60 | G-FR1-A1-6 |  |
| 40 | 15 | G-FR1-A1-16 |  |
|  | 30 | G-FR1-A1-17 |  |
|  | 60 | G-FR1-A1-6 |  |
| 60 | 30 | G-FR1-A1-18 |  |
|  | 60 | G-FR1-A1-6 |  |
| 80 | 30 | G-FR1-A1-19 |  |
|  | 60 | G-FR1-A1-6 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-1b: Performance criteria for continuous phenomena for bands n96 and n102

| NR channel bandwidth (MHz) as defined in TS 38.104 section 5.3.2-1 [2] | Sub-carrier spacing (kHz) | Bearer information data rate (as in annex A.1 in TS 38.104 [2]) | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 20 | 15 | G-FR1-A1-14 | Throughput > 95 % no loss of service |
|  | 30 | G-FR1-A1-15 |  |
|  | 60 | G-FR1-A1-6 |  |
| 40 | 15 | G-FR1-A1-16 |  |
|  | 30 | G-FR1-A1-17 |  |
|  | 60 | G-FR1-A1-6 |  |
| 60 | 30 | G-FR1-A1-18 |  |
|  | 60 | G-FR1-A1-6 |  |
| 80 | 30 | G-FR1-A1-19 |  |
|  | 60 | G-FR1-A1-6 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-2: FR2 performance criteria for continuous phenomena for BS

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate (as in annex A.1 in TS 38.104 [2]) | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-A1-2 |  |
| 100, 200, 400 | 120 | G-FR2-A1-3 |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

# << End of change >>