# 3GPP TSG-RAN WG4 Meeting #115	R4-2506334

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.5

Source:	China Telecom

Title:	Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

Issue 2-1-1: TC list for Rx BSF optimization Agreement in online session: Further discuss based on the following table: Index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CMCC. 3; Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); CATT. 2; Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra; 1 or 2 TCs; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Huawei. 4; Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter; TBA(e.g., DRX or non-DRX, testing quitting FBS or not); Samsung. 5; Scenario 5: Handover; 1 or 2 TCs; TBA(e.g., Intra-frequency or inter-frequency, unknown); OPPO. 6; Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection; 1 or 2 TCs; TBA(e.g., re-establishment or RRC redirection, Intra-frequency or inter-frequency, [or known] or unknown); Intel. Issue 2-1-2: detailed design for test cases - Proposal 1(CATT): Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing Rx BSF. - Table 2-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing Rx BSF in R4-2503418. - Proposal 2(OPPO): - For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be used as baseline. - For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline. And the transmission power of serving cell from the start of T2 is set to meet the trigger condition of FBS based measurement. - Proposal 3(Nokia): - Use legacy test cases as the baseline for L3 FBS test cases, where the delay to be tested is defined with the reduced beam sweeping factor. - Recommended WF - Due to the TU budget, moderator recommend to discuss this issue in next meeting.

For L3 measurement delay reduction by optimizing Rx BSF, in our views, 1 test case for each scenario is preferred to control the workload. In order to ensure that every conditions discussed in core part can be tested, for Scenario 1/2/3/4, 4 test cases are designed for four scenarios including 2 test cases with DRX mode and another 2 test cases with non-DRX mode. For Scenario 1/2/3/4, the solution on quitting FBS need also to be considered. Besides, existing L3 intra-frequency/inter-frequency measurement test cases can be used as the baseline for L3 measurement delay reduction by optimizing Rx BSF, e.g., A.7.6.1 and A.7.6.2.

For Scenario 5/7, 2 test cases are designed for two scenarios including 1 test case for intra-frequency and another test case for inter-frequency, and in order to ensure the worst case can be tested, unknown case need to be considered. Besides, existing Handover and RRC Connection Mobility Control test cases can be used as the baseline for L3 measurement delay reduction by optimizing Rx BSF, e.g., A.7.3.1 and A.7.3.2.

Proposal 1: For L3 measurement delay reduction by optimizing Rx BSF, 1 test case for each scenario is preferred.

Proposal 2: For Scenario 1/2/3/4, 4 test cases are designed for four scenarios including 2 test cases with DRX mode and another 2 test cases with non-DRX mode.

Proposal 3: For Scenario 1/2/3/4, the solution on quitting FBS need also to be considered.

Proposal 4: For Scenario 1/2/3/4, existing L3 intra-frequency/inter-frequency measurement test cases, e.g., A.7.6.1 and A.7.6.2, can be used as the baseline for L3 measurement delay reduction by optimizing Rx BSF.

Proposal 5: For Scenario 5/7, 2 test cases are designed for two scenarios including 1 test case for intra-frequency and another test case for inter-frequency, and unknown case need to be considered.

Proposal 6: For Scenario 5/7, existing Handover and RRC Connection Mobility Control test cases, e.g., A.7.3.1 and A.7.3.2, can be used as the baseline for L3 measurement delay reduction by optimizing Rx BSF.

Status in the WF [1] and summary [2]:

Issue 2-2-1: TC list for CSSF optimization - TC for solution 1: Agreement in online session: - Define the test cases for solution 1 of CSSF enhancement following the table below: TC index; Scenario; Need or not; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., FR2 CA or EN-DC, NR-DC [or NE-DC]), UE is only required to pass one of them; TBA; ZTE. - TC for solution 3: [Way forward]: - FFS: - Define the test cases for solution 3 of CSSF enhancement following the table below: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode (i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}) UE is only required to pass one of them; TBA; TBA. Issue 2-2-2: detailed design for test cases - Proposal 1(CATT): Use the following existing test cases as starting point to define test cases for FR2-1 L3 measurement delay reduction by optimizing CSSF. - Table 3-Test cases needed for FR2-1 L3 measurement delay reduction by optimizing CSSF in R4-2503418 - Use the existing test cases for SS-RSRP, SS-RSRQ and SS-SINR measurements as starting point. FFS details. - Proposal 2(Huawei): The following points should be considered in the performance requirements for Solution 3 of the CSSF reduction: - Test parameters of Cell amount, for instance at least 5. (e.g. an FR1 PCell (Cell 1); an FR1 or FR2 deactivated SCell (Cell 2) and an FR1 or FR2 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2); FR1 or FR2 deactivated SCell (Cell 4) and an FR1 or FR2 neighbour cell (Cell 5) on the same frequency as the SCell (Cell 4).) - Test requirements to define a new measurement reporting delay accuracy. - Recommended WF - Due to the TU budget, moderator recommend to discuss this issue in next meeting.

For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, it’s proposed to design test cases for three aspects, and for each aspect, 1 test case is designed per operation mode, i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}, and UE is only required to pass one of operation modes. Besides, existing L3 intra-frequency/inter-frequency measurement test cases for SA and EN-DC modes can be used as the baseline for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, e.g., A.5.6.1 and A.5.6.2, A.7.6.1 and A.7.6.2.

Proposal 7: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, it’s proposed to design test cases for three aspects, and for each aspect, 1 test case is designed per operation mode, i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}, and UE is only required to pass one of operation modes.

Proposal 8: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, existing L3 intra-frequency/inter-frequency measurement test cases for SA and EN-DC modes, e.g., A.5.6.1 and A.5.6.2, A.7.6.1 and A.7.6.2, can be used as the baseline for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC.

- Conclusion

The contribution presents our views on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode, with the following proposals:

Proposal 1: For L3 measurement delay reduction by optimizing Rx BSF, 1 test case for each scenario is preferred.

Proposal 2: For Scenario 1/2/3/4, 4 test cases are designed for four scenarios including 2 test cases with DRX mode and another 2 test cases with non-DRX mode.

Proposal 3: For Scenario 1/2/3/4, the solution on quitting FBS need also to be considered.

Proposal 4: For Scenario 1/2/3/4, existing L3 intra-frequency/inter-frequency measurement test cases, e.g., A.7.6.1 and A.7.6.2, can be used as the baseline for L3 measurement delay reduction by optimizing Rx BSF.

Proposal 5: For Scenario 5/7, 2 test cases are designed for two scenarios including 1 test case for intra-frequency and another test case for inter-frequency, and unknown case need to be considered.

Proposal 6: For Scenario 5/7, existing Handover and RRC Connection Mobility Control test cases, e.g., A.7.3.1 and A.7.3.2, can be used as the baseline for L3 measurement delay reduction by optimizing Rx BSF.

Proposal 7: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, it’s proposed to design test cases for three aspects, and for each aspect, 1 test case is designed per operation mode, i.e., {FR1 only CA and FR1 only NR-DC}, [{FR1 only EN-DC}], {FR1+FR2 CA}, {FR1+FR2 NR-DC}, and UE is only required to pass one of operation modes.

Proposal 8: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, for solution 3, existing L3 intra-frequency/inter-frequency measurement test cases for SA and EN-DC modes, e.g., A.5.6.1 and A.5.6.2, A.7.6.1 and A.7.6.2, can be used as the baseline for FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC.

- References

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503618, Topic summary for [114bis][208] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.