# 3GPP TSG-RAN WG4 Meeting #115	R4-2506335

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.18.6

Source:	China Telecom

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, RRM performance requirements of fast SCell activation for UE supporting Rel-18 EMR was discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

### Issue 2-2-1: Scenarios of test cases Further discuss this issue. - Proposals - Option 1: (OPPO) - RAN4 to define the following test cases for fast SCell activation: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 and FR2 - PUCCH SCell activation for FR1 and FR2 - Option 2: (CATT) - RAN4 consider the following aspects when defining test cases: - Target SCell can be in FR1 or FR2 band. - Applicable scenarios: EN-DC, SA CA, NE-DC, NR-DC, excluding the following scenarios: - SCell being contiguous to PCell in FR1 (i.e. intra-band contiguous CA) - SCell being in the same band as PCell in FR2 (i.e. intra-band CA) - Difference UE capabilities and configurations. - Type of CSI-RS resources for CQI reporting. - RAN4 to choose a subset of all combinations to define test cases. The list can be further discussed. - Option 3: (CMCC) - It is proposed to define test cases for fast SCell activation for UE supporting Rel-18 EMR. - For fast SCell activation, it is proposed to define test cases for following scenarios - Direct SCell Activation at SCell addition - SCell Activation Delay Requirement for Deactivated PUCCH SCell - For fast SCell activation, it is proposed to define test cases for following cases - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode - Option 4: (ZTE) - Define test cases to verify the SCell Activation Delay Requirement for unknown SCell based on measurement in IDLE/INACTIVE mode. - R16 EMR and R18 eEMR including valid EMR and valid cell reselection measurement should be considered when designing the test case. - Option 5: (HW) - For fast SCell activation for UE supporting Rel-18 EMR, RAN4 to define test cases for all three scenarios for both FR1 and FR2. And RAN4 could further discuss the applicability rule whether UE is required to pass all of them. - SCell Activation based on measurement in IDLE/INACTIVE mode - Direct SCell Activation at SCell addition - SCell Activation Delay Requirement for Deactivated PUCCH SCell - Option 6: (CT) - For Fast SCell activation for UE supporting Rel-18 EMR, it’s proposed to define test cases for following scenarios: - Single Direct SCell Activation at SCell addition in FR1; - Single Direct SCell Activation at SCell addition in FR2; - MAC CE based Single SCell Activation in FR1; - MAC CE based Single SCell Activation in FR2. - Option 7: (Nokia) - Test the scenario where measIdleValidityDuration-r18 is configured with successful SCell activation with EMR. - Test scenario where measReselectionValidityDuration-r18 is configured. - Test the scenario when measIdleValidityDuration-r18 is configured while T331 is running with SCell activation. - Discuss whether all test cases are done for both FR1 and FR2. - Discuss which scenarios to apply on each test case: - Direct SCell Activation at SCell addition - MAC CE based Single SCell Activation - MAC CE based PUCCH SCell activation - Recommended WF - RAN4 to define test cases for eEMR-based fast SCell activation based on the following aspects: - Scenarios: - Normal SCell activation - Direct SCell activation at SCell addition - PUCCH SCell activation - Frequency range: - FR1 and FR2 - UE capabilities and/or network configurations: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode - Discuss whether to choose a subset of combinations of the above aspects to define test cases. - Discuss the applicability rule whether UE is required to pass all of test cases. ### Issue 2-2-2: Details of test case design Further discuss this issue. - Proposals - Option 1: (OPPO) - The configuration of test cases should meet the conditions for fast SCell activation. - The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively. - T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation. - For direct SCell activation, T3 and T4 can be merged into one. - The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline. - The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline. - Option 2: (CATT) - Reuse the contents in the legacy test cases for Rel-18 eEMR measurement with valid result reporting and SCell activation. - Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions: - For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell, 5*DRX cycles); - For target SCell in FR2, time gap needs to be larger than 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4. - Option 3: (Ericsson) - The performance of Rel-19 fast SCell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing SCell activation test case with the Idle state fast CA/DC reporting test case. - Recommended WF - Basic principle is to reuse the legacy test cases as much as possible. Discuss the details based on the options.

In our understanding, based on the core part discussion in RAN4 previous meetings, the scenarios of Fast SCell activation for UE supporting Rel-18 EMR include Single Direct SCell Activation at SCell addition, Normal MAC CE based Single SCell Activation and PUCCH SCell activation, besides, FR1 and FR2 cases are considered in the two scenarios. Therefore, for Fast SCell activation for UE supporting Rel-18 EMR, it’s proposed to define test cases for following scenarios:

- Single Direct SCell Activation at SCell addition in FR1;

- Single Direct SCell Activation at SCell addition in FR2;

- Normal MAC CE based Single SCell Activation in FR1;

- Normal MAC CE based Single SCell Activation in FR2;

- PUCCH SCell activation in FR1;

- PUCCH SCell activation in FR2.

Proposal 1: For Fast SCell activation for UE supporting Rel-18 EMR, it’s proposed to define test cases for following scenarios:

- Single Direct SCell Activation at SCell addition in FR1;

- Single Direct SCell Activation at SCell addition in FR2;

- Normal MAC CE based Single SCell Activation in FR1;

- Normal MAC CE based Single SCell Activation in FR2;

- PUCCH SCell activation in FR1;

- PUCCH SCell activation in FR2.

- Conclusion

The contribution presents our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: For Fast SCell activation for UE supporting Rel-18 EMR, it’s proposed to define test cases for following scenarios:

- Single Direct SCell Activation at SCell addition in FR1;

- Single Direct SCell Activation at SCell addition in FR2;

- Normal MAC CE based Single SCell Activation in FR1;

- Normal MAC CE based Single SCell Activation in FR2;

- PUCCH SCell activation in FR1;

- PUCCH SCell activation in FR2.

- References

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.

- R4-2503619, Topic summary for [114bis][209] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #114bis, 7 – 11 April, 2025.