3GPP TSG-RAN WG4 Meeting #115	R4-2506347

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on RRM core requirements and test framework for beam management

Agenda Item:	7.21.4

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #114bis, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Core requirements

Issue 2-1: Metrics/KPIs for beam prediction

Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Meanwhile, recall the WF in RAN4 meeting #114 as the below.

Beam prediction KPIs Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Proposals - Option 1: - In case of the AI/ML model with label (beam ID) output type - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted. - In case of the AI/ML model with label (beam ID) + RSRP output type - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously. - Option 2: Option 2 with a combination of Top-K/1 and Top 1/K - Option 2a: update option 2 to Top-N/K, the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". - Option 3: Option 3 as it does not have the issue of UE failing if there are multiple beams within a small RSRP margin - Option 4: combinations of options, concrete proposals should be disucssed - Recommended WF - To be discussed The number of options should be downselected to make some progress or a clear path to choose an option should be discussed/agreed

The basic agreement on ‘The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB’, in our opinion, appears to assess the beam prediction accuracy from the perspective of the ground-truth RSRP comparison between the predicted beam and the genie-aided beam.

While this approach offers an agreed metric, it raises two concerns:

- Determination of parameter K

- Determination of parameter x

Regarding the first question, despite practical implementation, the choice of K is crucial for crucial evaluating prediction correctness. It seems appear that increasing K results in a more relaxed evaluation criterion.

It can be found that the top-K beams are sorted out and ordered by L1-RSRPs (referring the labelling), even L1-RSRP isn’t directly reported by the model, as an interpretation of the below statements duplicated from TR38.843.

Based on model output (e.g., probability of each beam in Set A to be the Top-1 beam, predicted L1-RSRPs), Top-1/N beam(s) among Set A of beams can be predicted and/or potentially with predicted L1-RSRPs (depending on the labelling). … For both BM-Case1 and BM-Case2, UE can report the prediction result to NW based on the output of a UE-side model, or NW can predict the Top-1/N beam(s) based on the reported measurements of Set B for a NW-side model. Figure 6.3.1-1: An example of the inference procedure for beam management.

At this moment, the issue about the order of the top-K beams is under discussion in RAN1. One of proposals is: The predicted Top K beam(s) are the best K beam(s) based on the predicted RSRP values, or the probability of each beam in Set A to be the Top-1 beam.

Observation 1: If only beam ID is reported, the order of top-K beams is able to indicate the order of RSRP implicitly.

In one example, K=2, including 2 reported beams in the power/RSRP order of the 1st beam and the 2nd beam,  and the ground-truth RSRP of the 2nd beam is the higher one.  If we follow the above agreements of test metrics, it can be observed that the 1st beam in practice has not any restriction, if the AI/ML model has a defect causing the 1st beam actually being allowed to have very low (lower than the 2nd beam) ground-truth RSRP. The consequence is the NW may choose the 1st beam and further network performance may be degraded due to suboptimal or bad beam selection. So, K=1 shall be evaluated at the least.

Given the above the analysis in a straightforward and intuitive way, it is recommended that performance metrics support both strict and relaxed evaluation scenarios for different evaluate criteria by simultaneously considering:

- K = 1

- K = [2]

Proposal 1:  Beam ID prediction metrics and KPIs should include evaluations for both K = 1 and K = [2] to ensure a comprehensive assessment of prediction performance.

The prerequisite of measurement accuracy requirements is fulfillment of the SNR side condition in convention. It is generally understood that all RSs higher than the SNR side condition are expected to satisfy identical measurement accuracy criteria, regardless of the RS resource set configuration.

In previous meetings, we spent some time with two categories of beams: the ‘top-K beams’ and the ‘Set A beams’, While prior discussions have largely focused on the top-K beams, a question remains unresolved: should the evaluation metrics be confined only to the top-K beams, or should all beams within Set A be taken into account?

Proposal 2: RAN4 to determine the test scope for the case of beam ID being reported, we prefer option 2 between the two below options.

- Option 1: Only the top-K beams shall be verified.

- Option 2: All Set A beams meeting the SNR side condition for BM prediction shall be verified.

A relevant question is the concern to SNR side condition. The top-K beams (e.g., K= 1 and K= [2]) is selected from all Set A beams. It may be interpreted that: either the SNR for the top-K beams shall meet the side condition for BM prediction, or the SNR for all Set A beams shall meet the side condition for BM prediction.

The distinction has implications for both model evaluation and the definition of test metrics.

Proposal 3: RAN4 to investigate the SNR side condition of the test metrics for the cases of beam ID reporting. Two potential options are under consideration in the context of beam ID reporting:

- Option 1: Top-K beams in the test metrics shall meet the side condition for BM prediction.

- Option 2: All Set A beams shall meet the side condition for BM prediction, in which, top-K has higher SNR side condition which shall be defined.

Regarding the second question, ‘x’ dB represents a margin factor in essence. It may be affected by measurement error and other errors in prediction (training and inference phase). Consequently, it’s expected that ‘x’ shall be selected with a number same or close to today’s measurement error. In TR38.843, 1dB is brought as a reference for simulations, but it is doubted.

Proposal 4: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors.

The intention of N in the issue is to exploit the prediction accuracies of more than one predicted beams in respective, not only ‘the beam with maximum ground-truth RSRP among top-K predicted beams’, which actually doesn’t care about the order of the beams in the top-K beams .To our understanding, the evaluation can be represented in the below way:

- The ground-truth RSRP among 1st  predicted beams larger than or equal to the ground-truth RSRP of the 1st strongest  genie-aided beam(s) – x1 dB,

- The ground-truth RSRP among 2nd  predicted beams larger than or equal to the ground-truth RSRP of the 2nd strongest  genie-aided beam(s) – x2 dB,

- …

- The ground-truth RSRP among Nth predicted beams larger than or equal to the ground-truth RSRP of the Nth  strongest genie-aided beam(s) – xV dB

It appears that the proposed solution may impose more stringent requirements than those previously agreed upon, as it involves RSRP verification of each predicted beam in top-N.

Observation 2: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, is advantageous for enhancing reliability and robustness in beam management.

Proposal 5: We’re open to introducing N (N is [2]) to enable verifying the RSRP of each beam within the top-N predicted beams. If N is introduced, the metrics may be represented to:

- The ground-truth RSRP of the 1st predicted beam larger than or equal to the ground-truth RSRP of the 1st strongest  genie-aided beam(s) – x1 dB,

- The ground-truth RSRP of the 2nd predicted beam larger than or equal to the ground-truth RSRP of the 2nd strongest  genie-aided beam(s) – x2 dB,

- …

- The ground-truth RSRP of the Nth predicted beam larger than or equal to the ground-truth RSRP of the Nth  strongest genie-aided beam(s) – xN dB

Furthermore, for the AI/ML model supporting RSRP reporting, an issue arose in the last meeting: whether RSRP and beam ID both shall be tested?

Furthermore, in the context of AI/ML models supporting RSRP reporting, a key issue was raised in the last meeting: should both RSRP and beam ID be subject to testing?

First of all, to our understanding, for AI/ML models that output both label (i.e., beam ID) and RSRP, it is essential that the evaluation metrics account for both prediction accuracy and RSRP accuracy.

An objection to the proposal may be that test metrics can follow the legacy way in which only RSRP is required. From our perspective, only verifying RSRP may be applicable when the test only tests one or very limited number of beams with deterministic beam ID/index information. However, it is different in beam prediction test case. In the UE reporting, a set of beams is typically predicted, indexed, and sorted in the order of RSRP.

Taking the top-1 predicted beam as illustrative example, we may list two accuracy requirements concerning RSRP accuracy and prediction accuracy.

- To ensure RSRP accuracy, the predicted L1-RSRP of the top-1 beam – the groundtruth of L1-RSRP of the top-1 beam is less than or equal to y dB.

- To ensure prediction accuracy, the ground-truth RSRP of top-1 beam is larger than or equal to the ground-truth RSRP of the top-1 genie-aided beam  – x dB.

If only RSRP accuracy is assessed —without any comparison to the genie-aided beam (i.e., the ideal beam determined with full knowledge)—there is no guarantee that the top-1 predicted beam is meaningfully related to the optimal beam. In such cases, a poorly performing prediction model could still satisfy RSRP accuracy requirements while selecting a beam that is far from optimal, undermining the purpose of accurate beam prediction.

Observation 3: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlining problem is that the Top-1/K predicted beam is far away from the Top-1/K genie-aided beam.

Proposal 6: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously.

Proposal 7:  For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type. Proper thresholds for the dual requirements shall be evaluated.

Issue 2-2: Absolute RSRP accuracy

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

Also,

Agreement in RAN4#112: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In the conventional measured L1-RSRP definition, only top-1 of measured beams is reported in absolute RSRP format. The precise wording from TS 38.214, Section 5.2.1.4.3 is reproduced below.

For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size. If the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance.

In a similar manner, one could agree that the absolute RSRP accuracy requirement applies only to the top-1 predicted beam.

However, it’s worth noting that this approach defined by RAN1 optimizes the reporting by reducing the amount of data transmitted, especially when multiple beams are involved, which is served from RAN1 perspective. On the contrary, from the perspective of test robustness, we propose that the absolute RSRP accuracy requirement should apply to the other predicted beams besides of top-1 predicted beam.

Yet, from the definitions in TS38.133, as quoted in the below, we don’t see that RAN4 differentiates the absolute L1-RSRP accuracy and the relative L1-RSRP accuracy for different SSB resource sets. It seems appear that all measurements are expected to meet the absolute L1-RSRP accuracy requirement, at a minimum.

##### 10.1.19.1 SSB based L1-RSRP accuracy requirements ##### Unless otherwise specified, the requirements for absolute and relative SSB based L1-RSRP accuracy in this clause apply to all SSBs of the serving cell configured for L1-RSRP measurement and all SSBs of cell(s) with different PCI from serving cell configured for L1-RSRP measurement in FR1.

##### 10.1.19.1.2 Relative Accuracy The relative SSB based L1-RSRP accuracy is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell (serving cell or cell with different PCI from serving cell) on which UE performs L1-RSRP measurements.

Proposal 8: The absolute RSRP accuracy requirement applies to other beams besides of the top-1 of predicted beams.

Yet there exists a leftover regarding the absolute RSRP accuracy with respect to the texts in the bracket in the agreement: ‘[in top-K beams based on groundtruth L1-RSRP]’.

In our opinion, the beams that shall be taken care are the predicted beams. In other words, the prediction (accuracy) shall focus on the beams that the UE reports but not the beams supplied by the genie way. It's questionable to adopt the top-K beams w.r.t. the ground truth RSRP. Two concerns are provided below:

- If the requirement is only relevant to the top-K beams with respect to the ground truth RSRP, does the beam's RSRP have no requirements if it is present in the top-K predicted beams but not in the genie beams?

- Conversely, why should we need the RSRP accuracy of a beam that the UE doesn't even report if it is present in the top-K genie beams but not in the predicted beams?

Proposal 9: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting.

There arises another question, if the top-K beams are the beams to be evaluated in RSRP requirements, how about the other predicted beam besides of the top-K beams, especially if all predicted beams meet the SNR side condition for BM prediction?

Proposal 10: RAN4 to determine the test metric scope for the cases of RSRP reporting, we prefer option 2 between the two below options.

- Option 1: Only the top-K beams shall be verified.

- Option 2: All Set A beams meeting the SNR side condition for BM prediction shall be verified.

With same concerns about SNR side condition in the above context, even the top-K beams were discussed intensively in meetings, from test configuration perspective, as we see it, all Set A beams shall meet the side condition for BM prediction.

Proposal 11: RAN4 to study specifying the SNR side condition of the test metrics for top-K beams for the cases of RSRP reporting, we prefer option 2 between the two below options.

- Option 1: Top-K beams in the test metrics shall meet the side condition for BM prediction.

- Option 2: All Set A beams shall meet the side condition for BM prediction, in which, top-K has higher SNR side condition which shall be defined.

Issue 2-2: Relative RSRP accuracy

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Firstly, recap the legacy relative RSRP accuracy definition in Clause 10.1.20.1.2 of TS38.133, which is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The relative accuracy requirement is designed for the comparison between multiple RSs, relative accuracy ensures that any measurement error in a single RSs is less impactful. In this way, satisfying the relative accuracy requirement essentially reduces the likelihood of measurement inconsistencies across SSBs.

Hence, for serving the same purpose, the reference L1-RSRP of relative accuracy shall be assessed in prediction area.

In legacy, there is no doubt that the reference L1-RSRP is the largest measured value of L1-RSRP. Differently, in AI/ML based beam management, since a definition of relative L3 RSRP accuracy has been introduced in the WI on AI for mobility, it is reasonable to adopt and align with that existing definition for evaluating AI/ML-based beam prediction performance: ‘Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].’

Regarding the first FFS ‘whether reported L1-RSRP can be measured RSRP’, the origin of the question is that RAN1 has studied that the UE is able to use measured RSRP instead of predicted RSRP in prediction report, as duplicated in the below.

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

However, from a testing perspective, it is our understanding that beams used for prediction evaluation should not have associated measurement configurations or measurement results. This is to ensure a clear separation between predicted and measured data, thereby preventing any bias or overlap that could compromise the integrity of the prediction performance assessment.

Proposal 12: In practice, reported L1-RSRP can be predicted or measured RSRP, it’s up to NW configurations. But, for test perspective, reported L1-RSRP shall be predicted and not measured RSRP by managing proper test configurations.

Regarding the second FFS ‘Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2’,  we do not identify any technical impediment to extending the relative RSRP accuracy requirements to Case 2. In our view, the application of such requirements in Case 2 is both feasible and consistent with the principles already established for Case 1.

Proposal 13: Relative RSRP accuracy requirement is applicable for Case 2.

Issue 2-3: Measurement period for inference

Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

In conventional way, measurement delay normally refers to Clause 9.5.4 of TS38.133, ‘When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.’. Given that, the requirement derives the minimal delay in which the UE shall be able to complete an entire L1-RSRP measurement while satisfying the accuracy.

However, those requirements may be different in the predication case. The timeline concerning measurement and inference phases in prediction in Figure 1. Note: those variants in the figure are just for interpretation demonstration.

Figure 1 Timeline of measurement and inference phase in prediction

It’s noted that BM case 1 and BM case 2 may have different set A and set B configurations. Also, the length of measurement/inference for BM case 1 and BM case 2 are typically different, e.g.,

- m, n for BM case 1 typically equal to 1, i.e., one-shot measurement and one-shot inference.

- On the contrary,  m, n for BM case 2 typically are larger than 1, for tracking beam changes.

- Wherein, m, n is the number of reference signal measurement occasions or predication occasions respectively. Normally, the exact number of m, n shall be configured by the NW or up to UE implementation/capability or predefined in specification.

Regarding measurement delay, which may be defined by one of the below options:

- Delay of single/one shot measurement occasion. Note: it is identical to measurement delay in legacy.

- Delay of all measurement occasions for prediction, including m measurement occasions.

Regarding measurement accuracy, a unified accuracy requirement shall be upheld for each L1-RSRP measurement delay in both BM-case 1 and BM-case 2. In other words, from an accuracy perspective, there is no distinction between BM-case 1 and BM-case 2. Furthermore, no necessity for accuracy enhancement has been identified thus far.

A single-shot measurement occasion shall be applicable to both BM-case 1 and BM-case 2, ensuring that an entire measurement is completed to obtain a qualified RSRP result. Consequently, further restrictions on the accumulated measurement accuracy—averaged across multiple measurement occasions—are unnecessary, provided that the accuracy of each individual measurement occasion is guaranteed as today.

In our opinion, multiple measurement occasions for BM case 2, typically are not intended to obtain an accumulated higher measurement accuracy. Instead, they mainly target facilitating beam characteristic analysis, which is typically a proprietary process managed by AI/ML model vendors, such as analyzing UE movements.

Proposal 14: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied, with the assumption that the measurement accuracy remains consistent.

Proposal 15: RAN4 to study whether specific requirements are necessary to evaluate the capability of a UE in tracking changes in L1-RSRP through measurements in BM-case 2.

Issue: Performance monitoring

Based on RAN1’s agreements, we may focus on Type 1, Option 2 of UE-side model monitoring.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In RAN WG1 #118, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN WG1 #118bis, the below agreements were achieved.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

In RAN WG1 #119, the below agreements were achieved.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Our understanding is that for both BM-Case 1 and BM-Case 2, when utilizing a UE-sided AI/ML model, in the context of Option 2 (UE-assisted performance monitoring), the UE  should be capable of comparing the differences between prediction results and measurement results across a designated resource set or specific resources used for monitoring.

Based on RAN1 agreements to date, we do not identify any fundamental differences between measurement/inference delay for prediction and measurement/inference delay for monitoring, given that the evaluation metric remains the same (e.g., Top-1 or Top-K beam prediction accuracy). Furthermore, no proposals or agreements have indicated a distinction between the two ones. However, the final selection of the metrics may differ, considering their respective objectives and use cases.

Proposal 16: RAN4 shall define measurement delay for monitoring.

Observation 4: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes.

Issue 3-3: TCI State knowledge

To our understanding, the UE is able to acquire its Rx beam, where the Rx beam is speculated through the process of Rx beam sweeping in order to detect and receive reference signals/beams, at the least in measurement phase. In the consequent inference phase, the UE may keep or discard the Rx beam information depending on UE capability, since the AI/ML model doesn’t necessitate the Rx beam as input/output. Even not mandating the Rx beam information, it appears that retaining the Rx beam information may gain the prediction performance enhancement from the association between the Tx beam and Rx beam if the UE is willing to do so, which is applicable for both two output types of the AI/ML model.

- For the label type, the AI/ML model type is classifier network.

- For label & RSRP type, the AI/ML model type is CNN or MLP network.

Taking AI/ML model into account, if the known condition of the TCI state switch still relies on the legacy way, e.g., depending on the criteria whether the RS has been measured or not, no gain is obtained from the AI/ML model application.

Such that, we suppose that switching to a TCI state, to which the RS referred is predicted in absence of measurement, is a crucial improvement on beam management speeding up the TCI state switch procedure delay. In this manner, the criteria for known conditions regarding the predicted RS instead of the measured RS must be reconstructed. Where, knowing the Rx beams is a crucial factor in TCI state switch enhancement with respect to prediction.

Proposal 17:  RAN4 shall study the new known condition of the TCI state switch, to which the RS referred is which is ‘known’ by the prediction without any measurement.

Proposal 18: Knowing the Rx beams is a crucial factor in TCI state switch enhancement in terms of RS prediction.

## Testability and interoperability

Issue 4-1: Baseline test setup

- Proposals - Option 1: Single AoA setup (different Tx beams emulated only with power difference?) - Is it possible to test if UE can also make a prediction when set B beams come from different directions? - How many Tx beams can be emulated when the spatial domain properties are lost? - Will UE need just a simple interpolation model or is a more complex model necessary to pass such a test? - Can additional (L1-)RSRP measurement accuracy requirement (e.g. spherical coverage) be used to complement Single AoA testing with directional/spatial UE performance? - Option 2: Single AoA setup with “two stage approach” - Will a UE which uses Rx beam information in the prediction model have to also implement a model which does not use the Rx beam information? - Are there any implementation constraints on a UE to execute/pass this test? - How will the TE know the UE Rx beam pattern? - Option 3: multi AoA setup– RRM test setup as baseline (including enhancement for up to 4AoA active probes) - How many Tx beams can be emulated with such a setup, what is the relationship between number of probes and number of set B Tx beams? - Can AoAs between the probes also be emulated (is it possible to “make it look like” signals are coming from other angles than where the probes are placed?) - Can a simplified CDL be emulated or only TDL from each probe? - Option 4: multi AoA setup – 3D MPAC system as baseline - How many Tx beams can be emulated with the baseline setup? - 4Tx beams with dual pol(set B?) can be emulated with 2 additional probes Pros&cons of different test setups - Option 5: multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. Single AoA; Single AoA with two stage; Multi AoA – RRM test setup as baseline; Multi AoA 3D MPAC. Tests that UE uses spatial information(UE actually makes measurement in multiple directions to make a prediction); No; Maybe?; Yes; Yes. Relevance for field performance; Low; Maybe?; Good/High; Good/High. Simplified CDL emulation; No; Yes; Yes. Test setup complexity; Low; Low; Medium; Medium/High?. Test method complexity; Low; High; Medium; Medium/High?. Reuse of test setup for other tests; Medium (multi AoA testing already exists in RRM); High?; Low/medium? Can only reuse the test setup for FR2 MIMO OTA?. - Recommended WF - Do not further discuss single AoA test setups, continue to study the two multi AoA candidate setups

Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

Issue 4-2: Channel model

- Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others - Recommended WF - To be discussed If simplified CDL is to be further considered, more detailed proposals on what to study/analyze further should be discussed (e.g. RX PAS, number of relevant clusters to emulate, dependency on number of clusters and number of probes in the chamber, etc)

Agreement: - Use CDL-based channel model as starting point

From AI/ML model testing perspective, it’s clear that there exist two options in last meetings.

- Option 1:  only inputting L1-RSRP of beams, single AoA with CDL/TDL channel can be applied.

- Option 2:  inputting beam information comprising L1-RSRP, Tx beam spatial distribution and Rx beam spatial distribution, multiple AOA with CDL channel shall be applied.

Option1 supposes that by emulating artificial Tx/Rx beam into the incoming beams, the single AoA test configuration can generate the needed L1-RSRP of beams.

Figure 3 Single AOA flow

However the practical Rx beam plays a crucial role in FR2 UE performance, a question to Option 1 is whether the difference between really tested Tx/Rx beam and the artificial Tx/Rx beam can impact AI/ML model.

Proposal 19: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model?

The simplification of the CDL channel is achieved by constraining the directions Tx beams and and AOA/AOD. Observations regarding possible approaches to simplifying AOA/AOD can be found in R4-2500825, R4-2411254, and other relevant contributions. The simplified CDL channel consolidates similar AOA/AOD angles to enable spatial evaluation with a reduced number of probes.

Given that the maximal number of beams to be measured is 32, a important consideration is whether the 32 beams after simplified CDL channel can be produced by 8 or 12 probes thereby achieving the balance between performance and implementation complexity.

Proposal 20: RAN4 to check if the 32 beams after simplified CDL channel can be produced by 8 or 12 probes which may achieve the balance between performance and implementation complexity.

A test solution involving the measurement of different Tx/Rx beams using a time-division multiplexing (TDM) approach was proposed in previous meetings. While we remain open to this approach, the method for controlling Rx switching requires further clarification.

Proposal 21: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed.

The simplified CDL channel can, in principle, be applied to both training and inference. However, within the scope of common test configurations, only inference needs to be specified, as training is not part of the conformance test. Consequently, a comparison between the simplified CDL channel and the reference CDL channel during training is not required. In other words, training data may be generated using the reference CDL channel.

Proposal 22: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 23: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Issue: Beam prediction testing process

- Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others - Recommended WF - Take Option 1 as baseline, discuss further details of tests such as under what condition should the ground truth be measured, how to ensure the UE does not cheat, etc.

Below are some preliminary analyses on the condition of ground truth, e.g., the SNR condition.

The purpose of defining the SNR condition is to get an ideal RSRP measurement accuracy by minimizing (most of) measurement errors caused by low SNR. Hence, determining the SNR condition may rely on the configurations of reference signal, e.g., periodicity, density, bandwidth, subcarrier spacing and channel model in order to obtain a SNR level which is applicable for most typical field scenarios.

Recap the procedure deriving the accuracy requirement on the measured L1-RSRP in Rel-15, the achievable accuracy at certain SNR level is derived by simulations. In a same way, it is rational to extend those simulations to a higher SNR level where the L1-RSRP estimation error can be less than a threshold with X percentile.

Generating such SNR level in the test environment is another issue to be addressed, a concern is that the realization of SNR level is not only about generating a proper signal level but also the noise floor level generated as a side effect needs to be considered. To realize high SNR, the probes in test environment shall be able to generate signals at high output power level over the air. An underlying problem is that the wanted signal level can sometimes become too high to be created in the OTA environment. In that case, the noise floor becomes dominant which may deteriorate the test performance. Test equipment vendors are encouraged to comment on the SNR level from a test environment perspective.

Proposal 24: RAN4 to evaluate the SNR condition for the ground truth by simulation.

If the ideal measurement of RSRP is acquirable, then the test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Still, arranging the details on the sequence and test configuration may need further investigation. In essence, to our understanding, the UE is deemed to execute the RSRP measurement and prediction with same test configurations except for different SNR conditions.

Proposal 25: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Issue: OTA Test setup for beam management

In this issue, we discuss the setup of channels to be applied in measurement and inference.

For BM-case1, which predicts the best beams in Set A (different from Set B) with respect to the measurements on Set B, at the same time domain with respect to spatial correlation between Set B and Set A. Since the time delay of measurement delay and inference delay is in dozens or hundreds of milli-seconds order, there’s no chance to rearrange random distributions and variations between measurement and prediction, the channel for measurement and prediction have to apply a same channel.

For BM-case2, which harnesses the historical predictable non-random variation of Set B to anticipate Set A (same as Set B) for one or more future time instances with respect to temporal correlation between Set B and Set A, from test perspective, it needs some trackable changes, e.g., UE monotonous movements which reflect the temporal correlation as prerequisite of the temporal prediction.

A question is relevant to measurement window and the inference window, which are configured by NW, may have respective minimal length with respect to a certain prediction performance. Such that a set of consequent changes of channel parameters shall be designed during measurement window and the inference window. At the least, establishing a set of channel instants from channel #a+1 to channel #a+m for measurement window and channel #b+1 to channel #b+n for inference window.

These channels contain a set of parameters which update sequentially and gladfully starting from measurement until end of inference. The parameters wherein to be updated may comprise one or more than one of the below:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- UE position and/or rotation

- Channel coefficient

In an illuminative example, for BM-case 2, the updates of parameters, in order to mimic the UE’s movement, may include the below:

- AOD in channel #1 is 10 degrees,

- AOD in channel #2 is 20 degrees,

- …,

- AOD in channel #k is (10*k) degree

Proposal 26: For BM-case1, same channel is applied for measurement and inference.

Proposal 27: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

Where,

- The channels instants comprise sequent and grade updates of a set of parameters.

- The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

Issue:	Datasets for training/testing

Agreement: Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

The rationale behind the agreements on the issue is that the coverage of datasets for training/testing shall be broad adequately to meet or be close to the practical field deployment scenario as much as possible. In this manner, we suggest considering defining the test environment/conditions which can demonstrate the robustness of the AI/ML model.

Proposal 28: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

Since the training data are uniquely affected by the specific UE hardware and digital processing units, the error of training data could be left for implementation from each UE vendor, the effect to the predict accuracy may be covered by the overall measurement error.

Observation 5: the measurement error in datasets for training/testing may be covered by the overall measurement error.

# Conclusion

Observation 1: If only beam ID is reported, the order of top-K beams is able to indicate the order of RSRP implicitly.

Observation 2: From the network perspective, obtaining a plurality of predicted beams, each subject to specific prediction accuracy requirements, is advantageous for enhancing reliability and robustness in beam management.

Observation 3: For AI/ML model supporting RSRP and beam ID reporting, if only RSRP accuracy is required, an underlining problem is that the Top-1/K predicted beam is far away from the Top-1/K genie-aided beam.

Observation 4: The metric/report type of prediction results, prediction accuracy or L1-RSRP offset, for prediction and the metric for monitoring may be different since they serve different purposes.

Observation 5: the measurement error in datasets for training/testing may be covered by the overall measurement error.

Proposal 1:  Beam ID prediction metrics and KPIs should include evaluations for both K = 1 and K = [2] to ensure a comprehensive assessment of prediction performance.

Proposal 2: RAN4 to determine the test scope for the case of beam ID being reported, we prefer option 2 between the two below options.

- Option 1: Only the top-K beams shall be verified.

- Option 2: All Set A beams meeting the SNR side condition for BM prediction shall be verified.

Proposal 3: RAN4 to investigate the SNR side condition of the test metrics for the cases of beam ID reporting. Two potential options are under consideration in the context of beam ID reporting:

- Option 1: Top-K beams in the test metrics shall meet the side condition for BM prediction.

- Option 2: All Set A beams shall meet the side condition for BM prediction, in which, top-K has higher SNR side condition which shall be defined.

Proposal 4: The parameter x in beam prediction accuracy metrics should be set to a value that reflects the expected magnitude of measurement error and other errors.

Proposal 5: We’re open to introducing N (N is [2]) to enable verifying the RSRP of each beam within the top-N predicted beams. If N is introduced, the metrics may be represented to:

- The ground-truth RSRP of the 1st predicted beam larger than or equal to the ground-truth RSRP of the 1st strongest  genie-aided beam(s) – x1 dB,

- The ground-truth RSRP of the 2nd predicted beam larger than or equal to the ground-truth RSRP of the 2nd strongest  genie-aided beam(s) – x2 dB,

- …

- The ground-truth RSRP of the Nth predicted beam larger than or equal to the ground-truth RSRP of the Nth  strongest genie-aided beam(s) – xN dB

Proposal 6: Metrics/KPIs for beam management requirements/tests are outlined by the below options:

- In case of the AI/ML model with label (beam ID) output type

  - Beam prediction accuracy is adopted.

- In case of the AI/ML model with label (beam ID) + RSRP output type

  - Beam prediction accuracy and RSRP accuracy, are adopted simultaneously.

Proposal 7:  For the case of the AI/ML model with label (beam ID) + RSRP output type, fulfilling beam prediction accuracy and RSRP accuracy may result in more stringent requirements compared with the case of the AI/ML model only with label (beam ID) output type. Proper thresholds for the dual requirements shall be evaluated.

Proposal 8: The absolute RSRP accuracy requirement applies to other beams besides of the top-1 of predicted beams.

Proposal 9: For the absolute RSRP, the index i shall be any beam index in all beams based on predicted L1-RSRP as per UE reporting.

Proposal 10: RAN4 to determine the test metric scope for the cases of RSRP reporting, we prefer option 2 between the two below options.

- Option 1: Only the top-K beams shall be verified.

- Option 2: All Set A beams meeting the SNR side condition for BM prediction shall be verified.

Proposal 11: RAN4 to study specifying the SNR side condition of the test metrics for top-K beams for the cases of RSRP reporting, we prefer option 2 between the two below options.

- Option 1: Top-K beams in the test metrics shall meet the side condition for BM prediction.

- Option 2: All Set A beams shall meet the side condition for BM prediction, in which, top-K has higher SNR side condition which shall be defined.

Proposal 12: In practice, reported L1-RSRP can be predicted or measured RSRP, it’s up to NW configurations. But, for test perspective, reported L1-RSRP shall be predicted and not measured RSRP by managing proper test configurations.

Proposal 13: Relative RSRP accuracy requirement is applicable for Case 2.

Proposal 14: Regarding measurement period for prediction, for both BM-case1 and BM-case2, the legacy L1-RSRP measurement delay shall be applied, with the assumption that the measurement accuracy remains consistent.

Proposal 15: RAN4 to study whether specific requirements are necessary to evaluate the capability of a UE in tracking changes in L1-RSRP through measurements in BM-case 2.

Proposal 16: RAN4 shall define measurement delay for monitoring.

Proposal 17:  RAN4 shall study the new known condition of the TCI state switch, to which the RS referred is which is ‘known’ by the prediction without any measurement.

Proposal 18: Knowing the Rx beams is a crucial factor in TCI state switch enhancement in terms of RS prediction.

Proposal 19: RAN4 to study the two questions regarding multiple/single AoA:

- What is the (maximal) difference between really tested Tx/Rx beam and the artificial Tx/Rx beam?

- If the difference can impact AI/ML model?

Proposal 20: RAN4 to check if the 32 beams after simplified CDL channel can be produced by 8 or 12 probes which may achieve the balance between performance and implementation complexity.

Proposal 21: Measuring Tx/Rx beams using a TDM approach may be a valid method; however, the control mechanism for Rx switching must be explicitly addressed.

Proposal 22: The simplified CDL channel only is considered to be applied in inference, no need to be applied in training.

Proposal 23: The metric to estimate the “goodness” of a simplified CDL channel model may be represented by the below steps:

- Step 1: Test (inference) with the reference CDL channel.

- Step 2: Test (inference) with the simplified CDL channel.

- Step 3: The prediction difference between the results of the two tests in the above steps is less than a threshold with [95] percentile.

Proposal 24: RAN4 to evaluate the SNR condition for the ground truth by simulation.

Proposal 25: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Note 1: Test configurations are kept same in the steps except for different SNR conditions.

Note 2: Arranging the details on the sequence and test configuration may need further investigation in performance part.

Proposal 26: For BM-case1, same channel is applied for measurement and inference.

Proposal 27: For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

- Generating a set of channel instants, from channel #a+1 to channel #a+m, for measurement, and

- Generating a set of channel instants, from channel #b+1 to channel #b+n for inference

Where,

- The channels instants comprise sequent and grade updates of a set of parameters.

- The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

Proposal 28: In order to avoid overfitting UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

# References

- R4-2505105 WF on AI/ML, Qualcomm