3GPP TSG-RAN WG4 Meeting #115	R4-2506348

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for Ambient-IoT

Agenda Item:	7.26.4

Document for:	Discussion

# Introduction

During RAN4#114bis, the RRM requirements for Ambient IoT were down selected after discussions, at the end, the issues and agreements approved from at RAN#106 meeting. Some relevant content is duplicated in the below as:

This contribution outlines our point of view on the leftover as follows.

# Discussion

Issue 1: timing

In RAN4#114-bis the following WF was approved [2]:

… Topic #1: RRM core requirements (7.24.3) ## Issue 1-1: A-IoT paging Agreement: - RAN4 not to define RRM requirements for A-IoT paging ## Issue 1-2: A-IoT data transmission Agreement: - Excepting the timing issue discussed in Issue 1-4, no other potential RRM requirement impact is identified for “A-IoT data transmission” ## Issue 1-3: A-IoT random access procedure Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details ## Issue 1-4: timing Agreement: - Further discuss whether there is RRM impact on transmit timing requirements of device pending on RAN1 conclusion. …

In this section we focus on issue 1-4, timing.

RAN1 will have its last meeting in May. The agreements up to and including April list a few topics where RAN4 agreements might have to be considered [3].

… R1-2502500 FL summary #3 on other aspects for Rel-19 Ambient IoT Moderator (vivo) Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. …

In the summary above it is stated:

For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective.

  - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion.

  - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19

  - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2

RAN1 leaves initial sampling frequency offset, SFO, up to RAN4 discussion and notes that SFO may still be considered when discussing time-domain resources for Msg1 when X=2.

Observation 1: RAN1 leaves initial sampling frequency offset, SFO, up to RAN4 discussion.

The observation that SFO may still be considered for X = 2 is correct since that represents a limiting and dimensioning case for device UL timing. For X=2 the D2R link has two separate TDMA slots and a late and Msg 1 or Msg3 might crash into a subsequent message sent slightly early. The typical minimum separation is depending on SFO and becomes:

2*SFO*Tmsg

This is shown in Figure 1.

Figure 1: Random access procedure with X>1 (top) versus X=1 (bottom)

Observation 2: 	For X=2 the D2R link has two separate TDMA slots and a late and Msg 1 or Msg3 might crash into a subsequent message sent slightly early.

RAN1 has the following working assumption [3]:

Working assumption

For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource:

- Derived based on the starting time of the first Msg1 time domain resource plus Toffset2

- FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

The calculation of Toffset2 will become a function of SFO. In our RAN1 contribution [4], we conclude:

### Calculation of Toffset2

Starting time for second Msg1 time resource however is not as straight-forward because of the following additional dependencies.

- Impact of SFO and the guard time associated with the SFO

With SFO of 10%, a guard time needs to be accounted between the TDMA occasions for each message to prevent any timing overlaps, which prolongs access occasion duration. This guard time is calculated as G= 2*(TMsg1)*SFO between the Msg1 TDMA occasions so as to prevent timing overlaps, where TMsg1 is the time duration of Msg1 transmission.

- Variabilities in Msg1 lengths: Even if the same data rate is assumed for all FDMed Msg1 transmissions, FLS on 9.4.4 [5], discusses that Msg1 scheduling supports the following fields which can introduce variabilities in Msg1 lengths -

- Preamble lengths – Short and long preamble support

- Midamble details

- FEC applicability/not

- Block repetition support

Finaly in [4] we conclude and propose that Toffset2 needs to be configured by the reader as part of Msg1 scheduling. The parameter Toffset2 is a function of SFO, as shown in (a) above.

Observation 3: 	Calculation of Toffset2 needs to be configured by the reader as part of Msg1 scheduling and Toffset2 is a function of SFO.

Because of this there exist a need to investigate the need of an SFO minimum requirement.

Proposal 1: RAN4 to investigate the need of an SFO minimum requirement.

Issue 2: Random access

Agreement: - RAN4 to include RRM requirements for correct A-IoT device behaviour during CBRA and CFRA. FFS the details

Since RAN1/2 are still working for random access, we may revisit the issue capturing ‘correct behaviour’ after the spec. by RAN1/2 are mature.

Proposal2 : RAN4 to revisit the issue capturing ‘correct behaviour for random access’ after the spec. by RAN1/2 are mature.

# Conclusion

Observation 1: RAN1 leaves initial sampling frequency offset, SFO, up to RAN4 discussion.

Observation 2: 	For X=2 the D2R link has two separate TDMA slots and a late and Msg 1 or Msg3 might crash into a subsequent message sent slightly early.

Observation 3: 	Calculation of Toffset2 needs to be configured by the reader as part of Msg1 scheduling and Toffset2 is a function of SFO.

Proposal 1: RAN4 to investigate the need of an SFO minimum requirement.

### Proposal 2: RAN4 to revisit the issue capturing ‘correct behaviour for random access’ after the spec. by RAN1/2 are mature.

# References

- RP-250796, New WID on Rel-19 Ambient IoT

- WF on RRM requirements for Ambient_IoT_Solutions, CMCC

- R1-2502500, FL summary #3 on other aspects for Rel-19 Ambient IoT	Moderator (vivo)

- R1- 2503223, Other aspects for Ambient IoT, Ericsson