3GPP TSG-RAN WG4 Meeting #115	R4-2506352

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for Ku bands for NR NTN

Agenda Item:	7.11.7

Document for:	Discussion

# Introduction

In the light of the WI for NR_NTN_Ku_bands, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

Issue 1-1: RRM scope for NR NTN on Ku bands

Agreement: - Define RRM requirements for added 15kHz SCS in FR1-NTN numerology for GSO in Ku-band

The agreement intends to confirm 15kHz SCS applicability in FR1-NTN numerology for GSO in Ku-band. It means that 15KHz and 30KHz SCS are supported by Ku-band FR1-NTN numerology VSAT UE.

The RRM requirements relevant to 15KHz SCS are UL transmit timing requirements and timing advance requirements, for Ku-band FR1-NTN numerology VSAT UE, they shall align with the requirements for the existing FR1-NTN UE with15kHz SCS.

Observation 1:  For Ku-band FR1-NTN numerology VSAT UE, with the addition of 15KHz SCS support, the SCS-aware requirements, i.e., UL transmit timing and timing advance, is identical to those requirements already specified for the existing FR1-NTN UE with15kHz SCS.

Issue 1-2: applicability update (section 3.6.12).

Proposals: - Option 1: The requirements for FR2-NTN/NTN band above 10GHz except for timing requirements in Clause 7 are applicable to Ku-band NTN with FR1-NTN and FR2-NTN numerologies. (LGE) Recommended WF - Continue discussion

In radio link monitoring requirements, the evaluation periods differ between FR1 and FR2-NTN. In this case, Ku-band NTN with FR1-NTN shall reuse the requirements for FR1.

Observation 2: The Option 1 in proposal isn’t comprehensive. For RRC_IDLE state mobility and handover requirements, it may be appropriately correct. But signalling characteristics, the existing requirements for FR2-NTN/NTN band above 10GHz could not be directly applied for Ku-band NTN with FR1-NTN numerologies. It may be reflected in respective CRs.

Issue 1-4: inter-satellite cell reselection requirements (section 4.2C).

Proposals: - Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs in Rel-19. (Nokia) Recommended WF - Continue discussion

In RAN4 meeting#114, it is agreed that the below four cases shall be considered for mobile VSAT UE:

- Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology

- Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology

- Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology

- Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

Meanwhile, the definitions of NTN VSAT Types in TS38.101-5 as follow.

Table 9.2.1.0-1: The definitions of NTN VSAT Types NTN VSAT class; NTN VSAT type; Type description. Fixed VSAT; 1; Fixed VSAT communicating with GSO and LEO with mechanical steering antenna.. 22; Fixed VSAT communicating with GSO and LEO with electronic steering antenna.. 3; Fixed VSAT communicating with LEO only with electronic steering antenna.. Mobile VSAT; 4; Mobile VSAT communicating with GSO with mechanical steering antenna.. 52; Mobile VSAT communicating with GSO with electronic steering antenna.. NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction..

Upon comparison of the two lists of VSAT types, it appears that Case 3 corresponds to the existing UE type as defined in Table 9.2.1.0-1. Conversely, Cases 1, 2, and 4 represent newly proposed configurations.

In our opinion, all cases shall have similar antenna architecture, i.e., not beyond the baseline of introducing FR2-NTN VSAT UE in Rel-18 (i.e., Case 3).

Proposal 1: Not to introduce inter-satellite cell reselection requirements given no implementation differences in antenna or VSAT architecture between the FR2-NTN VSAT UE defined in Rel-18 and the Ku-band VSAT UE.

Issue 1-5: UE measurement capability (section 4.2C.2.1).

Proposals: - Option 1: For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least: - Intra-frequency carrier [in the case of VSAT UE with single receive beam, non-simultaneous monitoring intra-frequency carriers from more than one SAN], and - Depending on UE capability, FR1-NTN UE 7 NR inter-frequency carriers and for all VSAT UE up to [8] inter frequency carriers. (Eutelsat Group) Recommended WF - Continue discussion

In Release 18, no RRC idle or inactive mobility requirements are specified for FR2-NTN VSAT UEs in inter-satellite mobility scenarios. Consistent with the rationale outlined above, we do not support the introduction of a new UE capability for performing measurements on intra- or inter-frequency layers.

Proposal 2: Not to introduce inter-satellite requirements for idle mode mobility.

Issue 1-6: NR SAN FR2-NTN – NR SAN FR2-NTN Handover (section 6.1C.1.3)

Agreement: - Reuse the same value of T_sat_beam as in Ka band for Ku band - For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

In essence, the satellite switching with re-sync belongs to inter-satellite mobility scenario. From this standpoint, it is not different from handover, particularly when accounting for the delay of steering UE antenna.

We understand that the exclusion of satellite switching with re-synchronization for FR2-NTN VSAT UEs in Rel-18 is mainly due to the relatively late phase at which this feature was introduced. But from a technical perspective, satellite switching with re-sync for Ku-band VSAT UE is valid and feasible.

Proposal 3: We can agree to define requirements of satellite switching with re-sync for Ku-band VSAT UE operating in FR1-NTN numerology or FR2-NTN numerology.

Issue 1-9: timing accuracy requirements for case 4 (section 7.1C)

Agreement: - The same Te_NTN requirement applies to case 4. On top of the side conditions agreed for Rel-18 Ka band, RAN4 to add the following side condition to Te_NTN requirements for Case 4 - Option 1: Capture it as a NOTE on the test case in the core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec in the core corresponding test case. - Option 2: Capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec.

Both options work for us, i.e., to capture it as a NOTE on the test case in the performance requirement or to Capture it as a NOTE in core requirement.

Proposal 4: Both options work for us, regarding timing accuracy requirements for case 4 (section 7.1C), i.e. to capture it as a NOTE on the test case in the core requirement or to capture it as a NOTE in core requirement.

Issue 1-10: timing accuracy requirements applicability (section 7.1C)

Agreement: - The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. FFS how to capture it in the specification.

It has been agreed that the timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. This can be captured as a note in the mobile VSAT served by NGSO requirement table.

Proposal 5: Capture that the timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only as a normative note inside the relevant section 7.1C table.

Issue 1-12: RLM (section 8.1C)

Proposals: - Option 1: Confirm the assumption that LMax is equal to 8 for FR1-NTN (Nokia) Recommended WF - Continue discussion

For frequencies above 3GHz, Lmax is equal to 8. Regarding the frequency band scope of Ku-band as presented in WI, Lmax is equal to 8 for FR1-NTN.

Even the assumption that LMax is equal to 8 for FR1-NTN is correct, it doesn’t change the existing Table 8.1C.1-2 in TS 38.133.

Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM Carrier frequency range of PCell; Lmax; Maximum number of RLM-RS resources, NRLM. FR1, ≤ 3 GHzNote; 4; 2. FR1, > 3 GHzNote; 8; 4. FR2-NTNNote 2; 64; 8. NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3 GHz is replaced by 1.88 GHz, as specified in clause 4.1 in TS 38.213 [3]. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated..

Proposal 6: Confirm the assumption that LMax is equal to 8 for FR1-NTN, the confirmation doesn’t change the existing Table 8.1C.1-2 in TS 38.133.

Issue 1-13: terminology update

Proposals: - Option 1: change ‘FR2-NTN’ to ‘[single receive beam] VSAT UE’ (Eutelsat Group) - Option 2: others Recommended WF - Continue discussion on how to align terminology.

It is a recognized that the term ‘VSAT UE in FR2-NTN’ is included in RRM requirements to denote a VSAT UE operating in FR2-NTN network, reflecting the assumption that only the VSAT UE is applicable in in FR2-NTN. Meanwhile, the term ‘FR2-NTN’ is included in RRM requirements for the UE served by the FR2-NTN cell without explicitly indicating the UE type.

In our opinion, the assumption of only VSAT UE being applied in FR2-NTN still is valid, since that there is no urgent demand to revisit ‘VSAT UE in FR2-NTN’ from UE perspective or ‘FR2-NTN’ from network perspective.

Proposal 7: For Ku-band in FR2-NTN numerology, not to change the existing terminologies: ‘VSAT UE in FR2-NTN’ and ‘FR2-NTN’.

Going back to Ku-band in FR1-NTN numerology, ‘UE in FR1-NTN’ or ‘FR1-NTN’ may be considered to cover the power class 3 UE (which is the legacy NTN UE in FR1-NTN numerology) and the VSAT UE. When a clear differentiation between these two types of UEs is required, the terms ‘power class 3 UE in in FR1-NTN’ and ‘VSAT UE in FR1-NTN’ can be applied then.

Proposal 8: For Ku-band in FR1-NTN numerology, the terminology ‘UE in FR1-NTN’ or ‘FR1-NTN’ may be regarded to cover the power class 3 UE and the VSAT UE. Where differentiation is necessary, the terminologies ‘power class 3 UE in in FR1-NTN’ and ‘VSAT UE in FR1-NTN’ can be used accordingly.

# Conclusion

Observation 1:  For Ku-band FR1-NTN numerology VSAT UE, with the addition of 15KHz SCS support, the SCS-aware requirements, i.e., UL transmit timing and timing advance, is identical to those requirements already specified for the existing FR1-NTN UE with15kHz SCS.

Observation 2: The Option 1 in proposal isn’t comprehensive. For RRC_IDLE state mobility and handover requirements, it may be appropriately correct. But signalling characteristics, the existing requirements for FR2-NTN/NTN band above 10GHz could not be directly applied for Ku-band NTN with FR1-NTN numerologies. It may be reflected in respective CRs.

Proposal 1: Not to introduce inter-satellite cell reselection requirements given no implementation differences in antenna or VSAT architecture between the FR2-NTN VSAT UE defined in Rel-18 and the Ku-band VSAT UE.

Proposal 2: Not to introduce inter-satellite requirements for idle mode mobility.

Proposal 3: We can agree to define requirements of satellite switching with re-sync for Ku-band VSAT UE operating in FR1-NTN numerology or FR2-NTN numerology.

Proposal 4: Both options work for us, regarding timing accuracy requirements for case 4 (section 7.1C), i.e. to capture it as a NOTE on the test case in the core requirement or to capture it as a NOTE in core requirement.

Proposal 5: Capture that the timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only as a normative note inside the relevant section 7.1C table.

Proposal 6: Confirm the assumption that LMax is equal to 8 for FR1-NTN, the confirmation doesn’t change the existing Table 8.1C.1-2 in TS 38.133.

Proposal 7: For Ku-band in FR2-NTN numerology, not to change the existing terminologies: ‘VSAT UE in FR2-NTN’ and ‘FR2-NTN’.

Proposal 8: For Ku-band in FR1-NTN numerology, the terminology ‘UE in FR1-NTN’ or ‘FR1-NTN’ may be regarded to cover the power class 3 UE and the VSAT UE. Where differentiation is necessary, the terminologies ‘power class 3 UE in in FR1-NTN’ and ‘VSAT UE in FR1-NTN’ can be used accordingly.

# References

- R4-2504897, WF on RRM requirements for NR_NTN_Ku_bands, Apple