3GPP TSG-RAN WG4 Meeting #115	R4-2506354

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Agenda Item:	7.31.5.1

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RedCap RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: General consideration on RRM requirements

Issue 1-1-1: SDT requirements

- Background RAN4#114: #### Issue 1-1-2: SDT requirements Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS. <Way Forward>: - Proposal 1: For the conclusion on how to define CG-SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, it is not necessary to send LS to RAN2. - The agreement reached in the last meeting only focus on RAN4’s requirements. - Proposal 2: RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

In TS 38.133, the requirements regarding TA validation for SDT are presented as follow.

### 5.2B.3 TA validation requirements for RedCap When cg-SDT-RSRP-ChangeThreshold TS 38.331 [2] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in TS 38.321 [7], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that - the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and, - timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in TS 38.321 [7].

In TS 36.133, the requirements regarding TA validation for PUR in TN are duplicated as follow.

- 4.7.4.2 Requirements on UE synchronization for transmission using PUR The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage. The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission. The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331]. - 4.7.4.3 Requirements on TA validation for transmission using PUR When rsrp-ChangeThresh [TS 36.331] is configured for TA validation based on the RSRP change criterion according to clause 5.3.3.19 in [TS 36.331], with or without other TA validation criteria, the UE is allowed to transmit using PUR using the timing derived using the latest available NTA value as specified in subclause 7.24.1 provided that - the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and, - timing alignment validation for transmission using PUR is valid according to the validation criteria in section 5.3.3. 19 in [TS 36.331] for all configured TA validation criteria.

In TS 36.133, the requirements regarding TA validation for PUR in IoT-NTN are duplicated as follow.

By observations to TA validation relevant requirements for RedCap TN, IoT TN and IoT NTN, RAN4 may focus on the requirements when no cg-SDT-RSRP-ChangeThreshold. Based on our interpretation of UE operational behaviour and the agreements in IoT NTN, A RedCap UE is able to manage TA validity by maintaining sufficient synchronization and timing accuracy independently even without checking RSRP.

We may reuse the statement on TA validation in TS 36.133 ‘The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission’.

Observation 1: When no cg-SDT-RSRP-ChangeThreshold, the UE is able to manage TA validity.

Proposal 1: RAN4 to define the SDT requirements as what have been done for PUR in IoT-NTN.

Issue 1-1-2: MDT requirements

- Background RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.. <Way Forward>: - Proposal 1: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN.

Proposal 2: Not to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap with NTN.

## Topic #3: The impact of HD-FDD

#### Issue 3-1-1: General HD-FDD related impact

Online agreement Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

Based on observations of the RAN1 agreement (duplicated below), there is no explicit prioritization of CSI-RS for CBD evaluation over uplink transmissions. Furthermore, since RAN2 has not yet defined a specific priority rule for this case. Given that, it is proposed that RAN4 may clarify the priority rule from the perspective of UE measurement implementation.

The [Minimum requirement for L1 indication for RLM and LR] in the issue is duplicated as follow.

#### 9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For FD-FDD and TDD RedCap UEs, there are no scheduling restrictions due to L1-RSRP measurement performed on SSB and CSI-RS configured as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1. For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

Proposal 3: RAN4 to decide that CBD evaluation follows the legacy requirements for HD-FDD RedCap UE.

The agreements in RAN1 in recent meetings are captured as follow:

Agreements in RAN1#116

Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching

Agreements in RAN1#117

Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

Agreements in RAN1#119

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration

Agreements in RAN1#120

Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS.

Agreements in RAN1#120-bis

Agreement Update the working assumption in RAN1 #120 with the following updates: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

#### Issue 3-2-2: The impact on other requirements due to HD-FDD

<Way Forward>: - Proposal 1: Clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected. - Proposal 2: For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured. - Proposal 3: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

We understand the proposal 1 may be derived from some priority rules e.g., up to UE implementation or NW indication in some cases. However, even it is outlined in RAN1’s agreements for some collision cases, we don’t observe skipping SSB or CSI-RS is explicitly stated in RAN1’s agreements.

Proposal 4: For HD-FDD (e)RedCap in NTN, RAN4 to confirm no measurement requirements with the RS(s) whose measurements may be skipped due to collision with UL transmission.

Regarding scheduling availability, our opinion is reusing the legacy approach, i.e., UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

Proposal 5: For HD-FDD (e)RedCap in NTN, support proposal 2. For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

# Conclusion

Observation 1: When no cg-SDT-RSRP-ChangeThreshold, the UE is able to manage TA validity.

Proposal 1: RAN4 to define the SDT requirements as what have been done for PUR in IoT-NTN.

Proposal 2: Not to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap with NTN.

Proposal 3: RAN4 to decide that CBD evaluation follows the legacy requirements for HD-FDD RedCap UE.

Proposal 4: For HD-FDD (e)RedCap in NTN, RAN4 to confirm no measurement requirements with the RS(s) whose measurements may be skipped due to collision with UL transmission.

Proposal 5: For HD-FDD (e)RedCap in NTN, support proposal 2. For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured.

# References

- R4-2504912, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT