3GPP TSG-RAN4 Meeting # 115	R4-2506357

# St Julian's, Malta,  19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_Ph3-Core) Draft CR on measurement requirement for RedCap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2015-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add measurement requirement for RedCap UEs with NTN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurement requirement for RedCap UEs with NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for RedCap UE in NTN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Add clauses: 9.1X, 9.2X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

## 9.1X	General measurement requirement for RedCap with SAN

### 9.1X.1	Introduction

This clause contains general requirements on the UE regarding measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency, inter-frequency and L1-RSRP measurements requirements. These measurements may be used by the NG-RAN. The measurement quantities are defined in TS 38.215 [4], the measurement model is defined in TS 38.300 [10], TS 37.340 [17] and measurement accuracies are specified in clause 10. Control of measurement reporting is specified in TS 38.331 [2].

### 9.1X.2	Measurement gap

For the UE configured with only PCell, which is served by SAN, if the UE requires measurement gaps to identify and measure cells operating in satellite access network and/or TN cells, in order for the requirements in the following clauses to apply the network must provide at least one per-UE measurement gap and the number of per-UE measurement gaps provided by the network must not exceed the number of measurement gaps supported by the UE, indicated by the capability parallelMeasurementGap.

If the UE is configured via LPP [34] to measure PRS for UE Rx-Tx time difference measurement defined in TS 38.215 [4], in order for the requirements in clauses 9.9C.4 to apply, the network must provide at least one per-UE measurement gap pattern for concurrent monitoring of all positioning frequency layers and intra-frequency and/or inter-frequency layers. The number of per-UE measurement gaps provided by the network must not exceed the number of measurement gaps supported by the UE, indicated by the capability parallelMeasurementGap.

During the per-UE measurement gaps the UE:

-	is not required to conduct reception/transmission from/to the PCell except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to [7].

UEs shall support the measurement gap patterns listed in table 9.1X.2-1 based on the the applicability specified in table 9.1X.2-3. UE determines measurement gap timing based on gap offset configuration and measurement gap timing advance configuration provided by higher layer signalling as specified in TS 38.331 [2] and TS 36.331 [16].

Table 9.1X.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

(a)	Measurement gap with MGL = N(ms) with MG timing advance of 0 ms for NR standalone operation configured with only single carrier

(b)	Measurement gap with MGL = N(ms) with MG timing advance of 0.5 ms for NR standalone operation configured with only single carrier

Figure 9.1X.2-1: Measurement GAP and total interruption time for NR standalone operation confgigured with only single carrier

In determining the measurement gap starting point, UE shall use the DL timing of the latest NR subframe occurring immediately before the configured measurement gap. If per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap. TMG is the MG timing advance value provided in mgta according to [2].

Table 9.1X.2-2: Total number of interrupted slots on PCell in NR standalone operation configured with only single carrier in FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0 ms is applied |  |  |  |  | When MG timing advance of 0.5 ms is applied |  |  |  |  |
| (kHz) | MGL=20 ms | MGL=10 ms | MGL=6 ms | MGL=4 ms | MGL=3 ms | MGL=20 ms | MGL=10 ms | MGL=6 ms | MGL=4 ms | MGL=3 ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21Note3 | 11Note3 | 7Note3 | 5Note3 | 4Note3 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| NOTE 1: Void NOTE 2: Non-overlapped half-slots occur before and after the measurement gap. Whether a UE can receive and/or transmit in those half-slots is up to UE implementation. |  |  |  |  |  |  |  |  |  |  |

Table 9.1X.2-3: Applicability for Gap Pattern Configurations supported by the UE with NR standalone operation with only single carrier in FR1

| Measurement gap pattern configuration | Serving cell | Measurement Purpose NOTE 2 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement | FR1 | FR1 | 0-11, 24, 25 NOTE 1 |
| NOTE 1: Measurement gap patterns #24 and #25 can be requested [2] only when the UE is configured with UE Rx-Tx measurements requiring such gaps and can only be used during the corresponding positioning measurement period. NOTE 2: Inclusion of positioning measurements for measurement gaps: Measurement purpose which includes FR1 measurements includes also UE Rx-Tx measurements. |  |  |  |

UL slots that are fully or partially overlapping with measurement gap, taking into account TA as defined in clause [7.1C.2], are interrupted.

NOTE: 	Network is supposed to take into account the possible difference between the estimated TA at network and actual TA at UE when scheduling UE in the above slot(s).

### 9.1X.8	Concurrent measurement gaps for RedCap with SAN

#### 9.1X.8.1	Introduction

When UE supports concurrent measurement gap pattern capability, network can provide multiple measurement gaps configured by RRC message(s) as specified in TS 38.331 [2].

#### 9.1X.8.2	Requirements

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells, and the UE supports concurrent measurement gap patterns (parallelMeasurementGap), in order for the requirements in the following clauses to apply the network can provide at most two per-UE measurement gap patterns for monitoring of all frequency layers.

For UE configured with the SA operation, when monitoring of multiple intra-frequency and inter-frequency NR carrier frequency layers as configured by PCell using gaps, each monitored carrier frequency layer, including following measurement types:

-	a measurement object with SSB based measurement,

can be associated to one or two measurement gap pattern(s) provided the network configures the concurrent measurement gap patterns.

When UE supports concurrent measurement gap patterns, each measurement gap pattern supported by the UE is listed in table 9.1X.2-1.

The requirements in clause 9.1X.2 are also applicable for the UE capable of and configured with multiple concurrent measurement gap patterns within each measurement gap pattern.

When UE supports concurrent measurement gap patterns and configured with more than 1 per-UE measurement gap patterns, requirements do not apply if the UE is configured with more than one measurement gap pattern (MGP) with measurement gap repetition period (MGRP) of 20 ms.

#### 9.1X.8.3	Collision between concurrent measurement gaps

Collisions between occasions of two concurrent measurement gaps may occur as specified in this clause if the two measurement gaps are

-	two per-UE measurement gaps.

When UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

-	the two occasions are fully or partially overlapping in time domain, or

-	the distance between the two occasions is equal to or smaller than 5 ms.

The distance between two measurement gap occasions is defined as the time difference between the ending point of the first occasion and the starting point of the second occasion, where the first measurement gap occasion occurs earlier in time than the second measurement gap occasion.

If two colliding measurement gaps are not fully overlapping, in case of collision between two measurement gap occasions, the UE shall perform measurements in the occasion of the measurement gap with higher priority, and the occasion of the measurement gap with lower priority shall be dropped. The UE shall be able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots that are not interrupted according to requirements in clause 9.1X.8.4.

If two colliding measurement gaps are fully overlapping with MGRP=160 ms, in case of collision between two measurement gap occasions, the UE shall perform measurements in the occasion of one of the measurement gaps, and it is up to UE implementation which occasion is used for measurement, provided that UE can meet the requirements in clause 9.2X and 9.3C.

If two measurement gaps are fully overlapping with MGRP<160 ms, no measurement requirements apply.

#### 9.1X.8.4	Measurement gap related requirements of concurrent measurement gaps

A slot is considered as interrupted if it is interrupted by an occasion of any of the configured concurrent measurement gaps following the measurement gap interruption requirements in clause 9.1X.2, except for a dropped measurement gap occasion.

### 9.1X.9	Collision between SMTC and measurement gap for RedCap with SAN

#### 9.1X.9.1	Introduction

This clause contains definition of collision between SMTCs and measurement gap for SAN.

#### 9.1X.9.2	Collision between SMTCs and measurement gap

Collision between occasions of SMTC and measurement gap may occur as specified in this clause.

When UE is configured with one or more than one measurement gap pattern, an SMTC occasion outside measurement gap and a measurement gap occasion are considered colliding if at least one of the following conditions is met:

-	the SMTC occasion is fully or partially overlapping in time domain with the measurement gap occasion, or

-	the magnitude of the distance between the SMTC occasion and the measurement gap occasion in time domain is less than or equals to 5 ms.

The distance between a SMTC occasion and a measurement gap occasion is defined as:

-	the time difference between the ending point of the SMTC occasion and the starting point of the measurement gap occasion, where the SMTC occasion occurs earlier in time than the MG occasion, or

-	the time difference between the ending point of the measurement gap occasion and the starting point of the SMTC occasion, where the measurement gap occasion occurs earlier in time than the SMTC occasion.

#### 9.1X.9.3	Collision between multiple SMTCs on a SAN carrier

Collision between occasions of multiple SMTCs may occur as specified in this clause.

When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met:

-	the two SMTC occasions are fully or partially overlapping in time domain, or

-	the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 5 ms.

The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion.

# End of Change 1

# Start of Change 2

## 9.2X	NR intra-frequency measurements for RedCap with SAN

### 9.2X.1	Introduction

The requirements in clause 9.2X apply for intra-frequency measurements on an SAN carrier frequency. The requirements apply provided that the valid parameters of ephemeris information, epoch time of the ephemeris, common TA, validity timer information, downlink polarization information for target NR SAN cell are send to UE.

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE can perform intra-frequency SSB based measurements without measurement gaps if

-	the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or

-	the SSB is completely contained in the active BWP of the UE, or

-	the active downlink BWP is initial BWP[3].

For intra-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.2X.5.3.

SSB based measurements are configured along with one or more measurement timing configuration(s) (SMTC(s)) which provides periodicity, duration and offset information on a window of up to 5 ms where the measurements are to be performed. For intra-frequency connected mode measurements,

-	when SSB-MTC4List-r17 is not configured, up to two measurement window periodicities may be configured with SSB-MTC and SSB-MTC2

-	when SSB-MTC4List-r17 is configured, multiple measurement window offsets may be configured with SSB-MTC and SSB-MTC4List-r17, and the requirements in clause 9.2X apply provided that the total number of measurement window offsets does not exceed the UE capability parallelSMTC-r17

When measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5 ms for frequency range FR1.

The intra-frequency measurement requirements in clause 9.2X.5 applies for the following scenarios:

-	SSB based intra-frequency measurements with no measurement gap,

-	for a UE supporting concurrent gaps and when concurrent gaps are configured:

-	When none of the SMTC occasions of this intra-frequency measurement object are overlapped by the union of concurrent measurement gaps.

-	When part of the SMTC occasions of this intra-frequency measurement object are overlapped by the union of concurrent measurement gaps.

-	otherwise, for a UE not supporting concurrent gaps or if concurrent gaps are not configured:

-	When none of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	When part of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

The intra-frequency measurement requirements in clause 9.2X.6 applies for the following scenarios:

-	SSB based intra-frequency measurements with measurement gap,

-	SSB based intra-frequency measurements with no measurement gap,

-	for a UE supporting concurrent gaps and when concurrent gaps are configured:

-	when all of the SMTC occasions of this intra-frequency measurement object are overlapped with the associated measurement gap in the concurrent measurement gaps, or

-	when part of the SMTC occasions of this intra-frequency measurement object are overlapped with the associated measurement gap and all the SMTC occasions of this intra-frequency measurement object are overlapped with the union of concurrent measurement gaps.

-	otherwise, for a UE not supporting concurrent gaps or if concurrent gaps are not configured:

-	when all of the SMTC occasions of this intra-frequency measurement object are overlapped with the measurement gap.

### 9.2X.2	Requirements applicability

The requirements in clause 9.2X apply, provided:

-	The cell being identified or measured is detectable.

-	Valid information for the satellite serving the target cell has been provided

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2Rx RedCap with NTN:

-	SS-RSRP related side conditions given in clauses 10.1.2C for FR1 and 10.1.3C for FR2-NTN, for a corresponding band,

-	SS-RSRQ related side conditions given in clauses 10.1.7C for FR1 and 10.1.8C for FR2-NTN, for a corresponding band,

-	SS-SINR related side conditions given in clauses 10.1.12C for FR1 and 10.1.13C for FR2-NTN, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.17 for a corresponding band.

-	For 1Rx RedCap with NTN:

-	SS-RSRP related side conditions given in clauses 10.1A.2 for FR1 for a corresponding band,

-	SS-RSRQ related side conditions given in clauses 10.1A.6 for FR1 for a corresponding band,

-	SS-SINR related side conditions given in clauses 10.1A.10 for FR1 for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.15 for a corresponding band.

### 9.2X.3	Number of cells and number of SSB

#### 9.2X.3.1	Requirements for FR1

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources.

-	4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

-	In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

### 9.2X.4	Measurement Reporting Requirements

#### 9.2X.4.1	Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses [10.1A.2.1] (SS-RSRP for FR1), [10.1A.6.1] (SS-RSRQ for FR1) and [10.1A.10.1] (SS-SINR for FR1).

#### 9.2X.4.2	Event-triggered Periodic Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses [10.1A.2.1] (SS-RSRP for FR1), [10.1A.6.1] (SS-RSRQ for FR1) and [10.1A.10.1] (SS-SINR for FR1).

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2X.4.3.

#### 9.2X.4.3	Event Triggered Reporting

Reported SS-RSRP, SS-RSRQ, and SS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses [10.1A.2.1] (SS-RSRP for FR1), [10.1A.6.1] (SS-RSRQ for FR1) and [10.1A.10.1] (SS-SINR for FR1).

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or Tidentify intra without index defined in clause 9.2X.5.1 or clause 9.2X.6.2. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2X.5.1 or clause 9.2X.6.2. When L3 filtering is used, an additional delay can be expected.

### 9.2X.5	Intra-frequency measurements without measurement gaps

#### 9.2X.5.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2X.5.1-1, 9.2X.5.1-2.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2X.5.1-3, 9.2X.5.1-4.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2X.5.2-1, 9.2X.5.2-2.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap_RedCap,i in clause 9.1A.5.1 for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps, or according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP_max is the maximum MGRP across all configured per-UE measurement gap. Otherwise, MGRP_max is the MGRP of configured measurement gap.

-	Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions within the window W that do not collide with any non-dropped MG occasion within or outside the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in clause 9.1C.8.3. The collision rule between SMTC occasion and measurement gap occasion is defined in clause 9.1C.9.1

Kp = [1] when Navailable_SAN = 0 and measurement gap sharing in clause 9.1.2.1a shall apply.

Kp = 1 when intra-frequency SMTC is fully non overlapping with measurement gaps.

For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Klayer1_measurement: it is scaling factor for sharing between L3 and L1 measurement, and Klayer1_measurement =1, if GEO satellites are measured on the carrier, or if LEO satellites are measured on the carrier and UE supports parallelMeasurementWithoutRestriction-r17, otherwise

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement =1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index

Table 9.2X.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) for 2Rx RedCap UE with NTN

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2X.5.1-2: Time period for PSS/SSS detection, (Frequency range FR1) for 1Rx RedCap UE with NTN

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2X.5.1-3: Time period for time index detection (FR1) for 2Rx RedCap UE with NTN

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2X.5.1-4: Time period for time index detection (FR1) for 1Rx RedCap UE with NTN

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(120 ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | Ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

The UE is allowed to skip measurements on intra-frequency cells, in the interval between t-serviceStart and the satellite switch completion, when it is performing soft satellite switching with resynchronization. In this case, for the measurement initiated but not completed before the beginning interval, the total time to detection can be longer.

The requirements in clause 9.2X.5.1 and 9.2X.5.2 are not applicable when the overall overhead ratio due to scheduling restriction caused by all configured SMTCs (i.e. scheduling restriction overhead of all SMTCs in one SMTC periodicity), is larger than 75%.

#### 9.2X.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2X.5.2-1, 9.2X.5.2-2.

If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra

Table 9.2X.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) for 2Rx RedCap UE with NTN

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2X.5.2-2: Measurement period for intra-frequency measurements without gaps (FR1) for 1Rx RedCap UE with NTN

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, ceil( [5] x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x [5] x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil([5] x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

#### 9.2X.5.3	Scheduling availability of UE during intra-frequency measurements

When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included. For UL, the scheduling restriction applies to UL symbols that fully or partially overlap with the restricted symbols as defined below.

##### 9.2X.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If deriveSSB-IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured(in TS 38.331 [2]), the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If deriveSSB-IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

The UE is expected to receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and/or the corresponding PDSCH, on SSB symbols to be measured. if the following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

##### 9.2X.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO

For UE which does not support the capability parallelMeasurementWithoutRestriction-r17 the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

-	If  deriveSSB-IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured (in TS 38.331 [2]), the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If deriveSSB-IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If the following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

-	The UE shall receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and/or the corresponding PDSCH, on SSB symbols to be measured.

##### 9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1

When the UE performs intra-frequency measurements in a HD-FDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a HD-FDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

### 9.2X.6	Intra-frequency measurements with measurement gaps

#### 9.2X.6.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + TSSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2X.6.2-1, 9.2X.6.2-2.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2X.6.2-3, 9.2X.6.2-4.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2X.6.3-1, 9.2X.6.3-2.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. When the UE is configured with concurrent measurement gaps and the two measurement gaps are fully overlapping with MGRP=160ms, Kgap = 2. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in clause 9.1.8.3.

Kgap is only applicable for UE supporting parallelMeasurementGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0, or if one SMTC overlaps more than one MGs associated to the frequency layer.

CSSFintra_RedCap: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_RedCap,i in clause 9.1A.5.2 for measurement conducted within measurement gaps.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and

if SMTCs within a measurement gap do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

if SMTCs within a measurement gap partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

-	NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap is the number of SMTCs that partially overlap with each other.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index or Tidentify_intra_with_index.

Table 9.2X.6.2-1: Time period for PSS/SSS detection (FR1) for 2Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x 5) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2X.6.2-1: Time period for PSS/SSS detection (FR1) for 1Rx RedCap UE

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_RedCap |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320 ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_RedCap |

Table 9.2X.6.2-3: Time period for time index detection (FR1) for 2Rx RedCap UE

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120 ms, 3 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil(1.5 x 3) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 3 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

Table 9.2X.6.2-4: Time period for time index detection (FR1) for 1Rx RedCap UE

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(160 ms, 6 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(160 ms, ceil(1.5 x 6) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320 ms | 6 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

#### 9.2X.6.3	Intrafrequency Measurement Period

The measurement period for FR1 intrafrequency measurements with gaps is as shown in table 9.2X.6.3-1, 9.2X.6.3-2.

Table 9.2X.6.3-1: Measurement period for intra-frequency measurements with gaps (FR1) for 2Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, 5 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5) x Kmulti_SMTC x Kgap x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 5 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

Table 9.2X.6.3-2: Measurement period for intra-frequency measurements with gaps (FR1) for 1Rx RedCap UE

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400 ms, [5] x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(400 ms, ceil(1.5x [5]) x Kmulti_SMTC x Kgap x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | [5] x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

The UE is allowed to skip measurements on intra-frequency cells, in the interval between t-serviceStart and the satellite switch completion, when it is performing soft satellite switching with resynchronization. In this case, for the measurement initiated but not completed before the beginning of this interval, the total time to measure can be longer.

# End of Change 2