3GPP TSG-RAN WG4 Meeting #115	R4-2506360

St Julian’s, Malta, 19th – 23rd May 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	7.18.3.2

Document for:	Discussion

# Introduction

As per WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, In last meeting 3GPP TSG RAN WG4 Meeting #114, the outcome of the debate, encompassing consensuses and unresolved issues, were documented in WF [2]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Topic #2-2: L3 measurement delay reduction by optimizing CSSF for CSSF outside gap in CA/DC

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618.

- Proposal 1 (OPPO): For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

First of all, the original reference in the existing specification is captured in the below.

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

In the beginning, the issue and solution aimed at the serving cell in discussions, then it was subsequently agreed that the solution 1 can be applied to the neighbour cell measurements as well. The reasoning is rational.

From the perspective of the serving cell measurement, the wordings ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’ shall not be removed. On other hand, in the context of neighbor cell measurements, if the UE is not configured with MO and servingcellMO on a SCC, then there exists no need for the UE to measure the SCC for the neighbour cell measurement purpose.

Given that, in the case that there are no SCCs on which UE is configured to report SSB based measurements in band, the network indication remains applicable for the serving cell.

Observation 1: If there are no SCCs on which UE is configured to report SSB based measurements in band, the network indication remains applicable for the serving cell.

Proposal 1: Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’.

Issue 1-2-2: Expand solution 1 to FR1 case

[Way forward]: - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

In principle, the network can assess the coherent relationship of two FR1 carriers to evaluate the feasibility of applying solution 1. The contiguous CA configuration here in the issue limiting the applicability is intended to enhance the likelihood of coherence between the carriers.

It seems appear that it may work effectively for the measurements on the 2 FR1 serving cells. That said, its effectiveness for neighboring cells remains uncertain and potentially limited a bit, since there are no such assumptions, e.g., neighbor cells on the carriers with intra-band contiguous CA configuration are always co-located/coherent.

Observation 2: Solution 1 for FR1 intra-band contiguous CA may work for the serving cell measurements, it appears to have limited effect for the neighbor cell measurements.

Proposal 2: Expand the solution 1 to at least FR1 intra-band contiguous CA, but limit the applicability to the serving cell measurements.

Issue 1-3-2: detailed design for solution 3 (by-default option 3 vs. need network indication for specific SCC)

[Way forward]: - FFS: - Option 3 in last meeting (WF R4-2502592) is used for CSSF enhancement solution 3. Not to introduce network signaling to indicate a specific SCC to use for the 3rd searcher of CSSF. - Note: 3-searchers will be used by UE if UE report this capability. - FFS: Other restriction (triggered by Huawei): - Option 1: NW can configure per-FR measurement gaps to UE when UE supports per-FR measurement gaps and new 3-searcehr capability simultaneously. But the SMTC occasions of outside gap measurement object in one FR should not be overlapped by per-FR measurement gaps in another FR. (HW, MTK, VIVO, Apple) - Option 2: When UE supports per-FR measurement gaps and 3-searcher capability simultaneously, no new requirements. (MTK, ZTE, Apple) - Option 3: UE is able to support per-FR measurement gaps and new 3-searceher capability simultaneously. UE will use legacy requirement (for example 2-searcher requirement) when all SMTC occasions of outside gap measurement object in one FR are overlapped by per-FR measurement GAP in another FR. (HW, MTK, Apple, VIVO, Ericssion, ZTE, CATT) - Option 3a: The new 3-searcher requirements are not applicable (UE will follow legacy 2-searcehr requirements at this moment) when UE supports per-FR measurement gaps and new 3-searceher capability simultaneously and all SMTC occasions of outside gap measurement object in one FR are overlapped with per-FR measurement GAP in another FR. (HW, OPPO)

Although the solution applying the 3rd searcher in the same manner as the 2nd searcher for the resource allocation can effectively reduce the ‘total’ measurement delay of all SCells, motivated by operators’ network configurations and scheduling plans, we expect at least one specific serving cell can be measured and reported more promptly without waiting for the completion of the ‘total’ measurement.

To address this, the network is expected to be able to indicate a intended serving cell. To simplify the CSSF management from UE implementation perspective, we can compromise that only one intended serving cell is able to be served by the 3rd searcher as per indication by NW.

Observation 3: CSSF solution shall aim to practical multiple bands scenario in real field.

Proposal 3: NW is able to indicate the single intended serving cell (FR1) to be exclusively served by the third searcher.

- In this case, the first and the second searcher manage other serving cells in the conventional manner.

- If no NW indication, the third searcher resource sharing is same as the second searcher sharing.

The above may be only applied in FR1 only configuration.

Regarding the per-FR measurement gap, it’s unnecessary to manage SMTC outside gap taking per-FR measurement gap and new 3-searceher capability into account if they have limitations. The existing measurement delay formula is already sufficiently complex, and adding further constraints would increase implementation burden without clear benefit. Meanwhile, NW configurations shall not be limited to preserve flexibility.

Proposal 4: Introduce a UE capability if necessary for managing the applicability of the CSSF solution,

- For UE capable of operating per-FR measurement gap and 3-searcher independently, new requirements are applied.

- For UE not capable of operating per-FR measurement gap and 3-searcher independently, no new requirements are applied.

A leftover of the agreed solution 3 is, if the UE supports capability and applies solution 3 in some particular cases, e.g., DC configuration including 1 PCC and 1 PSCC or CA configuration including 1 PCC and 1 SCC, in such cases, the 2nd and 3rd searcher both will be assigned to the sole PSCC or SCC. As a consequence, the measurement delay on the PSCC or SCC is effectively halved compared to that on the PCC, which is unnecessary and illogical.

Proposal 5: Applicability of the solution 3 shall exclude CA/DC configurations consisting of totally 2 CCs.

# Conclusion

Observation 1: If there are no SCCs on which UE is configured to report SSB based measurements in band, the network indication remains applicable for the serving cell.

Observation 2: Solution 1 for FR1 intra-band contiguous CA may work for the serving cell measurements, it appears to have limited effect for the neighbor cell measurements.

Observation 3: CSSF solution shall aim to practical multiple bands scenario in real field.

Proposal 1: Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’.

Proposal 2: Expand the solution 1 to at least FR1 intra-band contiguous CA, but limit the applicability to the serving cell measurements.

Proposal 3: NW is able to indicate the single intended serving cell (FR1) to be exclusively served by the third searcher.

- In this case, the first and the second searcher manage other serving cells in the conventional manner.

- If no NW indication, the third searcher resource sharing is same as the second searcher sharing.

The above may be only applied in FR1 only configuration.

Proposal 4: Introduce a UE capability if necessary for managing the applicability of the CSSF solution,

- For UE capable of operating per-FR measurement gap and 3-searcher independently, new requirements are applied.

- For UE not capable of operating per-FR measurement gap and 3-searcher independently, no new requirements are applied.

Proposal 5: Applicability of the solution 3 shall exclude CA/DC configurations consisting of totally 2 CCs.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2502592, WF for NR_RRM_Ph5, Apple