3GPP TSG-RAN WG4 Meeting # 115                                 R4-2506371

Malta, MT, 19th –23rd May, 2025

Agenda item:	7.28.4

Source: 	CMCC

Title: 	Discussion on LP-WUS BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#114, the LP-WUS BS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about LP-WUS BS RF requirements.

- Discussion

2.1 Core requirement for LP-WUS power boosting

Issue 1-1: How to specify the BS manufacturer declaration for LP-WUS - WF - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth. - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB. - X can be different for different channel bandwidth. - FFS on the value of X and use 3 as starting point.

From our perspective, it is challenging to establish a minimum requirement specific to the network. Given that different operators employ this technology across a range of scenarios, setting a uniform minimum requirement is complex. The same applies to the declaration tables of BS manufacturer. However, we believe that different BS CBW scenarios can be considered. Under the current definition of power boosting requirements, NR signals with smaller bandwidths are more significantly impacted by the same power boosting level.

In the discussion of the value of X, we will start from the perspective of protecting NR signals. Considering the boosting level has a greater impact on smaller bandwidths, we propose that for BS CBW with a bandwidth less than 50 MHz, the value of X should not be limited. For bandwidths equal or greater than 50 MHz, X can be set to 3.

Proposal 1: Leave the minimum power boosting level to manufacturer declaration.

Proposal 2: The minimum power boosting level could be different for different BS CBW.

- For channel bandwidth less than 50 MHz, the value of X should not be limited.

- For channel bandwidths equal or greater than 50 MHz, X can be set to 3.

# 3. Conclusions

Proposal 1: Leave the minimum power boosting level to manufacturer declaration.

Proposal 2: The minimum power boosting level could be different for different BS CBW.

- For channel bandwidth less than 50 MHz, the value of X should not be limited.

- For channel bandwidths equal or greater than 50 MHz, X can be set to 3.

# 4. Reference

- R4-2504744, WF on NR LP-WUS BS RF