3GPP TSG-RAN WG4 Meeting # 115                                    R4-2506374

Malta, MT, 19th –23rd May, 2025

Agenda item:	7.25.3.3

Source: 	CMCC

Title: 	Discussion on existing Rx requirements for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [1]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 ACS requirements

Following capture the agreements from TR 38.858 for ACS requirements:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Last meeting, we have the following requirements.

Issue 2-1: ACS requirement - FFS on this issue - Option 1: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD. - Option 2: Prioritize discussing the scenarios under non-colocation case first. - The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked. - Option 3: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level. - Option 4: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD. - Derive ACS requirement level based on the simulation results using 5% GS in SI. - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Similar to ACLR requirements, adjacent-channel co-existence simulation only focuses on not less than 10% grid shift values, and 0% grid shift is avoided. Considering it is hard to define the co-location ACS requirement, it is better to prioritize discussing the scenarios under non-colocation case first.

The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

2.2 In-band blocking

Last meeting we have approved the WF below:

WF: - FFS on this issue - Option 1: Define the in-band blocking requirement with the same grid-shift [TBD] for both SBFD-TDD adjacent operator scenario and SBFD-SBFD adjacent operator scenario. Additional emission requirement is included in this release - Option 2: Define the in-band blocking requirement with grid-shift 20-50% for SBFD-TDD adjacent operator scenario, while additional emission requirement is not considered in Rel-19. If a decision is made to require SBFD on adjacent operators in a band, a future WI will address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario

After the discussion in the last meeting, we have currently narrowed down the options to two. The main point of contention is whether to define additional coexistence metrics under the SBFD-SBFD scenario in the current Release. From the perspective of operators deploying SBFD, the definition of requirements for this scenario is necessary. However, considering the remaining time, it is clearly unreasonable to define blocking requirements at this stage based on some extreme grid shift assumptions, such as a 5% grid shift. Therefore, as a compromise, we can consider clarifying in the current release which requirement need to be defined for the coexistence of SBFD-SBFD, such as additional emission requirements, to facilitate the definition of requirements in subsequent release. At the same time, considering the actual deployment needs, we can first conduct RF requirements analysis based on 20% grid shift.

Proposal 3: As a compromise, we can consider which requirement need to be defined in subsequent release for the coexistence of SBFD-SBFD in R19.

Proposal 4: The results obtained with the parameters of [20%] grid shift can serve as references.

2.3 Receiver intermodulation

- Issue 2-5: Necessity of New RX intermodulation requirement with 1 interfering signal - FFS on this issue - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement - Option 2: RAN4 to consider the new RX IMD scenario with self-interference and 1 interfering signal, and to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies - Issue 2-6: Impact on existing RX intermodulation requirement with 2 interfering signals - FFS on this issue - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed - Option 3: RAN4 to consider new RX intermodulation requirement with 2 interfering signals. RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB

The introduction of the two new RX intermodulation requirements is primarily driven by the need to address BS2BS cross-link interference. For the general in-band blocking scenario has already accounted for the new Rx cross-link interference. So there is no need to define RX intermodulation requirement with 1 interfering signal. For the scenario for 2 input signals, it could be regarded as the worst case when setting requirement on receiver linearity. It is better to defined this requirement.

Proposal 5: there is no need to define new RX intermodulation requirement with 1 interfering signal.

Proposal 6: It is better to define new RX intermodulation requirement with 2 interfering signals.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Prioritize discussing the scenarios under non-colocation case first.

Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked.

Proposal 3: As a compromise, we can consider which requirement need to be defined in subsequent release for the coexistence of SBFD-SBFD in R19.

Proposal 4: The results obtained with the parameters of [20%] grid shift can serve as references.

Proposal 5: there is no need to define new RX intermodulation requirement with 1 interfering signal.

Proposal 6: It is better to define new RX intermodulation requirement with 2 interfering signals.

# 4. Reference

[1] R4-2504752, Way Forward for [114bis][307] NR_duplex_evo_BSRF