3GPP TSG-RAN4 Meeting  #115	R4-2506376

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.5.4

Source:	Spreadtrum, UNISOC

Title:	TP to TR 38.755 on UE RF requirements evaluation for fragmented carriers

Document for:	Approval

1	Introduction

This contribution provides text proposals to the TR [1] to capture as follow:

- The analysis of UE RF requirement impact for fragmented carriers from our contribution in [2], where the impact of Tx leakage is analyzed.

2	Text Proposal

<<Start of Change 1 for TR 38.755>>

7    Study on RF requirements for the inter-operator co-located BS scenario

7.X	RF requirements evaluation from company C

With common RF chain architecture, the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, both the PCC and SCC performance may take some hit.

We have provided analysis on the impact of tx leakage, i.e., the NF increase due to AGC adjustment (i.e., RX gain backoff). From table 1 to table 5 to show our analysis for CA configuration CA_n3(2A) (5MHz PCC+ 5MHz SCC, Wgap = 65MHz), CA_n25(2A) (5MHz PCC + 5MHz SCC, Wgap = 55MHz), CA_n2 (2A (5MHz PCC+5MHz SCC, Wgap=50MHz), n7 (2A (10MHz PCC+5MHz SCC, Wgap=55MHz) and n26 (2A (15MHz+10MHz, Wgap=10MHz) respectively.

Table 1: Link budge of self-interference for CA_3(2A)(5MHz+5MHz,Wgap=65MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -97 | -93.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -101 | -97.3 | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - |
| LPF output (dBm) | -36.4 | -102 | -98.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -94 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.6 | - |
| Noise floor (including ADC noise) | - | - | - | -83.4 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -88 | - | - |
| SNR after ADC (dB) | - | - | - | - | -10(PCC) |
|  |  |  |  |  | -10.7(SCC) |

Table 2: Link budge of self-interference for CA_25(2A)(5MHz+5MHz,Wgap=55MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -110.5 | -95.5 | - | - |
| LPF attenuation | 6 | 1 | 1 | - | - |
| LPF output (dBm) | -29 | -111.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -93.5 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.6 | - |
| Noise floor (including ADC noise) | - | - | - | -82.9 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -85 | - | - |
| SNR after ADC (dB) | - | - | - | - | -10.9 (PCC) |
|  |  |  |  |  | -11.5(SCC) |

Table 3: Link budge of self-interference for CA_2(2A)(5MHz+5MHz,Wgap=50MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -98 | -93 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -112 | -97 | - | - |
| LPF attenuation | 8 | 1 | 1 | - | - |
| LPF output (dBm) | -31 | -113 | -98 | - | - |
| Noise floor for basic REFSENS | - | - | - | -95 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.6 | - |
| Noise floor (including ADC noise) | - | - | - | -84.4 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -87 | - | - |
| SNR after ADC (dB) | - | - | - | - | -10.6 (PCC) |
|  |  |  |  |  | -11.1(SCC) |

Table 4: Link budge of self-interference for CA_7(2A)(10MHz+5MHz,Wgap=55MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -98 | -93 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -112 | -97 | - | - |
| LPF attenuation | 8 | 1 | 1 | - | - |
| LPF output (dBm) | -31 | -113 | -98 | - | - |
| Noise floor for basic REFSENS | - | - | - | -95 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 8.6 | - |
| Noise floor (including ADC noise) | - | - | - | -86.4 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -85.8 | - | - |
| SNR after ADC (dB) | - | - | - | - | -7.6 (PCC) |
|  |  |  |  |  | -7.7(SCC) |

Table 5: Link budge of self-interference for CA_26(2A)(15MHz+10MHz,Wgap=10MHz)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -92.7 | -69.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -106.7 | -73.3 | - | - |
| LPF attenuation | 11.5 | 1 | 1 | - | - |
| LPF output (dBm) | -33.5 | -107.7 | -74.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -89.7 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 8.6 | - |
| Noise floor (including ADC noise) | - | - | - | -81.1 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -59.5 | - | - |
| SNR after ADC (dB) | - | - | - | - | -11.1 (PCC) |
|  |  |  |  |  | -25.7(SCC) |

<<End of Change 1 for TR 38.755>>

4	References

TR 38.755 V0.2.0

R4-2504293, Discussion of impacts on UE RF requirements and DL performance. Spreadtrum, UNISOC