3GPP TSG-RAN4 Meeting # 115	R4-2506383

Malta, May.19th – 23th, 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Some general considerations on R19 HPUE IMD MSD rule

# Agenda item:	7.1.2.1.1

# Document for:	Approval

# 1 Introduction

In last meeting, the WF[1] on HPUE MSD simplification was agreed, in which RAN4 has achieved the agreements on the 1 band 1 UL CC HPUE MSD Simplification, and for 2 band 2UL CC IMD MSD, it is still open.

In this contribution, we give some general considerations on HPUE IMD MSD rule.

# 2	Discussion

- 2.1	 HPUE MSD v.s. PC3 MSD=0

- In the WF, one of an agreement is:

- Agreement 2 on handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD: - For new HPUE band combinations, do not specify the MSD requirements for 1UL HPUE PC21Tx PC22Tx and PC1.5 (2Tx). - For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for 1UL HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if 1UL HPUE MSD requirements are needed.

- For HPUE MSD, it includes:

- 1 band 1 UL CC

- 1 band 2 UL CC

- 2 band 2 UL CC

- 2 band 3 UL CC

- We have gave some backgrounds and reasons for PC3 MSD requirements are not defined in [2]. Actually, in the current TS38.101-1, there are some PC3 band combinations who have IMD problem without MSD requirement defined. For example for CA_n5-n77, for PC3 band combination, there are IMD2/4/5 problems but the IMD2 MSD is defined as N/A, and for PC2 requirement, IMD2 MSD are not included. Although we think there exists inconsistency (maybe it is mistake!) , IMD2 MSD requirements would not be defined for PC2 band combination due to the N/A is defined for PC3.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination |  | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | Duplex mode |  |  |
| PC3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5-n77 |  | n5 | N/A | N/A | N/A | N/A | N/A |  | FDD |  | IMD27 |
|  |  | n7712 | N/A | N/A | N/A | N/A | N/A |  | TDD |  | N/A |
|  |  | n5 | 844 | 5 | 25 | 889 | 8.3 |  | FDD |  | IMD413 |
|  |  | n77 | 3421 | 10 | 50 | 3421 | N/A |  | TDD |  | N/A |
|  |  | n5 | 829 | 5 | 25 | 874 | 5.5 |  | FDD |  | IMD513 |
|  |  | n77 | 4190 | 10 | 50 | 4190 | N/A |  | TDD |  | N/A |
|  |  | n5 | N/A | 5 | N/A | 880 | 8.6 |  | FDD |  | IMD4 |
|  |  | n7712 | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A |  | TDD |  | N/A |
|  |  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 |  |  |  |  |  |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). |  |  |  |  |  |  |  |  |  |  |  |
| PC2 |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5-n77 | n5 |  | 844 | 5 | 25 | 889 | 18.6 | FDD |  | IMD44,13 |  |
|  | n77 |  | 3421 | 10 | 50 | 3421 | N/A | TDD |  | N/A |  |
|  | n5 |  | N/A | 5 | N/A | 880 | 18.5 | FDD |  | IMD4 |  |
|  | n7712 |  | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD |  | N/A |  |
|  |  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 | N/A | TDD |  | N/A |  |

- Therefore, we think the similar approach as the above agreement 2 should be applied for 2 band 2UL CC cases. We have the following proposal:

- Proposal 1. For HPUE IMD MSD, handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD:

- For new HPUE band combinations, do not specify the MSD requirements for HPUE .

- For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if HPUE IMD MSD requirements are needed.

- 2.2	 HPUE MSD v.s. PC3 MSD IMD order

- In TS38.846, the rules for the IMD MSD order can be found in sub-clause 7.4, we copy&paste here for convenience. Note that the rules are applied PC3/HPUE band combination, and there are no further agreements to update this rule from then on.

– For Rel-18 new introduced band combinations, – For type 1 UL configurations (e.g. UL_CA_n41C or CA_n78(2A)) ○ The lowest order IMD is recommended as worst case to represent single band UL transmission with UL configured intra-band CA. ○ If the DL band may be affected by a mix of even and odd order IMD products, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications. ○ ........ – For type 2 UL configurations (e.g. UL_CA_n1A-n3A) ○ If only one IMD order occurs per victim band, the MSD value if any shall be defined in the specifications. ○ If the DL band may be affected by a mix of even and odd order IMD products, then the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications. ○ .....

- The above rules were introduced in Rel-17, it is not only applied for 1 band 2UL CC IMD MSD, but also applied for 2 band 2UL CC and 3 UL CC IMD MSD. Before that, it seems there were no uniform approach for the companies if there are mix of even and odd order IMD products, which make it flexible for proponent in their TP to TR to define the MSD, some of the proponents defined the MSD requirements for all of the IMD orders, and some of the proponents defined the MSD requirements for part of the IMD order with the NOTE saying ‘This band is subject to IMDx also which MSD is not specified’.

- It seems it was already cause inconsistency between PC3 and PC2 band combination (EX. CA_n5-n77 as above).

- For a band combination, when the DL band is affected by a mix of even and odd order IMD products, and when the above rule is not used for the corresponding PC3 band combination (i.e. MSD requirement are defined for all IMD odd orders or even orders) it should be discussed how to handle the IMD MSD for HPUE new band combination.

- For example, the DL band is affected by IMD2 and IMD 4 order, and both IMD2 and IMD4 MSD are defined for the existing PC3 band combination, then for the HPUE new band combination, if LUT approach is introduced for HPUE MSD, only IMD2 MSD is defined (per the above rule) or both IMD2 and IMD4 MSD are defined? For this example, only define IMD2 MSD is our preference.

- Of course, if the IMD requirements for some of the PC3 band combination have already obey the above rule, then  the IMD requirements for the corresponding HPUE should also obey the above rule.

- Proposal 2. If the DL band is affected by a mix of even and odd order IMD products:

- For new HPUE band combinations, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined by using LUT approach.

# 3 Conclusion

In this contribution, we give some general consideration on HPUE IMD MSD rule. The proposals and conclusions are:

- Proposal 1. For HPUE IMD MSD, handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD:

- For new HPUE band combinations, do not specify the MSD requirements for HPUE .

- For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if HPUE IMD MSD requirements are needed.

- Proposal 2. If the DL band is affected by a mix of even and odd order IMD products:

- For new HPUE band combinations, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined by using LUT approach.

# 4	Reference

[1] R4-2505143, WF on HPUE MSD simplification, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE

[2] R4-2503969, Consideration on R19 HPUE MSD rule, ZTE Corporation, Sanechips