| 3GPP TR 38.719-02-01V0.6.0 (2025-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-19 NR inter-band CA/DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

Introduction	15

1	Scope	16

2	References	16

3	Definitions of terms, symbols and abbreviations	16

3.1	Terms	16

3.2	Symbols	16

3.3	Abbreviations	16

4	TR Maintenance	17

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part	17

5.x	CA_nX-nY	17

5.x.1	Common for 1 band UL and 2 bands UL CA	17

5.x.1.1	Operating bands for CA	17

5.x.1.2	Channel bandwidths per operating band for CA	17

5.x.1.3	UE co-existence studies for 1 band UL	18

5.x.1.4	∆TIB,c and ∆RIB,c values	19

5.x.1.5	REFSENS requirements	20

5.x.1.6	OOB blocking exception requirements	20

5.x.2	Specific for 2 bands UL CA	20

5.x.2.1	Maximum output power for inter-band CA	20

5.x.2.2	UE co-existence studies for 2 bands UL	20

5.x.2.3	REFSENS requirements	21

5.1	CA_n3-n104	21

5.1.1	Common for 1 band UL and 2 bands UL CA	21

5.1.1.1	Operating bands for CA	21

5.1.1.2	Channel bandwidths per operating band for CA	22

5.1.1.3	UE co-existence studies for 1 band UL	22

5.1.1.4	∆TIB,c and ∆RIB,c values	24

5.1.1.5	REFSENS requirements	24

5.1.1.6	OOB blocking exception requirements	25

5.1.2	Specific for 2 bands UL CA	25

5.1.2.1	Maximum output power for inter-band CA	25

5.1.2.2	UE co-existence studies for 2 bands UL	25

5.1.2.3	REFSENS requirements	26

5.2	CA_n8-n104	26

5.2.1	Common for 1 band UL and 2 bands UL CA	26

5.2.1.1	Operating bands for CA	26

5.2.1.2	Channel bandwidths per operating band for CA	26

5.2.1.3	UE co-existence studies for 1 band UL	27

5.2.1.4	∆TIB,c and ∆RIB,c values	29

5.2.1.5	REFSENS requirements	29

5.2.1.6	OOB blocking exception requirements	29

5.2.2	Specific for 2 bands UL CA	29

5.2.2.1	Maximum output power for inter-band CA	29

5.2.2.2	UE co-existence studies for 2 bands UL	29

5.2.2.3	REFSENS requirements	30

5.3	CA_n2-n5	31

5.3.1	Common for 1 band UL and 2 bands UL CA	31

5.3.1.1	Operating bands for CA	31

5.3.1.2	Channel bandwidths per operating band for CA	31

5.3.1.3	UE co-existence studies for 1 band UL	31

5.3.1.4	∆TIB,c and ∆RIB,c values	33

5.3.1.5	REFSENS requirements	33

5.3.1.6	OOB blocking exception requirements	33

5.3.2	Specific for 2 bands UL CA	33

5.3.2.1	Maximum output power for inter-band CA	33

5.3.2.2	UE co-existence studies for 2 bands UL	33

5.3.2.3	REFSENS requirements	33

5.4	CA_n2-n48	33

5.4.1	Common for 1 band UL and 2 bands UL CA	33

5.4.1.1	Operating bands for CA	34

5.4.1.2	Channel bandwidths per operating band for CA	34

5.4.1.3	UE co-existence studies for 1 band UL	34

5.4.1.4	∆TIB,c and ∆RIB,c values	35

5.4.1.5	REFSENS requirements	35

5.4.1.6	OOB blocking exception requirements	35

5.4.2	Specific for 2 bands UL CA	36

5.4.2.1	Maximum output power for inter-band CA	36

5.4.2.2	UE co-existence studies for 2 bands UL	36

5.4.2.3	REFSENS requirements	36

5.5	CA_n2-n66	36

5.5.1	Common for 1 band UL and 2 bands UL CA	36

5.5.1.1	Operating bands for CA	36

5.5.1.2	Channel bandwidths per operating band for CA	36

5.5.1.3	UE co-existence studies for 1 band UL	37

5.6	CA_n5-n48	38

5.6.1	Common for 1 band UL and 2 bands UL CA	38

5.6.1.1	Operating bands for CA	38

5.6.1.2	Channel bandwidths per operating band for CA	38

5.6.1.3	UE co-existence studies for 1 band UL	39

5.6.1.4	∆TIB,c and ∆RIB,c values	42

5.6.1.5	REFSENS requirements	42

5.6.1.6	OOB blocking exception requirements	42

5.6.2	Specific for 2 bands UL CA	42

5.6.2.1	Maximum output power for inter-band CA	42

5.6.2.2	UE co-existence studies for 2 bands UL	42

5.6.2.3	REFSENS requirements	43

5.7	CA_n48-n66	43

5.7.1	Common for 1 band UL and 2 bands UL CA	43

5.7.1.1	Operating bands for CA	43

5.7.1.2	Channel bandwidths per operating band for CA	43

5.7.1.3	UE co-existence studies for 1 band UL	44

5.7.1.4	∆TIB,c and ∆RIB,c values	45

5.7.1.5	REFSENS requirements	45

5.7.1.6	OOB blocking exception requirements	45

5.7.2	Specific for 2 bands UL CA	45

5.7.2.1	Maximum output power for inter-band CA	45

5.7.2.2	UE co-existence studies for 2 bands UL	45

5.7.2.3	REFSENS requirements	46

5.8	CA_n26-n78	46

5.8.1	Specific for 1 bands UL CA	46

5.8.1.1	Operating bands for CA	47

5.8.1.2	Channel bandwidths per operating band for CA	47

5.8.1.3	Co-existence studies	47

5.8.1.4	∆TIB and ∆RIB values	47

5.8.1.5	REFSENs requirements	47

5.9	CA_n1-n71	47

5.9.1	Common for 1 band UL and 2 bands UL CA	48

5.9.1.1	Operating bands for CA	48

5.9.1.2	Channel bandwidths per operating band for CA	48

5.9.1.3	UE co-existence studies	48

5.9.1.4	∆TIB,c and ∆RIB,c values	49

5.9.1.5	REFSENS requirements	49

5.9.1.6	OOB blocking exception requirements	50

5.9.2	Specific for 2 bands UL CA	50

5.9.2.1	Maximum output power for inter-band CA	50

5.9.2.2	UE co-existence studies	50

5.9.2.3	REFSENS requirements	51

5.10	CA_n3-n71	51

5.10.1	Common for 1 band UL and 2 bands UL CA	51

5.10.1.1	Operating bands for CA	51

5.10.1.2	Channel bandwidths per operating band for CA	51

5.10.1.3	UE co-existence studies	52

5.10.1.4	∆TIB,c and ∆RIB,c values	53

5.10.1.5	REFSENS requirements	53

5.10.1.6	OOB blocking exception requirements	53

5.10.2	Specific for 2 bands UL CA	53

5.10.2.1	Maximum output power for inter-band CA	53

5.10.2.2	UE co-existence studies	53

5.10.2.3	REFSENS requirements	54

5.11	CA_n20-n77	54

5.11.1	Common for 1 band UL and 2 bands UL CA	54

5.11.1.1	Operating bands for CA	54

5.11.1.2	Channel bandwidths per operating band for CA	54

5.11.1.3	UE co-existence studies	55

5.11.1.4	∆TIB,c and ∆RIB,c values	56

5.11.1.5	REFSENS requirements	56

5.11.1.6	OOB blocking exception requirements	57

5.11.2	Specific for 2 bands UL CA	57

5.11.2.1	Maximum output power for inter-band CA	57

5.11.2.2	UE co-existence studies	57

5.11.2.3	REFSENS requirements	58

5.12	CA_n7-n29	58

5.12.1	Common for 1 band UL and 2 bands UL CA	58

5.12.1.1	Operating bands for CA	58

5.12.1.2	Channel bandwidths per operating band for CA	58

5.12.1.3	UE co-existence studies for 1 band UL	58

5.12.1.4	∆TIB,c and ∆RIB,c values	59

5.12.1.5	REFSENS requirements	60

5.12.1.6	OOB blocking exception requirements	60

5.13	CA_n29-n77	60

5.13.1	Common for 1 band UL and 2 bands UL CA	60

5.13.1.1	Operating bands for CA	60

5.13.1.2	Channel bandwidths per operating band for CA	60

5.13.1.3	UE co-existence studies for 1 band UL	60

5.13.1.4	∆TIB,c and ∆RIB,c values	62

5.13.1.5	REFSENS requirements	62

5.13.1.6	OOB blocking exception requirements	63

5.14	CA_n20-n41	63

5.14.1	Common for 1 band UL and 2 bands UL CA	63

5.14.1.1	Operating bands for CA	63

5.14.1.2	Channel bandwidths per operating band for CA	63

5.14.1.3	UE co-existence studies for 1 band UL	63

5.14.1.4	∆TIB,c and ∆RIB,c values	65

5.14.1.5	REFSENS requirements	65

5.14.1.6	OOB blocking exception requirements	65

5.14.2	Specific for 2 bands UL CA	65

5.14.2.1	Maximum output power for inter-band CA	65

5.14.2.2	UE co-existence studies for 2 bands UL	65

5.14.2.3	REFSENS requirements	66

5.15	CA_n20-n71	67

5.15.1	Common for 1 band UL and 2 bands UL CA	67

5.15.1.1	Operating bands for CA	67

5.15.1.2	Channel bandwidths per operating band for CA	67

5.15.1.3	UE co-existence studies for 1 band UL	67

5.15.1.4	∆TIB,c and ∆RIB,c values	68

5.15.1.5	REFSENS requirements	69

5.15.1.6	OOB blocking exception requirements	69

5.15.2	Specific for 2 bands UL CA	69

5.15.2.1	Maximum output power for inter-band CA	69

5.15.2.2	UE co-existence studies for 2 bands UL	69

5.15.2.3	REFSENS requirements	70

5.16	CA_n41-n78	70

5.16.1	Common for 1 band UL and 2 bands UL CA	70

5.16.1.1	Operating bands for CA	70

5.16.1.2	Channel bandwidths per operating band for CA	71

5.16.1.3	UE co-existence studies for 1 band UL	71

5.16.1.4	∆TIB,c and ∆RIB,c values	72

5.16.1.5	REFSENS requirements	72

5.16.1.6	OOB blocking exception requirements	72

5.16.2	Specific for 2 bands UL CA	72

5.16.2.1	UE co-existence studies for 2 bands UL	72

5.16.2.2	REFSENS requirements	73

5.17	CA_n71-n78	73

5.17.1	Common for 1 band UL and 2 bands UL CA	73

5.17.1.1	Operating bands for CA	73

5.17.1.2	Channel bandwidths per operating band for CA	73

5.17.1.3	UE co-existence studies for 1 band UL	73

5.17.1.4	∆TIB,c and ∆RIB,c values	74

5.17.1.5	REFSENS requirements	74

5.17.1.6	OOB blocking exception requirements	74

5.17.2	Specific for 2 bands UL CA	74

5.17.2.1	Maximum output power for inter-band CA	74

5.17.2.2	UE co-existence studies for 2 bands UL	74

5.17.2.3	REFSENS requirements	75

5.18	CA_n41-n104	75

5.18.1	Common for 1 band UL and 2 bands UL CA	75

5.18.1.1	Operating bands for CA	75

5.18.1.2	Channel bandwidths per operating band for CA	75

5.18.1.3	UE co-existence studies for 1 band UL	75

5.18.1.4	∆TIB,c and ∆RIB,c values	78

5.18.1.5	REFSENS requirements	79

5.18.1.6	OOB blocking exception requirements	79

5.18.2	Specific for 2 bands UL CA	79

5.18.2.1	Maximum output power for inter-band CA	79

5.18.2.2	UE co-existence studies for 2 bands UL	79

5.18.2.3	REFSENS requirements	79

5.19	CA_n1-n8	79

5.19.1	Common for 1 band UL and 2 bands UL CA	79

5.19.1.1	Operating bands for CA	79

5.19.1.2	Channel bandwidths per operating band for CA	80

5.19.1.3	UE co-existence studies for 1 band UL	80

5.19.1.4	∆TIB,c and ∆RIB,c values	81

5.19.1.5	REFSENS requirements	81

5.20	CA_n40-n71	81

5.20.1	Common for 1 band UL and 2 bands UL CA	81

5.20.1.1	Operating bands for CA	81

5.20.1.2	Channel bandwidths per operating band for CA	81

5.20.1.3	UE co-existence studies for 1 band UL	81

5.20.1.4	∆TIB,c and ∆RIB,c values	83

5.20.1.5	REFSENS requirements	83

5.20.1.6	OOB blocking exception requirements	83

5.20.2	Specific for 2 bands UL CA	83

5.20.2.1	Maximum output power for inter-band CA	83

5.20.2.2	UE co-existence studies for 2 bands UL	83

5.20.2.3	REFSENS requirements	84

5.21	CA_n40-n41	84

5.21.1	Common for 1 band UL and 2 bands UL CA	84

5.21.1.1	Operating bands for CA	84

5.21.1.2	Channel bandwidths per operating band for CA	85

5.21.1.3	UE Co-existence studies	85

5.21.1.4	∆TIB and ∆RIB values	85

5.21.1.5	REFSENs requirements	86

5.21.1.6	OOB blocking exception requirements	86

5.22	CA_n41-n79	86

5.22.1	Common for 1 band UL and 2 bands UL CA	86

5.22.1.1	Operating bands for CA	86

5.22.1.2	Channel bandwidths per operating band for CA	86

5.22.1.3	UE Co-existence studies	87

5.22.1.4	∆TIB and ∆RIB values	87

5.22.1.5	REFSENs requirements	87

5.22.1.6	OOB blocking exception requirements	88

5.22.2	Specific for 2 bands UL CA	88

5.22.2.1	Maximum output power for inter-band CA	88

5.22.2.2	UE co-existence studies	88

5.22.2.3	REFSENS requirements	88

5.23	CA_n2-n77	88

5.23.1	Common for 1 band UL and 2 bands UL CA	88

5.23.1.1	Operating bands for CA	88

5.23.1.2	Channel bandwidths per operating band for CA	88

5.23.1.3	UE co-existence studies for 1 band UL	89

5.23.1.4	∆TIB,c and ∆RIB,c values	91

5.23.1.5	REFSENS requirements	91

5.23.1.6	OOB blocking exception requirements	91

5.23.2	Specific for 2 bands UL CA	91

5.23.2.1	Maximum output power for inter-band CA	91

5.23.2.2	UE co-existence studies for 2 bands UL	91

5.23.2.3	REFSENS requirements	92

5.24	CA_n5-n77	92

5.24.1	Common for 1 band UL and 2 bands UL CA	92

5.24.1.1	Operating bands for CA	92

5.24.1.2	Channel bandwidths per operating band for CA	92

5.24.1.3	UE co-existence studies for 1 band UL	92

5.24.1.4	∆TIB,c and ∆RIB,c values	94

5.24.1.5	REFSENS requirements	94

5.24.1.6	OOB blocking exception requirements	94

5.24.2	Specific for 2 bands UL CA	95

5.24.2.1	Maximum output power for inter-band CA	95

5.24.2.2	UE co-existence studies for 2 bands UL	95

5.24.2.3	REFSENS requirements	96

5.25	CA_n66-n77	96

5.25.1	Common for 1 band UL and 2 bands UL CA	96

5.25.1.1	Operating bands for CA	96

5.25.1.2	Channel bandwidths per operating band for CA	96

5.25.1.3	UE co-existence studies for 1 band UL	96

5.25.1.4	∆TIB,c and ∆RIB,c values	98

5.25.1.5	REFSENS requirements	98

5.25.1.6	OOB blocking exception requirements	98

5.25.2	Specific for 2 bands UL CA	98

5.25.2.1	Maximum output power for inter-band CA	98

5.25.2.2	UE co-existence studies for 2 bands UL	98

5.25.2.3	REFSENS requirements	99

5.26	CA_n18-n77	100

5.26.1	Common for 1 band UL and 2 bands UL CA	100

5.26.1.1	Operating bands for CA	100

5.26.1.2	Channel bandwidths per operating band for CA	100

5.26.1.3	UE co-existence studies for 1 band UL	100

5.26.1.4	∆TIB,c and ∆RIB,c values	100

5.26.1.5	REFSENS requirements	100

5.26.1.6	OOB blocking exception requirements	101

6	Dual Connectivity: Specific Band Combination Part	101

6.x	DC_nX-nY	101

6.x.1	Configurations for DC_nX-nY	101

6.x.2	Maximum output power for NR-DC	101

Annex A: Band group definition	102

Annex B: Valid UL configurations	102

Annex C:	Void	102

Annex D:	Void	102

Annex E (informative): Change history	102

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

The present document is a technical report for NR inter-band CA and DC for 3 bands DL with 2 bands UL under Rel-19 time frame. The document covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination), including:

1:	Common issues for both 1 band UL and 2 bands UL NR CA, including the impact of UL/DL harmonic/ harmonic mixing associated with REFSEN,  ∆TIB, c and ∆RIB, c, and OOB blocking exception, etc.

2:	bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

It shall be noted that no new issue for inter-band NR DC combination, the corresponding 2 bands UL NR CA specific issues shall be re-used.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 2 bands DL with up to 2 bands UL for the Rel-19 band combinations.

This TR contains the RF requirements of band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

TDD	Time Division Duplex

UL	UpLink

4	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part

5.x	CA_nX-nY

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| NOTE X: Editor's note: Whether or not supporting simultaneous Rx/Tx capability should be identified for TDD-TDD or FDD-TDD band combination |  |  |  |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)?

Editor's note: If yes, please add the NOTE in the above table.

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nXA-nYA | - * or CA_nXA-nYA | nX | Channel BWs | X |
|  |  | nY | Channel BWs |  |
| CA_nX(A/B/C(2A)-nY(A/B/C(2A) | CA_nXB/C/(2A) or CA_nXA/B/C-nYA | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |

Editor's note*: ‘-’ is for 1UL

Editor's note: The table format shall be the same with Table 5.5A.3.1-1 in the latest TS 38.101-1. The table format above is from the latest Rel.18 38.101-1, note that the table format might be changed in Rel.19.

Editor's note: The valid UL configurations shall refer to the Annex B.

5.x.1.3	UE co-existence studies for 1 band UL

Editor’s Note: The tables in this section are provided to identify potential issues to be analysed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | nX | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow |  |  |  |  |  |  |
| nY | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | Editor’s Note: The analysis conclusion for the UL harmonic/harmonic mixing shall be included here, even if no issues are identified. |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| nX | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | Editor’s Note: The analysis conclusion for the UL harmonic/harmonic mixing shall be included here, even if no issues are identified. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_nX-nY.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis | Editor’s Note: The analysis conclusion for the cross- band isolation shall be included here, even if no issues are identified. |  | Editor’s Note: The analysis conclusion for the cross- band isolation shall be included here, even if no issues are identified. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.X.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for CA_ nX-nY

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| nX fUL |  |  | Order | flow | fhigh |
| nY fDL |  |  |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  |  |  |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| Analysis | Editor’s Note: The analysis conclusion for the UL CA IMD shall be included here, even if no issues are identified. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.x.1.4	∆TIB,c and ∆RIB,c values

Editor’s note: for the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1, the table below is from the latest Rel.18 38.101-1, note that the table format might be changed in Rel.19.

For CA_nX-nY, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_nX-nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_nX-nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

Editor's note 1: Text may be added if harmonics/harmonic mixing/cross band isolation/Intra-band ULCA IMD issues are identified in subclause 5.x.1.3.

Editor's note 2: Proponent are encouraged to provide the detailed technical analyzed of the MSD requirements. For example: RF architecrture, RF components parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

Editor's note 3: The table format shall be the same with the corresponding tables in TS38.101-1, i.e.: Table 7.3A.4-1, Table 7.3A.4-4, Table 7.3A.6-1 for harmonics, harmonic mixing and cross band isolation, respectively.

5.x.1.6	OOB blocking exception requirements

Table 5.x.1.6-1: CA band combination with exceptions allowed

CA band combination

5.x.2	Specific for 2 bands UL CA

Editor's note: Text will be added if 2 bands UL CA are supported, otherwise all the clauses shall be void.

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +x/-y |
| CA_nXA/B/C-nYA | 23 | +x/-y |

5.x.2.2	UE co-existence studies for 2 bands UL

Editor’s note: The tables in this section are provided to identify potential issues to be analysed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

Table 5.x.2.2-1 lists Band nX + Band nY 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band nX and Band nY for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, nth order IMD may also fall into Rx frequencies of bands nX or band nY.

Table 5.x.2.2-2 lists Band nX + Band nY 2 bands UL CA(3CC) triple beat IMD analysis for Band nX- Band nY

Table 5.x.2.2-2: Band nX and Band nY for 3CC UL IMD products

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | – |  | – |  |
| fDL | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the 3UL CC IMD shall be included here, even if no issues are identified. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.x.2.2-3 lists the protected bands required for the 2UL bands CA configuration.

Editor's note: If no need to add band combination to the protected band list, then Table 5.x.2.2-3 can be omitted.

Table 5.x.2.2-3: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_nX-nY | Frequency range |  | - |  |  |  | x |
| NOTE x: ..... Editor's note: The NOTE order must keep consistent with the Table 6.5A.3.2.3-1 in TS38.101-1. |  |  |  |  |  |  |  |

5.x.2.3	REFSENS requirements

Editor's note 1: Text will be added if  IMD due to 2 bands UL with 2 UL carriers or triple beat due to 2 bands UL with 3 UL carriers issues are identified in table 5.x.2.2-1 and table 5.x.2.2-2, respectively.

Editor's note 2: Proponent are encouraged to provide the detailed technical analysis of the MSD requirements. For example: RF architecture, RF components parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

5.1	CA_n3-n104

5.1.1	Common for 1 band UL and 2 bands UL CA

5.1.1.1	Operating bands for CA

Table 5.1.1.1-1:  CA band combination of band n3+n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n3 | 1710 MHz | - | 1785 MHz | 1805 MHz | - | 1880 MHz | FDD |
| n104 | 6425 MHz | - | 7125 MHz | 6425 MHz | - | 7125 MHz | TDD |

5.1.1.2	Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported bandwidths per CA band combination of band n3+n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n104A | CA_n3A-n104A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n104 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n104C | CA_n104C CA_n3A-n104A CA_n3A-n104C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n104 | CA_n104C_BCS0 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n104 | CA_n104C_BCS 4 and 5 |  |

5.1.1.3	UE co-existence studies for 1 band UL

Table 5.1.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n104.

Table 5.1.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n3 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 | MSD type |
| n104 | fLow | fHigh | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 6425 | 7125 | N/A | - | - | D | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n3 UL4 / n104 DL1, and no near miss n3 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| UL/DL |  | n104 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| n3 | fLow | fHigh | 7125 | 14250 | 21375 | 28500 | 35625 |  |
| DL1 | 1805 | 1880 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3610 | 3760 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5415 | 5640 | - | - | N/A | - | N/A |  |
| DL4 | 7220 | 7520 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9025 | 9400 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit n104 ULx / n1 DLy nor Near miss n104 ULx / n3 DLy (20MHz) |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.1.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n3-n104.

Table 5.1.1.3-2: Cross-band isolation analysis

| Bands | n3 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1710 | 1785 | 6425 | 7125 |
| fDL (MHz) | 1805 | 1880 | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1660 | 1835 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1610 | 1885 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1560 | 1935 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1510 | 1985 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1460 | 2035 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n3 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n3 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There are no cross-band isolation problem for n104 UL to n3 DL up to ACLR 5 according to the calculation. Moreover, n3 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.1.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA_n104C may occur for CA_ n3-n104

Table 5.1.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n104 fUL | 6425 | 7125 | Order | flow | fhigh |
| n3 fDL | 1805 | 1880 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 200 |  | 0 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 6225 | 7325 |  | 0 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 6025 | 7525 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 5825 | 7725 |  | 12650 | 14450 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 5625 | 7925 |  | 12450 | 14650 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 5425 | 8125 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 5225 | 8325 |  | 19075 | 21575 |
| Analysis | There are no IMD problem for n3 UL caused by CA_n104C according to the calculation. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.1.1.4	∆TIB,c and ∆RIB,c values

For CA_n3-n104, it is proposed to use n3-n102  TIB,c and RIB,c requirements, which are given in the tables below.

Table 5.1.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.1.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.1.1.5	REFSENS requirements

Considering the n3 is mid band and n104 is ultra-band, here we use the full separate antenna RF architecture assumption for the MSD derivation. The MSD for H4 caused by n3 UL falls into n104 DL are proposed in the table 5.1.1.5-1:

Table 5.1.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n104 | 5 | 15 | 6 | 20 | 11.7 | NOTE 4 | UL4/DL1 direct-hit |
| n3 | n104 | 5 | 15 | 6 | 100 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

5.1.1.6	OOB blocking exception requirements

No OOB blocking issues for CA_n3-n104.

    - 5.1.2	Specific for 2 bands UL CA

5.1.2.1	Maximum output power for inter-band CA

Table 5.1.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n104A | 23 | +2/-3 |
| CA_n3A-n104C | 23 | +2/-3 |

5.1.2.2	UE co-existence studies for 2 bands UL

Table 5.1.2.2-1 lists Band n3 + Band n104 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.2.2-1: Band n3 and Band n104 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 6425 | 7125 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 4640 -- 5415 |  | 8135 -- 8910 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3705 -- 2855 |  | 11065 -- 12540 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9845 -- 10695 |  | 14560 -- 16035 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1995 -- 1070 |  | 17490 -- 19665 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 10830 -- 9280 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high+1*y_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high+1*fx_high¦ |
| IMD frequency limits (MHz) | 11555 -- 12480 |  | 20985 -- 23160 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2*y_high¦ |  |  |
| IMD frequency limits (MHz) | 16270 -- 17820 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 26790 -- 23915 |  | 715 -- 285 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 17955 -- 15705 |  | 7495 -- 9120 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27410 -- 30285 |  | 13265 -- 14265 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + *fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + *fx_high¦ |
| IMD frequency limits (MHz) | 22695 -- 24945 |  | 17980 -- 19605 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.1.2.2-1, 4th order IMD may also fall into Rx frequencies of bands n3.

Regarding the triple beat IMD cause by the UL CA_n3A-n104C, in terms of the band group definition in Annex A, we can see that band n3 belongs to FR1-b frequency group while band n104 belongs to FR1-e frequency group, which means these two bands are not part of the same or adjacent band groups, so the triple beat analysis can be ignored. There are no triple beat IMD MSD needed to be defined.

For CA_n3-n104, although PHS frequency range 1884.5-1915.7MHz is as pretected band of band n3, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n3-n104. Thus no protected bands are needed.

5.1.2.3	REFSENS requirements

Based on the co-existence studies there is a need to define IMD4 MSD values, which is proposed in table 5.1.2.3-1:

Table 5.1.2.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n104 | n3 | 1750 | 5 | 25 | 1845 | 9.0 | FDD | IMD4 |
|  | n104 | 7095 | 20 | 50 | 7095 | N/A | TDD | N/A |

5.2	CA_n8-n104

5.2.1	Common for 1 band UL and 2 bands UL CA

5.2.1.1	Operating bands for CA

Table 5.2.1.1-1:  CA band combination of band n8+n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n8 | 880 MHz | - | 915 MHz | 925 MHz | - | 960 MHz | FDD |
| n104 | 6425 MHz | - | 7125 MHz | 6425 MHz | - | 7125 MHz | TDD |

5.2.1.2	Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination of band n8+n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n104A | CA_n8A-n104A | n8 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n104 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n104C | CA_n104C CA_n8A-n104A CA_n8A-n104C | n8 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n104 | CA_n104C_BCS0 |  |
|  |  | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n104 | CA_n104C_BCS 4 and 5 |  |

5.2.1.3	UE co-existence studies for 1 band UL

Table 5.2.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n104.

Table 5.2.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n8 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 880 | 1760 | 2640 | 3520 | 4400 | MSD type |
| n104 | fLow | fHigh | 915 | 1830 | 2745 | 3660 | 4575 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n8 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| UL/DL |  | n104 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| n8 | fLow | fHigh | 7125 | 14250 | 21375 | 28500 | 35625 |  |
| DL1 | 925 | 960 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1850 | 1920 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2775 | 2880 | - | - | N/A | - | N/A |  |
| DL4 | 3700 | 3840 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 4625 | 4800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n104 ULx / n8 DLy (20MHz) |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.2.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n8-n104.

Table 5.2.1.3-2: Cross-band isolation analysis

| Bands | n8 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 880 | 915 | 6425 | 7125 |
| fDL (MHz) | 925 | 960 | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 35 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 845 | 950 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 810 | 985 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 775 | 1020 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 740 | 1055 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 705 | 1090 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n8 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n8 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There are no cross-band isolation problem for n104 UL to n8 DL up to ACLR 5 according to the calculation. Moreover, n8 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.2.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA_n104C may occur for CA_ n8-n104

Table 5.1.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n104 fUL | 6425 | 7125 | Order | flow | fhigh |
| n8 fDL | 925 | 960 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 200 |  | 0 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 6225 | 7325 |  | 0 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 6025 | 7525 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 5825 | 7725 |  | 12650 | 14450 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 5625 | 7925 |  | 12450 | 14650 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 5425 | 8125 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 5225 | 8325 |  | 19075 | 21575 |
| Analysis | There are no IMD problem for n8 DL caused by UL CA_n104C according to the calculation. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.2.1.4	∆TIB,c and ∆RIB,c values

For CA_n8-n104, the TIB,c and RIB,c requirements are given in the tables below.

Table 5.2.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n8-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n8 the band order from left to right is n1 and n8. |  |  |

Table 5.2.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n8-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.2.1.5	REFSENS requirements

There are no harmonic/harmonic mixing/cross band isolation co-existence problem according to UE co-existence study in section 5.2.1.3, so there is no need to defined additional REFSEN requirements(MSD).

5.2.1.6	OOB blocking exception requirements

Since band n8 is a low band and n104 is a high and wide band, the OOBB exception is needed.

Table 5.2.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n8-n104

    - 5.2.2	Specific for 2 bands UL CA

5.2.2.1	Maximum output power for inter-band CA

Table 5.2.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n104A | 23 | +2/-3 |
| CA_n8A-n104C | 23 | +2/-3 |

5.2.2.2	UE co-existence studies for 2 bands UL

Table 5.2.2.2-1 lists Band n8 + Band n104 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.2.2.2-1: Band n8 and Band n104 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 880 | 915 | 6425 | 7125 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 5510 -- 6220 |  | 7305 -- 8015 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5340 -- 4595 |  | 11935 -- 13320 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8185 -- 8930 |  | 13730 -- 15115 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high– 1*fy_low¦ | ¦3*fy_low–1*fx_high¦ | ¦3*fy_high– 1*fx_low¦ |
| IMD frequency limits (MHz) | 4460 -- 3680 |  | 18360 -- 20420 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high– 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 12440 -- 11020 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1*y_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9065 -- 9845 |  | 20155 -- 22215 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high+2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 14610 -- 16030 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 27520 -- 24785 |  | 2765 -- 3580 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high–3*fy_low¦ | ¦2*fy_low–3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 19540 -- 17445 |  | 10105 -- 11560 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 26580 -- 29315 |  | 9945 -- 10760 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high+ *fy_high¦ | ¦2*fy_low+3*fx_low¦ | ¦2*fy_high+3*fx_high¦ |
| IMD frequency limits (MHz) | 21035 -- 23130 |  | 15490 -- 16945 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.2.2.2-1, No IMD fall into Rx frequencies of bands n8.

Regarding the triple beat IMD cause by the UL CA_n8A-n104C, in terms of the band group definition in Annex A, we can see that band n8 belongs to FR1-a frequency group while band n104 belongs to FR1-e frequency group, which means these two bands are not part of the same or adjacent band groups, so the triple beat analysis can be ignored. There are no triple beat IMD MSD needed to be defined.

For CA_n8-n104, although PHS frequency range 1884.5-1915.7MHz is as pretected band of band n8, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n8-n104. Thus no protected bands are needed.

5.2.2.3	REFSENS requirements

There are no IMD co-existence problem according to UE co-existence study in section 5.2.2.2, so there is no need to defined additional REFSEN requirements(MSD).

5.3	CA_n2-n5

5.3.1	Common for 1 band UL and 2 bands UL CA

5.3.1.1	Operating bands for CA

Table 5.3.1.1-1:  CA band combination of band n2+n5

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

5.3.1.2	Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported bandwidths per CA band combination of band n2+n5

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A | CA_n2A-n5A | n2 | n2 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n2A-n5B | CA_n2A-n5A CA_n5B | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | CA_n5B_BCS0 |  |
|  |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |
| CA_n2(2A)-n5B | CA_n2A-n5A CA_n5B | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n5 | CA_n5B_BCS 4 and 5 |  |

5.3.1.3	UE co-existence studies for 1 band UL

Table 5.3.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2A-n5A.

Table 5.3.1.3-1: UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n5 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n2 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n5 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 869 | 894 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 1738 | 1788 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 2607 | 2682 |  |  | N/A |  | N/A |  |
| DL4 | 3476 | 3576 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 4345 | 4470 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on Table 5.3.1.3-1:

-	There is no harmonic or harmonic mixing collision detected for this band combination.

Table 5.3.1.3-2 summarizes frequency ranges where cross-band isolation may occur for CA_n2A-n5A.

Table 5.3.1.3-2: Cross-band isolation analysis

| Bands | n2 |  | n5 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 824 | 849 |
| fDL (MHz) | 1930 | 1990 | 869 | 894 |
| CBW (MHz) | 5 | 40 | 5 | 20 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 804 | 869 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 784 | 889 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 764 | 909 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 744 | 929 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 724 | 949 |
| Analysis | There is no overlap of the band n2 ACLR range with the band n5 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n5 so no cross-band MSD is needed. |  | There is no overlap of the band n5 ACLR range with the band n2 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n2 so no cross-band MSD is needed. |  |

Based on Table 5.3.1.3-2:

-	There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

Table 5.3.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for n5B.

Table 5.3.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n5 fUL | 824 | 849 | Order | flow | fhigh |
| n2 fDL | 1850 | 1910 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 25 |  | 10 | 25 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 50 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 799 | 874 |  | 30 | 75 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 774 | 899 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 749 | 924 |  | 1623 | 1723 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 724 | 949 |  | 1598 | 1748 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 699 | 974 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 674 | 999 |  | 2447 | 2572 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Based on Table 5.3.1.3-3:

-	There is no uplink interference for the intra-band ULCA case.

5.3.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n5, the ∆TIB,c and ∆RIB,c values have already been specified.

5.3.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

5.3.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

5.3.2	Specific for 2 bands UL CA

5.3.2.1	Maximum output power for inter-band CA

Maximum output power requirements have already been specified.

5.3.2.2	UE co-existence studies for 2 bands UL

Study has already been completed for the supported 2 band UL operation.

5.3.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.4	CA_n2-n48

5.4.1	Common for 1 band UL and 2 bands UL CA

5.4.1.1	Operating bands for CA

Table 5.4.1.1-1:  CA band combination of band n2+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination of band n2+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band |  |  | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n48A | CA_n2A-n48A | n2 |  |  | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n48 |  |  | n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n2A-n48B | CA_n48B CA_n2A-n48A CA_n2A-n48B |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  | n48 | CA_n48B_BCS 4 and 5 |  |  |  |
| CA_n2(2A)-n48B | CA_n48B CA_n2A-n48A CA_n2A-n48B |  | n2 | CA_n2(2A)_BCS 4 and 5 |  | 4 and 5 |  |
|  |  |  | n48 | CA_n48B_BCS 4 and 5 |  |  |  |

5.4.1.3	UE co-existence studies for 1 band UL

Table 5.4.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2-n48.

Table 5.4.1.3-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n48 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n2 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n48 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 3550 | 3700 | N/A | N |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a near miss collision between the band n2 2nd harmonic to the band n48 DL. |  |  |  |  |  |

Based on Table 5.4.1.3-1:

For the n2 uplink 2nd harmonic falling to the band n48, the MSD defined in Table 7.3A.4-1 of 38.101-1 can be applied. There is no newer requirement needed.

Table 5.4.1.3-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.4.1.3-2: Cross-band isolation analysis

| Bands | n2 |  | n48 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 3550 | 3700 |
| fDL (MHz) | 1930 | 1990 | 3550 | 3700 |
| CBW (MHz) | 5 | 40 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 3450 | 3800 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 3350 | 3900 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 3250 | 4000 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 3150 | 4100 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 3050 | 4200 |
| Analysis | There is no overlap of the band n2 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  | There is no overlap of the band n48 ACLR range with the band n2 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n2 so no cross-band MSD is needed. |  |

Based on Table 5.4.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

5.4.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n48, the ∆TIB,c and ∆RIB,c values have already been specified.

5.4.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

5.4.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

5.4.2	Specific for 2 bands UL CA

5.4.2.1	Maximum output power for inter-band CA

Table 5.4.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n48B | 23 | +2/-3 |

5.4.2.2	UE co-existence studies for 2 bands UL

Table 5.4.2.2-2 lists Band n2 + Band n48 2 bands UL CA(3CC) triple beat IMD analysis for Band n2- Band n48

Table 5.4.2.2-2: Band n2 and Band n48 for 3CC UL IMD products

| Band / CA1 | n2A |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1850-1910 |  | 3550-3700 |  |
| fDL | 1930-1990 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -100 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1750-1850 |  | 1910-2010 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Based on Table 5.4.2.2-2: The bands are not of the same or adjacent band groups, so NOTE 1 applies.

5.4.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.5	CA_n2-n66

5.5.1	Common for 1 band UL and 2 bands UL CA

5.5.1.1	Operating bands for CA

Table 5.5.1.1-1:  CA band combination of band n2+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

5.5.1.2	Channel bandwidths per operating band for CA

Table 5.5.1.2-1: Supported bandwidths per CA band combination of band n2+n66

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n66A | CA_n2A-n66A | n2 | See n2 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

5.5.1.3	UE co-existence studies for 1 band UL

##### 5.5.1.3.1	Co-existence studies for 2UL band with 1CC per band

Table 5.5.1.3.1-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2-n66.

Table 5.5.1.3.1-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n66 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n2 | fLow | fHigh | 1780 | 3560 | 5340 | 7120 | 8900 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n66 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 2110 | 2200 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 4220 | 4400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6600 |  |  | N/A |  | N/A |  |
| DL4 | 8440 | 8800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 10550 | 11000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on the above table: There is no harmonic or harmonic mixing collision detected for this band combination.

Table 5.5.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.5.1.3.1-2: Cross-band isolation analysis

| Bands | n2 |  | n66 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 1710 | 1780 |
| fDL (MHz) | 1930 | 1990 | 2110 | 2200 |
| CBW (MHz) | 5 | 40 | 5 | 45 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 1665 | 1825 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 1620 | 1870 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 1575 | 1915 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 1530 | 1960 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 1485 | 2005 |
| Analysis | There is an overlap of the band n2 ACLR5 range with the band n66 DL. Therefore, the cross-band isolation MSD should be studied. |  | There are an overlaps of the band n66 ACLR 4 and 5 ranges with the band n2 DL. Therefore, the cross-band isolation MSD should be studied. |  |

Based on the table above:

For the cross-band overlapping from uplink n66 to n2 downlink, the same degradation proposal approved in CR [4] will be applied as shown below. No additional MSD is needed.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n2 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.2 | >ACLR2 |

For the cross-band overlapping from uplink n2 to n66 downlink, the RBs in uplink n2 are never fully loaded in REFSENS configuration. There is no issue and there is no need to specify MSD for the PC3.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1910 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | [0] | >ACLR2 |

5.6	CA_n5-n48

5.6.1	Common for 1 band UL and 2 bands UL CA

5.6.1.1	Operating bands for CA

Table 5.6.1.1-1:  CA band combination of band n5+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

5.6.1.2	Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination of band n5+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n48A | CA_n5A-n48A | n5 | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n5A-n48B | CA_n48B CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
|  | CA_n48B CA_n5A-n48A CA_n5A-n48B | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
| CA_n5B-n48A | CA_n5B CA_n5A-n48A CA_n5A-n48A | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5B-n48B | CA_n5B CA_n48B CA_n5A-n48A CA_n5A-n48B | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48B_BCS 4 and 5 |  |
| CA_n5B-n48(2A) | CA_n5B CA_n5A-n48A | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS 4 and 5 |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

5.6.1.3	UE co-existence studies for 1 band UL

Table 5.6.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n5-n48.

Table 5.6.1.3-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n48 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n5 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 869 | 894 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 1738 | 1788 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 2607 | 2682 |  |  | N/A |  | N/A |  |
| DL4 | 3476 | 3576 | D | N/A | N/A | N/A | N/A |  |
| DL5 | 4345 | 4470 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is collision detected from downlink band n5 4th harmonic mixing with n48 uplink. |  |  |  |  |  |
| UL/DL |  | n5 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n48 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 3550 | 3700 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on Table 5.6.1.3-1:

-	For the 4th DL harmonic-mixing falling on the downlink, the BSD for CA_n5-n48 for PC3 should be same as the defined CA_n26-n48. This is because the new higher channel bandwidth introduced for band n48 BCS 4 and 5 is the same as the BCS 0 and 1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n48 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |

Table 5.6.1.3-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.6.1.3-2: Cross-band isolation analysis

| Bands | n5 |  | n48 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 824 | 849 | 3550 | 3700 |
| fDL (MHz) | 869 | 894 | 3550 | 3700 |
| CBW (MHz) | 5 | 25 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 799 | 874 | 3450 | 3800 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 774 | 899 | 3350 | 3900 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 749 | 924 | 3250 | 4000 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 724 | 949 | 3150 | 4100 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 699 | 974 | 3050 | 4200 |
| Analysis | There is no overlap of the band n5 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  | There is no overlap of the band n48 ACLR range with the band n5 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n5 so no cross-band MSD is needed. |  |

Based on Table 5.6.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

Table 5.6.1.3-3 summarizes frequency ranges where IMD products caused by n48 UL band with 2CC intra-band UL CA may occur for n48B.

Table 5.6.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n48 fUL | 3550 | 3700 | Order | flow | fhigh |
| n5 fDL | 869 | 894 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 20 | 100 |  | 20 | 100 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 40 | 200 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 3450 | 3800 |  | 60 | 300 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 3350 | 3900 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 3250 | 4000 |  | 7000 | 7500 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 3150 | 4100 |  | 6900 | 7600 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 3050 | 4200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 2950 | 4300 |  | 10550 | 11200 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Based on Table 5.6.1.3-3:

-	There are no issues for uplink CA_n48B

Table 5.6.1.3-4 summarizes frequency ranges where IMD products caused by n5 UL band with 2CC intra-band UL CA may occur for n5B.

Table 5.6.1.3-4: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n5 fUL | 824 | 849 | Order | flow | fhigh |
| n48 fDL | 3550 | 3700 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 25 |  | 10 | 25 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 50 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 799 | 874 |  | 30 | 75 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 774 | 899 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 749 | 924 |  | 1623 | 1723 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 724 | 949 |  | 1598 | 1748 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 699 | 974 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 674 | 999 |  | 2447 | 2572 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Based on Table 5.6.1.3-4:

-	There are no issues for uplink CA_n5B.

5.6.1.4	∆TIB,c and ∆RIB,c values

For CA_n5-n48, the ∆TIB,c and ∆RIB,c values have already been specified.

5.6.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

5.6.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

5.6.2	Specific for 2 bands UL CA

5.6.2.1	Maximum output power for inter-band CA

Table 5.6.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n48B | 23 | +2/-3 |

5.6.2.2	UE co-existence studies for 2 bands UL

Table 5.6.2.2-1 lists Band n5 + Band n48 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.6.2.2-1: Band n5 and Band n48 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2701 - 2876 |  | 4374 - 4549 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1852 - 2052 |  | 6251 - 6576 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5198 - 5398 |  | 7924 - 8249 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1003 - 1228 |  | 9801 - 10276 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 5402 - 5752 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6022 - 6247 |  | 11474 - 11949 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8748 - 9098 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13351 - 13976 |  | 154 - 404 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8952 - 9452 |  | 4553 - 4928 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15024 - 15649 |  | 6846 - 7096 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12298 - 12798 |  | 9572 - 9947 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.6.2.2-2 lists Band n5 + Band n48 2 bands UL CA(3CC) triple beat IMD analysis for Band n5- Band n48

Table 5.6.2.2-2: Band n5 and Band n48 for 3CC UL IMD products

| Band / CA1 | n5 |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 824-849 |  | 3550-3700 |  |
| fDL | 869-894 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -100 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 724-824 |  | 849-949 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Based on Table 5.6.2.2-2: 	The bands are not of the same or adjacent band groups, so NOTE 1 applies.

5.6.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.7	CA_n48-n66

5.7.1	Common for 1 band UL and 2 bands UL CA

5.7.1.1	Operating bands for CA

Table 5.7.1.1-1:  CA band combination of band n48+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination of band n48+n66

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n66A | CA_n48A-n66A | n48 | See n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| CA_n48B-n66A | CA_n48B CA_n48A-n66A CA_n48B-n66A | n48 | CA_n48B_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48B-n66(2A) | CA_n48B CA_n48A-n66A CA_n48B-n66A | n48 | CA_n48B_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

5.7.1.3	UE co-existence studies for 1 band UL

Table 5.7.1.3-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n48-n66.

Table 5.7.1.3-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n66 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n48 | fLow | fHigh | 1780 | 3560 | 5340 | 7120 | 8900 |  |
| DL1 | 3550 | 3700 | N/A | D |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a direct hit from the band n66 2nd harmonic to the band n48 |  |  |  |  |  |
| UL/DL |  | n48 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n66 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 2110 | 2200 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 4220 | 4400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6600 |  |  | N/A |  | N/A |  |
| DL4 | 8440 | 8800 |  | N/A | N/A |  | N/A |  |
| DL5 | 10550 | 11000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing |  |  |  |  |  |

Based on Table 5.7.1.3-1:

-	For the uplink n66 2nd harmonic falling to the band n77, the MSD value has been defined in Table 7.3A.4-1 of 38.101-1. In fact, the band n66 channel bandwidth for BCS 4/5 is the same as BCS 2. Therefore, there is no newer requirement needed.

Table 5.7.1.3-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.7.1.3-2: Cross-band isolation analysis

| Bands | n48 |  | n66 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 3550 | 3700 | 1710 | 1780 |
| fDL (MHz) | 3550 | 3700 | 2110 | 2200 |
| CBW (MHz) | 5 | 100 | 5 | 45 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3450 | 3800 | 1665 | 1825 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3350 | 3900 | 1620 | 1870 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3250 | 4000 | 1575 | 1915 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 3150 | 4100 | 1530 | 1960 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 3050 | 4200 | 1485 | 2005 |
| Analysis | There is no overlap of the band n48 ACLR range with the band n66 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n66 so no cross-band MSD is needed. |  | There is no overlap of the band n66 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  |

Based on Table 5.7.1.3-2:

-	There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

5.7.1.4	∆TIB,c and ∆RIB,c values

For CA_n48-n66, the ∆TIB,c and ∆RIB,c values have already been specified.

5.7.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

5.7.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

5.7.2	Specific for 2 bands UL CA

5.7.2.1	Maximum output power for inter-band CA

Table 5.4.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n48B-n66A | 23 | +2/-3 |

5.7.2.2	UE co-existence studies for 2 bands UL

Table 5.7.2.2-1 lists Band n48 + Band n66 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.7.2.2-1: Band n48 and Band n66 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1770 - 1990 |  | 5260 - 5480 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5320 - 5690 |  | 10 - 280 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8810 - 9180 |  | 6970 - 7260 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8870 - 9390 |  | 1430 - 1790 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3540 - 3980 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12360 - 12880 |  | 8680 - 9040 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10520 - 10960 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3140 - 3570 |  | 12420 - 13090 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1760 - 2270 |  | 7090 - 7680 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10390 - 10820 |  | 15910 - 16580 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12230 - 12740 |  | 14070 - 14660 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.7.2.2-2 lists Band n48 + Band n66 2 bands UL CA(3CC) triple beat IMD analysis for Band n48- Band n66

Table 5.7.2.2-2: Band n48 and Band n66 for 3CC UL IMD products

| Band / CA1 | n66 |  | CA_n48B |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1710-1780 |  | 3550-3700 |  |
| fDL | 2110-2200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -100 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1610-1710 |  | 1780-1880 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Based on Table 5.7.2.2-2:

-	The bands are not of the same or adjacent band groups, so NOTE 1 applies.

5.7.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.8	CA_n26-n78

5.8.1	Specific for 1 bands UL CA

5.8.1.1	Operating bands for CA

CA_n26-n78 has already been specified in TS 38.101-1 and this section does not need to be revisited.

5.8.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) need to be added to DL CA_n26A-n78(2A) and DL CA_n26(2A)-n78(2A) in the existing configuration table of 38.101-1. See Table 5.8.1.2-2 below.

Table 5.8.1.2-2: Supported bandwidths per CA band combination of band n26+n78

| CA_n26A-n78(2A) | CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n26 | n26 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n26(2A)-n78(2A) | CA_n26(2A) CA_n78(2A) CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |

5.8.1.3	Co-existence studies

Table 5.8.2.2-3 gives a non-contiguous uplink IMD interference analysis

Table 5.8.2.2-3: Non-contiguous uplink IMD analysis

| UE UL carriers | fx_low = 3300 | fx_high = 3320 | fy_low = 3780 | fy_high = 3800 |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 460 – 500 |  | 7080 – 7120 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2800 – 2860 |  | 4240 – 4300 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10380 – 10440 |  | 10860 – 10920 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6100 – 6180 |  | 8020 – 8100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 920 – 1000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 13680 – 13760 |  | 14640 – 14720 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 14160 – 14240 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11800 – 11900 |  | 9400 – 9500 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4700 – 4800 |  | 2300 – 2400 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18420 – 18520 |  | 16980 – 17080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17940 – 18040 |  | 17460 – 17560 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the table 5.8.2.2-3, there are no IMD products falling inside n26 DL.

5.8.1.4	∆TIB and ∆RIB values

CA_n26-n78 has already been specified and this section does not need to be revisited.

5.8.1.5	REFSENs requirements

Based on the co-existence studies there is no need to define MSD values.

5.9	CA_n1-n71

5.9.1	Common for 1 band UL and 2 bands UL CA

5.9.1.1	Operating bands for CA

Table 5.9.1.1-1:  CA band combination of band n1+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.9.1.2	Channel bandwidths per operating band for CA

Table 5.9.1.2-1: Supported bandwidths per CA band combination of band n1+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n71A | CA_n1A-n71A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.9.1.3	UE co-existence studies

Table 5.9.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n1-n71.

Table 5.9.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n1 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | 1920 | 1920 | 3840 | 5760 | 7680 | 9600 | MSD type |
| n71 | 663 | 698 | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 617 | 652 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 1234 | 1304 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | D | Clear | N/A | Clear | N/A |  |
| DL4 | 2468 | 2608 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | There is UL1/DL3 harmonic mixing into n71 |  |  |  |  |  |
| UL/DL |  | n71 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | 663 | 663 | 1326 | 1989 | 2652 | 3315 |  |
| n1 | 1920 | 1980 | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 2110 | 2170 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 4220 | 4340 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 6330 | 6510 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 8440 | 8680 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 10550 | 10850 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | No issues |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.9.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n1-n71.

| Bands | n1 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1920 | 1980 | 663 | 698 |
| fDL (MHz) | 2110 | 2170 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 30 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2030 | 633 | 728 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2080 | 603 | 758 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2130 | 573 | 788 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2180 | 543 | 818 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2230 | 513 | 848 |
| Analysis | No overlap |  | No overlap |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.9.1.4	∆TIB,c and ∆RIB,c values

For CA_n1-n71, the TIB,c and RIB,c values are given in the tables below re-used from CA_n1-n105.

Table 5.9.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n1-n71 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.9.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n1-n71 | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.9.1.5	REFSENS requirements

Based on the co-existence studies n1 uplink and n71 third harmonic mixes into the n71 downlink. The value is based on DC_1A_n71A, which has similar harmonic issue.

Table 5.9.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.9.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.9.2	Specific for 2 bands UL CA

5.9.2.1	Maximum output power for inter-band CA

Table 5.9.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n71A | 23 | +2/-3 |

5.9.2.2	UE co-existence studies

Table 5.9.2.2-1 lists Band n1 + Band n71 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.9.2.2-1: Band n1 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1890 - 2010 |  | 613 - 748 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5236 - 5286 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.9.2.2-1, 3rd order IMD may also fall into Rx frequencies of bands n1 or band n71. The analysis shows IMD3 will fall inside n71 DL.

As agreed, there’s no regulatory requirements that need to be protected in the geographically area that this combination is used.

5.9.2.3	REFSENS requirements

Table 5.9.2.3-1 lists the MSD required due to the 3rd IMD product for the dual uplink configuration. It is based on the similar case of DC_1A_n71A.

Table 5.9.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |

5.10	CA_n3-n71

5.10.1	Common for 1 band UL and 2 bands UL CA

5.10.1.1	Operating bands for CA

Table 5.10.1.1-1:  CA band combination of band n3+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.10.1.2	Channel bandwidths per operating band for CA

Table 5.10.1.2-1: Supported bandwidths per CA band combination of band n3+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n71A | CA_n3A-n71A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.10.1.3	UE co-existence studies

Table 5.10.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n3-n71.

Table 5.10.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n1 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | 1710 | 1710 | 3420 | 5130 | 6840 | 8550 | MSD type |
| n71 | 663 | 698 | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 617 | 652 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 1234 | 1304 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 2468 | 2608 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | No issues |  |  |  |  |  |
| UL/DL |  | n71 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | 663 | 663 | 1326 | 1989 | 2652 | 3315 |  |
| n1 | 1710 | 1785 | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 1805 | 1880 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 3610 | 3760 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 5415 | 5640 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 7220 | 7520 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 9025 | 9400 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | No issues |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.10.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n3-n71

| Bands | n3 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1710 | 1785 | 663 | 698 |
| fDL (MHz) | 1805 | 1880 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 30 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1660 | 1835 | 633 | 728 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1610 | 1885 | 603 | 758 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1560 | 1935 | 573 | 788 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1510 | 1985 | 543 | 818 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1460 | 2035 | 513 | 848 |
| Analysis | No overlap |  | No overlap |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.10.1.4	∆TIB,c and ∆RIB,c values

For CA_n3-n71, the TIB,c and RIB,c values are given in the tables below re-used from DC_3_n71.

Table 5.10.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n71 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.10.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n71 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.10.1.5	REFSENS requirements

Based on the co-existence there is no requirements to REFSENS.

5.10.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.10.2	Specific for 2 bands UL CA

5.10.2.1	Maximum output power for inter-band CA

Table 5.10.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n71A | 23 | +2/-3 |

5.10.2.2	UE co-existence studies

Table 5.10.2.2-1 lists Band n3 + Band n71 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.10.2.2-1: Band n3 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1680 - 1815 |  | 613 - 748 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4816 - 4896 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The analysis shows there’s no IMD issues.

As agreed, there’s no regulatory requirements that need to be protected in the geographically area that this combination is used.

5.10.2.3	REFSENS requirements

There is no additional REFSENS requirements.

5.11	CA_n20-n77

5.11.1	Common for 1 band UL and 2 bands UL CA

5.11.1.1	Operating bands for CA

Table 5.11.1.1-1:  CA band combination of band n20+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.11.1.2	Channel bandwidths per operating band for CA

Table 5.11.1.2-1: Supported bandwidths per CA band combination of band n20+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n20A-n77A | CA_n20A-n77A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n77(2A) | CA_n20A-n77A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |

5.11.1.3	UE co-existence studies

Table 5.11.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n20-n77.

Table 5.11.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n20 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | 832 | 832 | 1664 | 2496 | 3328 | 4160 | MSD type |
| n77 | 3300 | 4200 | 862 | 1724 | 2586 | 3448 | 4310 |  |
| DL1 | 3300 | 4200 | N/A | Clear | Clear | D | D | UL harmonic |
| DL23 | 6600 | 8400 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 13200 | 16800 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | UL4 and UL5 falls inside n77 DL |  |  |  |  |  |
| UL/DL |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | 3300 | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n20 | 832 | 862 | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 791 | 821 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 1582 | 1642 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 2373 | 2463 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 3164 | 3284 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 3955 | 4105 | D | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | DL5 falls inside n77 DL |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.11.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n20-n77.

Table 5.11.1.3-2: Cross-band isolation analysis

| Bands | n20 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 832 | 862 | 3300 | 4200 |
| fDL (MHz) | 791 | 821 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 20 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 812 | 882 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 792 | 902 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 772 | 922 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 752 | 942 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 732 | 962 | 2800 | 4700 |
| Analysis | No overlap |  | No overlap |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.11.1.4	∆TIB,c and ∆RIB,c values

For CA_n20-n41, the TIB,c and RIB,c values are given in the tables below re-used from DC_20_n77.

Table 5.11.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n77 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.11.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n77 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.11.1.5	REFSENS requirements

Based on the co-existence studies n20 fourth and fifth harmonic uplink falls into the n77 downlink. The UL harmonic MSD value is based on CA_n20A_n78A, from current meeting document R4-2413019 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC3.

Table 5.11.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n20 | n77 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n20 | n77 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Based on the co-existence studies n77 falls into the fifth harmonic n20 downlink. The harmonic mixing MSD value is based on CA_n28A_n77A, from current meeting document R4-2412621 CR for EN-DC Harmonic Mixing clean-up PC3.

Table 5.11.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | 20 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 20 | 10 | 15 | 25 | 20 | 23.5 | NOTE 2 | UL1/DL5 |
| NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

5.11.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.11.2	Specific for 2 bands UL CA

5.11.2.1	Maximum output power for inter-band CA

Table 5.11.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n20A-n77A | 23 | +2/-3 |

5.11.2.2	UE co-existence studies

Table 5.11.2.2-1 lists Band n20 + Band n77 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, where only n20 can fall victim, since it is FDD+TDD.

Table 5.11.2.2-1: Band n20 and Band n41 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 732 - 762 |  | 3280 - 4180 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |

The analysis shows IMD4 and IMD5 will fall inside n20 DL.

Based on the table 5.11.2.2-1, the 4th order and 5th order intermodulation product fall inside the RX band of n20.

The two bands are not part of the same or adjacent band groups, so the Triple beat analysis is omitted.

As agreed, there’s no regulatory requirements that need to be protected in the geographically area that this combination is used.

5.11.2.3	REFSENS requirements

Table 5.11.2.3-1 lists the MSD required due to the 4rd and 5th IMD product for the dual uplink configuration. It is based on the similar case of DC_20A_n77A.

Table 5.11.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n77 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n77 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
|  | n20 | 840 | 5 | 25 | 799 | 6.5 | FDD | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | TDD | N/A |

5.12	CA_n7-n29

5.12.1	Common for 1 band UL and 2 bands UL CA

5.12.1.1	Operating bands for CA

Table 5.12.1.1-1:  CA band combination of band n7+n29

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n29 | N/A | 717 MHz – 728 MHz | SDL |

5.12.1.2	Channel bandwidths per operating band for CA

Table 5.12.1.2-1: Supported bandwidths per CA band combination of band n7+n29

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n29A | - | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n29 | n29 channel bandwidths in Table 5.3.5-1 |  |

5.12.1.3	UE co-existence studies for 1 band UL

Table 5.12.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n7-n29.

Table 5.12.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n7 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 2500 | 5000 | 7500 | 10000 | 12500 |  |
| n29 | fLow | fHigh | 2570 | 5140 | 7710 | 10280 | 12850 |  |
| DL1 | 717 | 728 | N/A |  |  |  |  | UL harmonic |
| DL23 | 1434 | 1456 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 2151 | 2184 |  |  | N/A |  | N/A |  |
| DL4 | 2868 | 2912 |  | N/A | N/A | N/A | N/A |  |
| DL54 | 3585 | 3640 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL harmonics |  | n29 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| n7 | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no need to analyze harmonics/harmonic mixing for n29 UL/n7DL, since band n29 is SDL band and only has DL. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.12.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n7-n29.

Table 5.12.1.3-2: Cross-band isolation analysis

| Bands | n7 |  | n29 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2500 | 2570 |  |  |
| fDL (MHz) | 2620 | 2690 | 717 | 728 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 |  |  |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2450 | 2620 |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2400 | 2670 |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2350 | 2720 |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2300 | 2770 |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2250 | 2820 |  |  |
| Analysis | There is no overlap of the band n7 ACLR range with the band n29 DL up to order 5. Moreover, n7 and n29 are not the same or adjacent band group as described in table A.1, so no cross-band MSD is needed. |  | There is no need to calculate the band n29 ACLR range, since band n29 is SDL band and only has DL. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.12.1.4	∆TIB,c and ∆RIB,c values

For CA_n7-n29, the TIB,c and RIB,c values from CA_7-29 and are reused in the tables below.

Table 5.12.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n7-n29 | 0.3 | N/A |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.12.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n7-n29 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.12.1.5	REFSENS requirements

Based on the co-existence studies, no need to define exceptional REFSENS requirements.

5.12.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.12.1.6-1: CA band combination with exceptions allowed

CA band combination

5.13	CA_n29-n77

5.13.1	Common for 1 band UL and 2 bands UL CA

5.13.1.1	Operating bands for CA

Table 5.13.1.1-1:  CA band combination of band n29+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n29 | N/A | 717 MHz – 728 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.13.1.2	Channel bandwidths per operating band for CA

Table 5.13.1.2-1: Supported bandwidths per CA band combination of band n29+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n77A | - | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n29A-n77(2A) | CA_n77(2A) | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n29A-n77(3A) | CA_n77(2A) | n29 | n29 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |

5.13.1.3	UE co-existence studies for 1 band UL

Table 5.13.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n29-n77.

Table 5.13.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n77 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n29 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 717 | 728 | N/A |  |  |  |  | UL harmonic |
| DL23 | 1434 | 1456 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 2151 | 2184 |  |  | N/A |  | N/A |  |
| DL4 | 2868 | 2912 |  | N/A | N/A | N/A | N/A |  |
| DL54 | 3585 | 3640 | D |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is UL1/DL5 harmonic mixing into n29 |  |  |  |  |  |
| UL/DL harmonics |  | n29 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| n77 | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no need to analyze harmonics/harmonic mixing for n29 UL/n7DL, since band n29 is SDL band and only has DL. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.13.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n29-n77.

Table 5.13.1.3-2: Cross-band isolation analysis

| Bands | n29 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) |  |  | 3300 | 4200 |
| fDL (MHz) | 717 | 728 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  |  |  | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  | 2800 | 4700 |
| Analysis | There is no need to calculate the band n29 ACLR range, since band n29 is SDL band and only has DL. |  | There is no overlap of the band n77 ACLR range with the band n29 DL up to order 5. Moreover, n7 and n29 are not the same or adjacent band group as described in table A.1, so no cross-band MSD is needed. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.13.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for CA_ n29-n77.

Table 5.13.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n29 fDL | 717 | 728 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 600 |  | 0 | 600 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 1200 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2700 | 4800 |  | 0 | 1800 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2100 | 5400 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 1500 | 6000 |  |  |  |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 900 | 6600 |  |  |  |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 300 | 7200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | -300 | 7800 |  |  |  |
| Analysis | The analysis shows there’s IMD4 due to one UL band with 2CC intra-band UL CA. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.13.1.4	∆TIB,c and ∆RIB,c values

For CA_n29-n77, the TIB,c and RIB,c have already been specified and this section does not need to be revisited.

5.13.1.5	REFSENS requirements

-  Based on the co-existence studies on Table 5.13.1.3-1, the 5th harmonic mixing falls into Rx frequencies of band 29. And the MSD value has been defined in Table 7.3A.4-4 of TS38.101-1. There is no newer requirement needed.
-  Based on co-existence studies on Table 5.13.1.3-3, IMD4 of ULCA_n77(2A) falls into the n29DL, and the MSD requirements are defined as following.

Table 5.13.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n29-n77 | n29 | N/A | 5 | N/A | 720 | 8.6 | SDL | IMD415 |
|  | n7712 | 3540 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3850 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

5.13.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.13.1.6-1: CA band combination with exceptions allowed

CA band combination

5.14	CA_n20-n41

5.14.1	Common for 1 band UL and 2 bands UL CA

5.14.1.1	Operating bands for CA

Table 5.14.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n41 | 2496-2690 | 2496-2690 | TDD |

5.14.1.2	Channel bandwidths per operating band for CA

Table 5.14.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n41A | CA_n20A-n41A | n20 | 5,10,15,20 | 0 |
|  |  | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 |  |

5.14.1.3	UE co-existence studies for 1 band UL

Table 5.14.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n20-n41.

Table 5.14.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n20 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 832 | 1664 | 2496 | 3328 | 4160 |  |
| n41 | fLow | fHigh | 862 | 1724 | 2586 | 3448 | 4310 |  |
| DL1 | 2496 | 2690 | N/A | - | D | - | - | UL harmonic |
| DL23 | 4992 | 5380 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7488 | 8070 | - | - | N/A | - | N/A |  |
| DL4 | 9984 | 10760 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 12480 | 13450 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is 3rd order uplink harmonic into n41 DL (UL3/DL1) |  |  |  |  |  |
| UL/DL |  | n41 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 2496 | 4992 | 7488 | 9984 | 12480 |  |
| n20 | fLow | fHigh | 2690 | 5380 | 8070 | 10760 | 13450 |  |
| DL1 | 791 | 821 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1582 | 1642 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2373 | 2463 | - | - | N/A | - | N/A |  |
| DL4 | 3164 | 3284 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3955 | 4105 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issue |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.14.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n20-n41.

Table 5.14.1.3-2: Cross-band isolation analysis

| Bands | n20 |  | n41 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 832 | 862 | 2496 | 2690 |
| fDL (MHz) | 791 | 821 | 2496 | 2690 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 20 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 812 | 882 | 2396 | 2790 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 792 | 902 | 2296 | 2890 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 772 | 922 | 2196 | 2990 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 752 | 942 | 2096 | 3090 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 732 | 962 | 1996 | 3190 |
| Analysis | No issues |  | No issues |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if:- The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.14.1.4	∆TIB,c and ∆RIB,c values

For CA_n20-n41, the TIB,c and RIB,c values are given in the tables below re-used from DC_20_n41.

Table 5.14.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n41 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.14.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n41 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.14.1.5	REFSENS requirements

Based on the co-existence studies n20 third harmonic uplink falls into the n41 downlink. The value is based on DC_20A_n41A, which has similar harmonic issue.

Table 5.14.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n20 | n41 | 5 | 15 | 8 | 10 | 10.3 | NOTE 3 | UL3/DL1 |
| NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier |  |  |  |  |  |  |  |  |

5.14.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.14.2	Specific for 2 bands UL CA

5.14.2.1	Maximum output power for inter-band CA

Table 5.14.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n20A-n41A | 23 | +2/-3 |

5.14.2.2	UE co-existence studies for 2 bands UL

Table 5.14.2.2-1 lists Band n20 + Band n41 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.14.2.2-1: Band n20 and Band n41 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 - 1858 |  | 3328 - 3552 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 - 772 |  | 4130 - 4548 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 - 4414 |  | 5824 - 6242 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 - 90 |  | 6626 - 7238 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3716 - 3268 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 - 5276 |  | 8320 - 8932 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6656 - 7104 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 - 9122 |  | 952 - 638 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 - 5764 |  | 2406 - 2884 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 - 11622 |  | 5824 - 6138 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 - 9794 |  | 7488 - 7966 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The analysis shows IMD3 and IMD5 will fall inside n20 DL.

Table 5.14.2.2-3 lists the protected bands required for the 2UL bands CA configuration.

Table 5.14.2.2-3: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n20-n41 | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz |  |  |  |  |  |  |  |

5.14.2.3	REFSENS requirements

Table 5.14.2.3-1 lists the MSD required due to the 3rd and 5th IMD product for the dual uplink configuration. It is based on the similar case of DC_20A_n41A.

Table 5.14.2.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n41 | n20 | 851 | 5 | 25 | 810 | 12.1 | FDD | IMD3 |
|  | n41 | 2512 | 10 | 50 | N/A | N/A | TDD | N/A |
|  | n20 | 841 | 5 | 25 | 800 | 8.1 | FDD | IMD5 |
|  | n41 | 2564 | 10 | 50 | N/A | N/A | TDD | N/A |

5.15	CA_n20-n71

5.15.1	Common for 1 band UL and 2 bands UL CA

5.15.1.1	Operating bands for CA

Table 5.15.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n20 | 832-862 | 791-821 | FDD |
| n71 | 663-698 | 617-652 | FDD |

5.15.1.2	Channel bandwidths per operating band for CA

Table 5.15.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n20A-n71A | CA_n20A-n71A | n20 | 5,10,15,20 | 0 |
|  |  | n71 | 5,10,15,20 |  |

5.15.1.3	UE co-existence studies for 1 band UL

Table 5.15.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n20-n71.

Table 5.15.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n20 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 832 | 1664 | 2496 | 3328 | 4160 |  |
| n71 | fLow | fHigh | 862 | 1724 | 2586 | 3448 | 4310 |  |
| DL1 | 617 | 652 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1234 | 1304 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | - | - | N/A | - | N/A |  |
| DL4 | 2468 | 2608 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issue |  |  |  |  |  |
| UL/DL |  | n71 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 663 | 1326 | 1989 | 2652 | 3315 |  |
| n20 | fLow | fHigh | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 791 | 821 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1582 | 1642 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2373 | 2463 | - | - | N/A | - | N/A |  |
| DL4 | 3164 | 3284 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3955 | 4105 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issue |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.15.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n20-n71.

Table 5.15.1.3-2: Cross-band isolation analysis

| Bands | n20 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 832 | 862 | 663 | 698 |
| fDL (MHz) | 791 | 821 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 20 | 5 | 20 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 812 | 882 | 643 | 718 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 792 | 902 | 623 | 738 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 772 | 922 | 603 | 758 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 752 | 942 | 463 | 898 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 732 | 962 | 413 | 873 |
| Analysis |  |  | There is ACLR5 into n20 |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.15.1.4	∆TIB,c and ∆RIB,c values

For CA_n20-n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.15.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n71 | 0.8 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.15.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n71 | 0.4 | 0.4 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.15.1.5	REFSENS requirements

Table 5.15.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n20 | 688 | 20 | 15 | 20 (RBstart=86) | 796 | 5 | 3.0 | >ACLR2 |
| n20 | n71 | 842 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 2.6 | >ACLR2 |

5.15.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.15.2	Specific for 2 bands UL CA

5.15.2.1	Maximum output power for inter-band CA

Table 5.15.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n20A-n71A | 23 | +2/-3 |

5.15.2.2	UE co-existence studies for 2 bands UL

Table 5.15.2.2-1 lists Band n20 + Band n71 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.15.2.2-1: Band n20 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 199 - 134 |  | 1495 - 1560 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 966 - 1061 |  | 464 - 564 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2327 - 2422 |  | 2158 - 2258 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1798 - 1923 |  | 1127 - 1262 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 268 - 398 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3159 - 3284 |  | 2821 - 2956 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 2990 - 3120 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1960 - 1790 |  | 2785 - 2630 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 430 - 265 |  | 1260 - 1100 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3484 - 3654 |  | 3991 - 4146 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3653 - 3818 |  | 3822 - 3982 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

There are no intermodulation products in any of the receive bands.

Table 5.15.2.2-3 lists the protected bands required for the 2UL bands CA configuration.

Table 5.15.2.2-3: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n20-n71 | Frequency range | 758 | - | 788 | -50 | 1 |  |

5.15.2.3	REFSENS requirements

Based on the co-existence there’s no additional REFSENS requirements.

5.16	CA_n41-n78

5.16.1	Common for 1 band UL and 2 bands UL CA

5.16.1.1	Operating bands for CA

Table 5.16.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

5.16.1.2	Channel bandwidths per operating band for CA

Table 5.16.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41A-n78C | CA_n78C CA_n41A-n78A CA_n41A-n78C | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 | 0 |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |

5.16.1.3	UE co-existence studies for 1 band UL

Table 5.16.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for headline

Table 5.16.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n78 fUL | 3300 | 3800 | Order | flow | fhigh |
| n41 fDL | 2496 | 2690 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 | Min2CCBW | Max2CCBW |
|  | 70 | 200 | (1-1) | 70 | 200 |
| Close to UL IMD range2 |  |  | IMD4 | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh | (2-2) | 140 | 400 |
| IMD3 | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 | 3*Min2CCBW | 3*Max2CCBW |
| (2-1) | 3100 | 4000 | (3-3) | 210 | 600 |
| IMD5 | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
| (3-2) | 2900 | 4200 | Order | flow | fhigh |
| IMD7 | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| (4-3) | 2700 | 4400 | (3-1) | 6400 | 7800 |
| IMD9 | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| (5-4) | 2500 | 4600 | (4-2) | 6200 | 8000 |
| IMD11 | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
| (6-5) | 2300 | 4800 | Order | flow | fhigh |
| IMD13 | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| (7-6) | 2100 | 5000 | (4-1) | 9700 | 11600 |
| Analysis | IMD9, IMD11 and IMD13 of n78C may fall into n41 receive band |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.16.1.4	∆TIB,c and ∆RIB,c values

Already captured in fallbacks.

5.16.1.5	REFSENS requirements

The impact of the IMD9 product is found similar to that or CA_n41C-n77A with uplink CA_n41C into n77 and the MSD value is re-used.

Table 5.16.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n78 | n41 | N/A | 10 | N/A | 2500 | 2.7 | TDD | IMD9 |
|  | n78 | 3350 3450 | 100 | 1 (RBSTART=0) 1 (RBSTART=272) | N/A | N/A | TDD | N/A |

5.16.1.6	OOB blocking exception requirements

Already captured in fallbacks.

5.16.2	Specific for 2 bands UL CA

5.16.2.1	UE co-existence studies for 2 bands UL

Table 5.16.2.1-2 lists Band n41 + Band n78 2 bands UL CA(3CC) triple beat IMD analysis for Band n41- Band n78. Note that this is TDD-TDD, so there’s no active RX during TX operation of both uplink bands

Table 5.16.2.1-2: Band n41 and Band n78 for 3CC UL IMD products

| Band / CA1 | n41 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 2496-2690 |  | 3300-3800 |  |
| fDL | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | TDD-TDD combination so no RX bands are impacted. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.16.2.2	REFSENS requirements

No impact on Refsens in inter-band ULCA for these TDD bands.

5.17	CA_n71-n78

5.17.1	Common for 1 band UL and 2 bands UL CA

5.17.1.1	Operating bands for CA

Table 5.17.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n71 | 663-698 | 617-652 | FDD |
| n78 | 3300-3800 | 3300-3800 | TDD |

5.17.1.2	Channel bandwidths per operating band for CA

Table 5.17.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n71A-n78C | CA_n78C CA_n71A-n78A CA_n71A-n78C | n71 | 5, 10, 15, 20 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 & 5 |  |

5.17.1.3	UE co-existence studies for 1 band UL

UL studies have been completed for CA_n71A-n78A, but the intra-band non-contiguous ULCA must be analyzed for impact into the other FDD band.

Table 5.17.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for headline

Table 5.17.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n78 fUL | 3300 | 3800 | Order | flow | fhigh |
| n71 fDL | 663 | 698 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 | Min2CCBW | Max2CCBW |
|  | 70 | 200 | (1-1) | 70 | 200 |
| Close to UL IMD range2 |  |  | IMD4 | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh | (2-2) | 140 | 400 |
| IMD3 | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 | 3*Min2CCBW | 3*Max2CCBW |
| (2-1) | 3100 | 4000 | (3-3) | 210 | 600 |
| IMD5 | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
| (3-2) | 2900 | 4200 | Order | flow | fhigh |
| IMD7 | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| (4-3) | 2700 | 4400 | (3-1) | 6400 | 7800 |
| IMD9 | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| (5-4) | 2500 | 4600 | (4-2) | 6200 | 8000 |
| IMD11 | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
| (6-5) | 2300 | 4800 | Order | flow | fhigh |
| IMD13 | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| (7-6) | 2100 | 5000 | (4-1) | 9700 | 11600 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPRnassumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.17.1.4	∆TIB,c and ∆RIB,c values

Not relevant.

5.17.1.5	REFSENS requirements

Not relevant.

5.17.1.6	OOB blocking exception requirements

Not relevant.

5.17.2	Specific for 2 bands UL CA

5.17.2.1	Maximum output power for inter-band CA

Not relevant.

5.17.2.2	UE co-existence studies for 2 bands UL

Table 5.17.2.2-2 lists Band n71 + Band n78 2 bands UL CA(3CC) triple beat IMD analysis for Band n71- Band n78

Table 5.17.2.2-2: Band n71 and Band n78 for 3CC UL IMD products

| Band / CA1 | n71 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 663-698 |  | 3300-3800 |  |
| fDL | 617-652 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 463-663 |  | 698-898 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

5.17.2.3	REFSENS requirements

Not relevant.

5.18	CA_n41-n104

    - 5.18.1	Common for 1 band UL and 2 bands UL CA

5.18.1.1	Operating bands for CA

Table 5.18.1.1-1:  CA band combination of band n41+n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n41 | 2496 MHz | - | 2690 MHz | 2496 MHz | - | 2690 MHz | TDD |
| n104 | 6425 MHz | - | 7125 MHz | 6425 MHz | - | 7125 MHz | TDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

5.18.1.2	Channel bandwidths per operating band for CA

Table 5.18.1.2-1: Supported bandwidths per CA band combination of band n41+n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n104A | CA_n41A-n104A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41C-n104A | CA_n41C CA_n41A-n104A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n104C | CA_n104C CA_n41A-n104A | n41 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n104 | CA_n104C_BCS0 |  |

5.18.1.3	UE co-existence studies for 1 band UL

Table 5.18.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n104.

Table 5.18.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n41 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 2496 | 4992 | 7488 | 9984 | 12480 | MSD type |
| n104 | fLow | fHigh | 2690 | 5380 | 8070 | 10760 | 13450 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n41 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| UL/DL |  | n104 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| n41 | fLow | fHigh | 7125 | 14250 | 21375 | 28500 | 35625 |  |
| DL1 | 2496 | 2690 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4992 | 5380 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7488 | 8070 | - | - | N/A | - | N/A |  |
| DL4 | 9984 | 10760 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 12480 | 13450 | - | D | N/A | N/A | N/A |  |
| Analysis |  |  | UL2 of n104 direct hit the DL5 of n41. There are no near miss n104 ULx / n41 DLy (20MHz) |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit.When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.18.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n41-n104.

Table 5.18.1.3-2: Cross-band isolation analysis

| Bands | n41 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2496 | 2690 | 6425 | 7125 |
| fDL (MHz) | 2496 | 2690 | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 100 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2396 | 2790 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2296 | 2890 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2196 | 2990 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2096 | 3090 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1996 | 3190 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n41 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n41 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There are no cross-band isolation problem for n104 UL to n41 DL up to ACLR 5 according to the calculation. Moreover, n41 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.18.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA_n104C may occur for CA_ n41-n104

Table 5.18.1.3-3: Intra-band UL CA_n104 IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n104 fUL | 6425 | 7125 | Order | flow | fhigh |
| n41 fDL | 2496 | 2690 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 200 |  | 0 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 6225 | 7325 |  | 0 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 6025 | 7525 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 5825 | 7725 |  | 12650 | 14450 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 5625 | 7925 |  | 12450 | 14650 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 5425 | 8125 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 5225 | 8325 |  | 19075 | 21575 |
| Analysis | There are no IMD problem for n41 DL caused by UL CA_n104C according to the calculation. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Table 5.18.1.3-4 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA_n141C may occur for CA_ n41-n104

Table 5.18.1.3-4: Intra-band UL CA_n41 IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n104 fDL | 6425 | 7125 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 200 |  | 0 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2296 | 2890 |  | 0 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2096 | 3090 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 1896 | 3290 |  | 4792 | 5580 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1696 | 3490 |  | 4592 | 5780 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1496 | 3690 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1296 | 3890 |  | 7288 | 8270 |
| Analysis | There are no IMD problem for n104 DL caused by UL CA_n41C according to the calculation. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.18.1.4	∆TIB,c and ∆RIB,c values

For CA_n41-n104, the TIB,c and RIB,c requirements are given in the tables below.

Table 5.18.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n41-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n41 the band order from left to right is n1 and n41. |  |  |

Table 5.18.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n41-n104 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.18.1.5	REFSENS requirements

There are no harmonic/cross band isolation co-existence problem according to UE co-existence study in section 5.18.1.3 except harmonic mixing UL2/DL5 MSD, so there is need to defined additional REFSEN requirements(MSD).

Considering the n41 is high band and n104 is ultra-higher band, here we use the full separate antenna RF architecture assumption for the MSD derivation. The MSD is proposed in Table 5.18.1.5-1:

Table 5.18.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n41 | 20 | 15 | 12 | 10 | 22.0 | NOTE x | UL2/DL5 |
| n104 | n41 | 20 | 15 | 12 | 100 | 11.9 | NOTE x | UL2/DL5 |
| NOTE x: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.250.1 and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

5.18.1.6	OOB blocking exception requirements

No OOB blocking issues for CA_n41-n104.

    - 5.18.2	Specific for 2 bands UL CA

5.18.2.1	Maximum output power for inter-band CA

Table 5.18.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41A-n104A | 23 | +2/-3 |

5.18.2.2	UE co-existence studies for 2 bands UL

Since both n41 and n104 are TDD band, so there is no need to check the 2UL IMD for the UE-to-UE coexistence analysis.

For CA_n41-n104, although PHS frequency range 1884.5-1915.7MHz is as protected band of band n41, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n41-n104. Thus no protected bands are needed.

5.18.2.3	REFSENS requirements

There are no IMD co-existence problem according to UE co-existence study in section 5.18.2.2, so there is no need to defined additional REFSEN requirements(MSD).

5.19	CA_n1-n8

5.19.1	Common for 1 band UL and 2 bands UL CA

5.19.1.1	Operating bands for CA

Table 5.19.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |

5.19.1.2	Channel bandwidths per operating band for CA

Table 5.19.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n8A | CA_n1A-n8A | n1 | n1 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n8 | n8 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

5.19.1.3	UE co-existence studies for 1 band UL

For CA_n1-n8, there is no need to analyse harmonics and/or harmonics mixing since it has already been done.

Table 5.19.1.3-1 summarizes frequency ranges where cross band isolation may occur for CA_n1-n8.

Table 5.19.1.3-1: Cross-band isolation analysis

| Bands | n1 |  | n8 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1920 | 1980 | 880 | 915 |
| fDL (MHz) | 2110 | 2170 | 925 | 960 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 35 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2030 | 845 | 950 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2080 | 810 | 985 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2130 | 775 | 1020 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2180 | 740 | 1055 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2230 | 705 | 1090 |
| Analysis | There is no overlap of the band n1 ACLR range with the band n8 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n8 so no cross-band MSD is needed. |  | There is no overlap of the band n8 ACLR range with the band n1 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n1 so no cross-band MSD is needed. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Based on the table above, no MSD for the cross-band overlapping from uplink n1 to n8 downlink is needed.

5.19.1.4	∆TIB,c and ∆RIB,c values

For CA_n1-n8, the TIB,c and RIB,c values have been specified.

5.19.1.5	REFSENS requirements

There are no additional MSD requirements for this band combination.

5.20	CA_n40-n71

5.20.1	Common for 1 band UL and 2 bands UL CA

5.20.1.1	Operating bands for CA

Table 5.20.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |

5.20.1.2	Channel bandwidths per operating band for CA

Table 5.20.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n71A | CA_n40A-n71A | n40 | n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

5.20.1.3	UE co-existence studies for 1 band UL

Table 5.20.1.3-1 CA_n40A-n71A UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n40 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 2300 | 4600 | 6900 | 9200 | 11500 | MSD type |
| n71 | fLow | fHigh | 2400 | 4800 | 7200 | 9600 | 12000 |  |
| DL1 | 617 | 652 | N/A | No | No | No | No | UL harmonic |
| DL23 | 1234 | 1304 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | No | No | N/A | No | N/A |  |
| DL4 | 2468 | 2608 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no UL/DL harmonics/haromic mixing issue for this combo |  |  |  |  |  |
| UL/DL |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 663 | 1326 | 1989 | 2652 | 3315 |  |
| nX | fLow | fHigh | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 2300 | 2400 | N/A | No | No | No | No | UL harmonic |
| DL23 | 4600 | 4800 | No | N/A | No | N/A | N/A | Harmonic mixing |
| DL34 | 6900 | 7200 | No | No | N/A | No | N/A |  |
| DL4 | 9200 | 9600 | No | N/A | N/A | N/A | N/A |  |
| DL54 | 11500 | 12000 | No | No | N/A | N/A | N/A |  |
| Analysis |  |  | There is no UL/DL harmonics/haromic mixing issue for this combo |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.20.1.3-2 CA_n40A-n71A UL/DL cross band isolation analysis

| Bands3 | n40 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2300 | 2400 | 663 | 698 |
| fDL (MHz) | 2300 | 2400 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 100 | 5 | 35 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2200 | 2500 | 628 | 733 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2100 | 2600 | 593 | 768 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2000 | 2700 | 558 | 803 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1900 | 2800 | 523 | 838 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1800 | 2900 | 488 | 873 |
| Analysis | There is no cross band isolation issue |  | There is no cross band isolation issue |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.20.1.4	∆TIB,c and ∆RIB,c values

For CA_n40-n71, the TIB,c and RIB,c values are given in the tables below referring to CA_n28-n40.

Table 5.20.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n40-n71 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.20.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n40-n71 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.20.1.5	REFSENS requirements

There is no MSD requirements for this combo.

5.20.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements for CA_n40-n71.

5.20.2	Specific for 2 bands UL CA

5.20.2.1	Maximum output power for inter-band CA

Table 5.20.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n40A-n71A | 23 | +2/-3 |

5.20.2.2	UE co-existence studies for 2 bands UL

Table 5.20.2.2-1 lists Band n40 + Band n71 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.20.2.2-1: Band n40 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1737 | 1602 | 2963 | 3098 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3902 | 4137 | 1074 | 904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5263 | 5498 | 3626 | 3796 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6202 | 6537 | 411 | 206 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3204 | 3474 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7563 | 7898 | 4289 | 4494 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5926 | 6196 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 252 | 8937 | 8502 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2506 | 2811 | 5874 | 5504 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4952 | 5192 | 9863 | 10298 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6589 | 6894 | 8226 | 8596 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.20.2.2-1, there is no IMD issue for CA_n40-n71

5.20.2.3	REFSENS requirements

Based on the assumption of restricted frequency range for CA_n40-n71, there is no MSD issue due to intermodulation interference.

5.21	CA_n40-n41

5.21.1	Common for 1 band UL and 2 bands UL CA

5.21.1.1	Operating bands for CA

Table 5.21.1.1-1: CA band combination CA_n40-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n40-n41 | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

5.21.1.2	Channel bandwidths per operating band for CA

Same configurations in the specification.

5.21.1.3	UE Co-existence studies

For the UL/DL harmonic, harmonic mixing and cross band isolation caused by single band n40 or single band n41, it was already analyzed in the fallback configurations of DL CA_n40A-n41A with UL single carrier or UL CA_n40A-n41A, and it can be applied for DL CA_n40A-n41C with UL single carrier or UL CA_n40A-n41C.

Due to simultaneous Rx/Tx operation is supported for TDD bands n40 and n41, thus the co-existence studies caused by UL CA_n41C should be analyzed additionally for CA_n40A-n41C, respectively, which can be found in table Table 5.21.1.3.1-1:

Table 5.21.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA of n41

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n40 fDL | 2300 | 2400 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 160 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2336 | 2850 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2176 | 3010 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2016 | 3170 |  | 4832 | 5540 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1856 | 3330 |  | 4672 | 5700 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1696 | 3490 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1536 | 3650 |  | 7328 | 8230 |
| Analysis | 3rd order, 5th order, 7th order IMD , 9th order, 11th order and 13th order caused by CA_n41C impact band n40 DL |  |  |  |  |

5.21.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n40-n41 are already defined in the spec TS38.101-1.

5.21.1.5	REFSENs requirements

According to the co-existence study, additional IMD MSD requirements caused by UL CA_n41C should be defined for CA_n40A-n41C.

In terms of the WF R4-2502955, it was agreed that the mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1.

The MSD value given in the WF R4-2502955 are shown in table 5.21.1.5-1:

Table 5.21.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 41.7 | TDD | IMD314,16,X |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=52) | 2644.8 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 35.5 | TDD | IMD314,16,X,Y |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
| NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. NOTE 16: This band is subject to IMD7 also which MSD is not specified. NOTE X: This band is subject to IMD5, IMD9, IMD11 and IMD13 also which MSD are not specified. NOTE Y: This second test point is optional. |  |  |  |  |  |  |  |  |

5.21.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

5.22	CA_n41-n79

5.22.1	Common for 1 band UL and 2 bands UL CA

5.22.1.1	Operating bands for CA

Table 5.22.1.1-1: CA band combination CA_n41-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n79 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4000 MHz | – | 5000 MHz | TDD |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? ==> YES.

5.22.1.2	Channel bandwidths per operating band for CA

Table 5.22.1.2-1: Supported bandwidths per CA band combination CA_n41-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n79A | CA_n41C-n79A | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

5.22.1.3	UE Co-existence studies

For the UL/DL harmonic, harmonic mixing and cross band isolation caused by single band n41 or single band n79, it was already analyzed in the fallback configurations of DL CA_n41A-n79A with UL single carrier, and it can be applied for DL CA_n41C-n79A with UL single carrier.

Due to simultaneous Rx/Tx operation is supported for TDD bands n41 and n79, thus the co-existence studies caused by UL CA_n41C should be analyzed additionally for CA_n41C-n79A, respectively, which can be found in table Table 5.22.1.3.1-1:

Table 5.22.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA of n41

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n41 fUL | 2496 | 2690 | Order | flow | fhigh |
| n79 fDL | 4400 | 5000 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 0 | 160 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2336 | 2850 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2176 | 3010 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2016 | 3170 |  | 4832 | 5540 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6(4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1856 | 3330 |  | 4672 | 5700 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 1696 | 3490 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1536 | 3650 |  | 7328 | 8230 |
| Analysis | IMD4 and IMD6 caused by n41C would fall into n79 DL Rx. |  |  |  |  |

5.22.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n41-n79 are already defined in the spec TS38.101-1.

5.22.1.5	REFSENs requirements

According to the co-existence study, additional IMD MSD requirements caused by UL CA_n41C should be defined for CA_n41C-n79A.

In terms of the WF R4-2502955, it was agreed that the mandatory 1RB+1RB test point and Full+Full test points are captured in the current MSD table due to IMD in section 7.3A.5 of TS 38.101-1.

The MSD value given in the WF R4-2502955 are shown in table 5.22.1.5-1:

Table 5.22.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=89) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 1 (RBSTART=93) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 10.4 | TDD | IMD414,15 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2644.8 | 60 | 162 (RBSTART=0) | 2644.8 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 6 | TDD | IMD414,15,X |
| NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. NOTE 15: This band is subject to IMD6 also which MSD is not specified NOTE X: This second test point is optional. |  |  |  |  |  |  |  |  |

5.22.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

5.22.2	Specific for 2 bands UL CA

5.22.2.1	Maximum output power for inter-band CA

Table 5.22.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41C-n79A | 23 | +2/-3 |

5.22.2.2	UE co-existence studies

Due to both band n41 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n41A-n79C since the Tx and Rx for either n41 or n79 will not be operation at the same time.

5.22.2.3	REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

5.23	CA_n2-n77

5.23.1	Common for 1 band UL and 2 bands UL CA

5.23.1.1	Operating bands for CA

Table 5.23.1.1-1: CA band combination of band n2+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n2 | 1850-1910 | 1930-1990 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.23.1.2	Channel bandwidths per operating band for CA

Table 5.23.1.2-1: Supported bandwidths per CA band combination of band n2+n77

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n2A-n77C | n778, 9 CA_n77C CA_n2A-n77A8 CA_n2A-n77C | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n2(2A)-n77C | n778, 9 CA_n77C CA_n2A-n77A CA_n2A-n77C | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |

Note that the single uplink carrier and UL configuration with note 8 or 9 are already supported in the specification.

5.23.1.3	UE co-existence studies for 1 band UL

Table 5.23.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n2-n77.

Table 5.23.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n2 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n77 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 3300 | 4200 | N/A | D | - | - | - | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL2/DL1 |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n2 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 1930 | 1990 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3860 | 3980 | D | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5790 | 5970 | - | - | N/A | - | N/A |  |
| DL4 | 7720 | 7960 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9650 | 9950 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL1/DL2 |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

The requirements are already found in the specification.

Table 5.23.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n2-n77.

Table 5.23.1.3-2: Cross-band isolation analysis

| Bands | n2 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1850 | 1910 | 3300 | 4200 |
| fDL (MHz) | 1930 | 1990 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 40 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 2800 | 4700 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.23.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for n77C

Table 5.23.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n2 fDL | 1930 | 1990 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 20 | 200 |  | 20 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 40 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 3100 | 4400 |  | 60 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2900 | 4600 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2700 | 4800 |  | 6400 | 8600 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 2500 | 5000 |  | 6200 | 8800 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 2300 | 5200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 2100 | 5400 |  | 9700 | 12800 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.23.1.4	∆TIB,c and ∆RIB,c values

For CA_n2-n77, the ∆TIB,c and ∆RIB,c values have already been specified.

5.23.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

5.23.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

5.23.2	Specific for 2 bands UL CA

5.23.2.1	Maximum output power for inter-band CA

Table 5.23.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n77C | 23 | +2/-3 |

5.23.2.2	UE co-existence studies for 2 bands UL

Table 5.23.2.2-1 lists Band n2 + Band n77 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.23.2.2-1: Band n2 and Band n77 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 - 2350 |  | 5150 - 6110 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 500 - 520 |  | 4690 - 6550 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 - 8020 |  | 8450 - 10310 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1350 - 2430 |  | 7990 - 10750 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2780 - 4700 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 - 9930 |  | 11750 - 14510 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10300 - 12220 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11290 - 14950 |  | 3200 - 4340 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6080 - 8900 |  | 870 - 2850 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 - 18710 |  | 10700 - 11840 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 - 16420 |  | 12150 - 14130 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.23.2.2-2 lists Band n2 + Band n77 2 bands UL CA(3CC) triple beat IMD analysis for Band n2- Band n77.

Table 5.23.2.2-2: Band n2 and Band n77 for 3CC UL IMD products

| Band / CA1 | n2 |  | CA_n77C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1850-1910 |  | 3300-4200 |  |
| fDL | 1930-1990 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1650-1850 |  | 1910-2110 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Based on Table 5.23.2.2-2: The bands are not of the same or adjacent band groups, so NOTE 1 applies.

5.23.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.24	CA_n5-n77

5.24.1	Common for 1 band UL and 2 bands UL CA

5.24.1.1	Operating bands for CA

Table 5.24.1.1-1: CA band combination of band n5+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824-849 | 869-894 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.24.1.2	Channel bandwidths per operating band for CA

Table 5.24.1.2-1: Supported bandwidths per CA band combination of band n5+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n77C | CA_n77C CA_n5A-n77A CA_n5A-n77C | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n5B-n77C | CA_n5B CA_n77C CA_n5A-n77A CA_n5A-n77C | n5 | CA_n5B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |

5.24.1.3	UE co-existence studies for 1 band UL

Table 5.24.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n77.

Table 5.24.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n5 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n77 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 3300 | 4200 | N/A | - | - | D | D | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL4/DL1, UL5/DL1 |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n5 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 869 | 894 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1738 | 1788 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2607 | 2682 | - | - | N/A | - | N/A |  |
| DL4 | 3476 | 3576 | D | N/A | N/A | N/A | N/A |  |
| DL54 | 4345 | 4470 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL1/DL4 |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

The requirements are already found in the specification.

Table 5.24.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n5-n77.

Table 5.24.1.3-2: Cross-band isolation analysis

| Bands | n5 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 824 | 849 | 3300 | 4200 |
| fDL (MHz) | 869 | 894 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 25 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 799 | 874 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 774 | 899 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 749 | 924 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 724 | 949 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 699 | 974 | 2800 | 4700 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.24.1.3-3 summarizes frequency ranges where IMD products caused by n77C UL band with 2CC intra-band UL CA may occur for n77C

Table 5.24.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n5 fDL | 869 | 894 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 20 | 200 |  | 20 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 40 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 3100 | 4400 |  | 60 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2900 | 4600 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2700 | 4800 |  | 6400 | 8600 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 2500 | 5000 |  | 6200 | 8800 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 2300 | 5200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 2100 | 5400 |  | 9700 | 12800 |
| Analysis | No issues found |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs:- The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.24.1.4	∆TIB,c and ∆RIB,c values

For CA_n5-n77, the ∆TIB,c and ∆RIB,c values have already been specified.

5.24.1.5	REFSENS requirements

There is no need for new REFSENS requirements.

5.24.1.6	OOB blocking exception requirements

There is no need for new OOB blocking exception.

5.24.2	Specific for 2 bands UL CA

5.24.2.1	Maximum output power for inter-band CA

Table 5.24.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n77C | 23 | +2/-3 |

5.24.2.2	UE co-existence studies for 2 bands UL

Table 5.24.2.2-1 lists Band n5 + Band n77 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.24.2.2-1: Band n5 and Band n77 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 - 3376 |  | 4124 - 5049 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1602 - 2552 |  | 5751 - 7576 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 - 5898 |  | 7424 - 9249 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 753 - 1728 |  | 9051 - 11776 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 4902 - 6752 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5772 - 6747 |  | 10724 - 13449 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 8248 - 10098 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12351 - 15976 |  | 96 - 904 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8202 - 10952 |  | 4053 - 5928 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 - 17649 |  | 6596 - 7596 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 - 14298 |  | 9072 - 10947 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD rangeshall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.24.2.2-2 lists Band n5 + Band n77 2 bands UL CA(3CC) triple beat IMD analysis for Band n5- Band n77

Table 5.24.2.2-2: Band n5 and Band n77 for 3CC UL IMD products

| Band / CA1 | n5 |  | CA_n77C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 824-849 |  | 3300-4200 |  |
| fDL | 869-894 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 624-824 |  | 849-1049 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Based on Table 5.24.2.2-2:

The bands are not of the same or adjacent band groups, so NOTE 1 applies.

5.24.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.25	CA_n66-n77

5.25.1	Common for 1 band UL and 2 bands UL CA

5.25.1.1	Operating bands for CA

Table 5.25.1.1-1: CA band combination of band n66+n77

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n66 | 1710-1780 | 2110-2200 | FDD |
| n77 | 3300-4200 | 3300-4200 | TDD |

5.25.1.2	Channel bandwidths per operating band for CA

Table 5.25.1.2-1: Supported bandwidths per CA band combination of band n66+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n77C | CA_n77C CA_n66A-n77A CA_n66A-n77C | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n66(2A)-n77C | CA_n77C CA_n66A-n77A CA_n66A-n77C | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |

5.25.1.3	UE co-existence studies for 1 band UL

Table 5.25.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n66-n77.

Table 5.25.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n66 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n77 | fLow | fHigh | 1780 | 3560 | 5340 | 7120 | 8900 |  |
| DL1 | 3300 | 4200 | N/A | D | - | - | - | UL harmonic |
| DL23 | 6600 | 8400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | - | - | N/A | - | N/A |  |
| DL4 | 13200 | 16800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL2/DL1 |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n66 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 2110 | 2200 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4220 | 4400 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 6330 | 6600 | - | D | N/A | - | N/A |  |
| DL4 | 8440 | 8800 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 10550 | 11000 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is an issue found at UL2/DL3 |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hitnWhen the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

The requirements are already found in the specification.

Table 5.25.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n66-n77.

Table 5.25.1.3-2: Cross-band isolation analysis

| Bands | n66 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1710 | 1780 | 3300 | 4200 |
| fDL (MHz) | 2110 | 2200 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 45 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1665 | 1825 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1620 | 1870 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1575 | 1915 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1530 | 1960 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1485 | 2005 | 2800 | 4700 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.25.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for n77C.

Table 5.25.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n66 fDL | 2110 | 2200 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 20 | 200 |  | 20 | 200 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 40 | 400 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 3100 | 4400 |  | 60 | 600 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2900 | 4600 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2700 | 4800 |  | 6400 | 8600 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 2500 | 5000 |  | 6200 | 8800 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 2300 | 5200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 2100 | 5400 |  | 9700 | 12800 |
| Analysis | There is an issue of IMD13 into n66, but Note 2 below applies |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.25.1.4	∆TIB,c and ∆RIB,c values

For CA_n66-n77, the ∆TIB,c and ∆RIB,c values have already been specified.

5.25.1.5	REFSENS requirements

There is IMD13 falling inside n66. However, Note 2 applies, meaning no MSD is needed specified.

5.25.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

5.25.2	Specific for 2 bands UL CA

5.25.2.1	Maximum output power for inter-band CA

Table 5.25.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n66A-n77C | 23 | +2/-3 |

5.25.2.2	UE co-existence studies for 2 bands UL

Table 5.25.2.2-1 lists Band n66 + Band n77 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.25.2.2-1: Band n66 and Band n77 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 - 2490 |  | 5010 - 5980 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 260 - 780 |  | 4820 - 6690 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 - 7760 |  | 8310 - 10180 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 930 - 2040 |  | 8120 - 10890 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3040 - 4980 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 - 9540 |  | 11610 - 14380 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 10020 - 11960 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11420 - 15090 |  | 2640 - 3820 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6340 - 9180 |  | 1260 - 3270 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 - 18580 |  | 10140 - 11320 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 - 16160 |  | 11730 - 13740 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The requirements are already found in the specification.

Table 5.25.2.2-2 lists Band n66 + Band n77 2 bands UL CA(3CC) triple beat IMD analysis for Band n66- Band n77

Table 5.25.2.2-2: Band n66 and Band n77 for 3CC UL IMD products

| Band / CA1 | n66 |  | CA_n77C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 1710-1780 |  | 3300-4200 |  |
| fDL | 2110-2200 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510-1710 |  | 1780-1980 |  |
| Analysis | There is an issue of IMD3, but no requirements, since the bands are not in same or adjacent bandgroups |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Based on Table 5.25.2.2-2:

-	The bands are not of the same or adjacent band groups, so NOTE 1 applies.

5.25.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

5.26	CA_n18-n77

5.26.1	Common for 1 band UL and 2 bands UL CA

5.26.1.1	Operating bands for CA

Table 5.26.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.26.1.2	Channel bandwidths per operating band for CA

Unaffected parts are omitted.

5.26.1.3	UE co-existence studies for 1 band UL

Unaffected parts are omitted.

Table 5.26.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.26.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n18 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 815 | 1630 | 2445 | 3260 | 4075 |  |
| n77 | fLow | fHigh | 830 | 1660 | 2490 | 3320 | 4150 |  |
| DL1 | 3300 | 4200 | N/A |  |  | D | D | UL harmonic |
| DL23 | 6600 | 8400 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 |  |  | N/A |  | N/A |  |
| DL4 | 13200 | 16800 |  | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | The two direct hits from the band n18 to the band n77: the 4th and the 5th harmonic to the band n77, have been specified in Rel-17 and hence can be omitted. |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n18 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 860 | 875 | N/A |  |  |  |  | UL harmonic |
| DL23 | 1720 | 1750 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 2580 | 2625 |  |  | N/A |  | N/A |  |
| DL4 | 3440 | 3500 | D | N/A | N/A | N/A | N/A |  |
| DL54 | 4300 | 4375 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a harmonic mixing direct hit case, i.e., UL4/DL1. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

5.26.1.4	∆TIB,c and ∆RIB,c values

TIB,c and RIB,c values have already been defined so this section is omitted.

5.26.1.5	REFSENS requirements

Reference sensitivity exceptions due to the two direct harmonic hits from UL n18 to DL n77 have been specified in Rel-17 and hence are omitted.

Reference sensitivity exceptions due to harmonic mixing from UL n77 for DL CA_n18-n77 is shown in Table 5.26.1.5.

Table 5.26.1.5 (for Table 7.3A.4-4): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4X |
| n77 | n18 | 10 | 15 | 25 | 15 | 2.5 | NOTE 8 | UL1/DL4X |
| Unaffected part (NOTE 8) is omitted. NOTE X: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

5.26.1.6	OOB blocking exception requirements

OOB exception for this CA combination has already been defined in Rel-17, so this section is omitted.

6	Dual Connectivity: Specific Band Combination Part

6.x	DC_nX-nY

Editor's note: The texts for NR DC can only be added together with the texts for the corresponding inter-band 2 bands UL NR CA above, which means TP to TR to include pure NR DC configuration is not allowed.

6.x.1	Configurations for DC_nX-nY

Table 6.x.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |
| DC_nX(A/B/C(2A)-nY(A/B/C(2A) | DC_nXA/B/C-nYA |

6.x.2	Maximum output power for NR-DC

Table 6.x.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_nXA-nYA | 23 | +x/-y |
| DC_nXA/B/C-nYA | 23 | +x/-y |

Annex A:
Band group definition

For the following MSD issues detection, a criterion applies in terms of band group as defined in Table A.1:

-	For 2DL 1UL/1CC cross-band isolation, and for the Tx noise floor MSD to be considered, the two bands should be part of the same or adjacent band group.

-	For 2DL 2UL/3CC triple beat MSD to be considered, the two bands should be part of the same or adjacent band group.

Table A.1: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex B:
Valid UL configurations

For CA_nX-nY two band DL inter band combinations, the following UL configurations are applicable:

-	One band UL with one CC: nXA, nYA

-	One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A)

-	two band UL with one CC per band: CA_nXA-nYA

-	two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

Annex C:	Void

Annex D:	Void

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411938 |  |  |  | TR skeleton | 0.0.0 |
| 2024-08 | RAN4 #112 | R4-2411939 |  |  |  | Some updates in the tables 5.x.1.3-1/2/3 and Table 5.x.2.2-1/2 for the skeletion. Including the following approved TP in RAN4 #112 meeting: R4-2414326 TP for TR 38.719-02-01: PC3 NR BCS4/5 inter-band CA R4-2414328 TP for TR38.719-02-01_CA_n3A-n104A R4-2414329 TP for TR38.719-02-01_CA_n8A-n104A R4-2414330 TP to TR 38.719-02-01 adding CA_n78(2A) UL to CA_n26A-n78(2A) and CA_n26(2A)-n78(2A) DL R4-2414336 TP to TR 38.719-02-01 Addition of CA_n1A-n71A R4-2414337 TP to TR 38.719-02-01 Addition of CA_n3A-n71A R4-2414338 TP to TR 38.719-02-01 Addition of CA_n20A_n77A | 0.1.0 |
| 2024-10 | RAN4 #112bis | R4-2415921 |  |  |  | Including the following approved TP in RAN4 #112bis meeting: R4-2414989 TP for TR 38.719-02-01 CA_n7-n29 R4-2417124 TP for TR 38.719-02-01 CA_n29-n77 R4-2417127 TP to TR 38.719-02-01 CA_n20-n41 R4-2417128 TP to TR 38.719-02-01 CA_n20-n71 R4-2415417 TP to TR 38.719-02-01 CA_n41-n78 w ULCA R4-2417130 TP to TR 38.719-02-01 CA_n71-n78 R4-2417131 TP for TR38.719-02-01_CA_n41A-n104A | 0.2.0 |
| 2024-11 | RAN4 #113 | R4-2418712 |  |  |  | Including the following approved TP in RAN4 #113 meeting: R4-2418710 TP for TR38.719-02-01_CA_n3A-n104C R4-2420415 draft TP to TR 38.719-02-01 include CA_n1A-n8A | 0.3.0 |
| 2025-02 | RAN4 #114 | R4-2500967 |  |  |  | Including the following approved TP in RAN4 #114 meeting: R4-2500953 TP for TR38.719-02-01_CA_n8A-n104C R4-2500954 TP for TR38.719-02-01_Intra-band UL CA for CA_n41-n104 | 0.4.0 |
| 2025-04 | RAN4 #114bis | R4-2503981 |  |  |  | Including the following approved TP in RAN4 #114bis meeting: R4-2505163 TP for TR 38.719-02-01 on introduction of CA_n40A-n71A R4-2503975 TP for TR38.719-02-01_CA_n40A-n41C R4-2503976 TP for TR38.719-02-01_CA_n41C-n79A R4-2504133 TP to TR 38.719-02-01 CA_n2-n5 R4-2505167 TP to TR 38.719-02-01 CA_n2-n48 R4-2505168 TP to TR 38.719-02-01 CA_n2-n77 R4-2505169 TP to TR 38.719-02-01 CA_n5-n48 R4-2505170 TP to TR 38.719-02-01 CA_n5-n77 R4-2505171 TP to TR 38.719-02-01 CA_n48-n66 R4-2505172 TP to TR 38.719-02-01 CA_n66-n77 R4-2504492 TP for TR 38.719-02-01 to correct CA_n20-n77 | 0.5.0 |
| 2025-05 | RAN4 #115 | R4-2506397 |  |  |  | Correct the formatting and header issues in Tables as well as the text | 0.5.1 |
| 2025-05 | RAN4 #115 | R4-2506398 |  |  |  | Including the following approved TP in RAN4 #115 meeting: R4-2507985 DraftCR to 38.719-02-01 to correct analysis for CA_n66-n77 R4-2507973 TP to TR 38.719-02-01 for adding impact of harmonic mixing to CA_n18-n77 with UL n77 R4-2506393 TP for TR38.719-02-01_UL CA_n3A-n104C R4-2506394 TP for TR38.719-02-01_UL CA_n8A-n104C R4-2506395 TP for TR38.719-02-01_Correct the harmonic MSD test points for CA_n3-n104 and n20-n41 | 0.6.0 |