3GPP TSG-RAN WG4 Meeting #115	   	R4-2506411

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.12.5

Source: 	CMCC

Title:			  Discussion on RRM core requirements for R19 ATG

Document for:	Discussion

- Introductions

In last meeting, most of requirements have been discussed and defined for Air-to-ground network enhancement, the WF has been approved [1]. In this contribution, we further discuss UE capability issues and potential enhancement considering ATG specific CA scenario.

- Discussion

Issue 1-1-2: UE Beam Type capability signaling

Issue 1-1-2-1: The description of capability definition

Agreement:

- Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC.

  - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination.

Issue 1-1-2-2: The Report granularity of UE capability

FFS:

- Per BC

Issue 1-1-2-3: The Report feature

Agreement:

- Conditional mandatory

  - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band [of the supported Band combination]

Issue 1-1-2-4: The Report functionalities between FDD and TDD

Agreement:

- No functionality difference between FDD and TDD

- FFS how to reflect it on the spec.

Issue 1-1-2-5: The Report functionalities between FR1 and FR2

Agreement:

- FR1 only

In last meeting, RF session agreed that Case 4 UE implementation is possible for CA_n3-n39. Meanwhile, for other BCs which are not introduced in R19, it is common understanding that whether UE could support common Rx beam type for inter-band CA is highly depending on frequency range of operating BCs. To make this UE capability more generic, the report granularity of this UE capability shall be per BC.

Proposal 1: The report granularity shall be per BC.

For the report functionalities between FDD and TDD, after checking the spec, if there is no functionality difference between FDD and TDD, ‘No’ shall be used.

Proposal 2: ‘No’ shall be indicated by UE for the field ‘Need of FDD/TDD differentiation’.

Based on the agreements and above discussion, we propose following UE feature, and this shall be captured in R19 NR UE feature table.

Proposal 3: Define the UE feature of Rx beam type as following

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_ATG_enh | 50-1 | common Rx beam | Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

Issue 1-2-2: Interruption requirements

- Proposals:

  - Proposal 1:

    - For the interruptions caused by SCell addition/release and any types of SCell activation/deactivation, distinguish the interruption requirements for Category 1 and 2.

      - Category 1: antenna array assumption in intra-band CA and Case 4 in inter-band CA

      - Category 2: other cases

    - For other interruptions, define unified interruption requirements for all UE antenna types.

- In current spec, shorter interruption requirement is expected when two Rx chain will be used between the active serving cells and the SCells being added/released/activated/de-activated, longer interruption requirement is expected when one Rx chain will be shared between the active serving cells and the SCells being added/released/activated/de-activated.

For ATG UE capable of inter-band CA, including Case 4 and Case 5, when RAN4 define the requirements, such as MRTD, the working assumption is that UE is capable of two separate Rx chains. Follow the legacy methodology, shorter interruption requirement shall be used.

Proposal 4: Regardless UE Rx beam type, if the active serving cell and the SCell being added/released or activated/de-activated are in a FR1 band pair, the interruption requirement for inter-band CA shall be used.

Issue 1-2-3: SCell activation delay enhancement

- Proposals

  - Proposal 1: If the target SCell is known by the UE and frequency ranges of both bands are less than or equivalent to X MHz with common beam management the Tactivation_time can be 3ms

  - Proposal 2: if the target SCell is unknown, frequency ranges of both bands are less than or equal to X MHz Tactivation_time is 3ms+TFirstSSB+ 2ms.

In legacy, except for SSB-less scenario, for intra-band CA known case, the sync time in SCell activation delay requirement is needed. For SSB-less intra-band CA scenario, 3ms can be used at least provided that RTD is within ±260ns and RPD is within 6 dB. For inter-band CA known case, the sync time in activation delay requirement is always needed.

For ATG intra-band CA, RTD within ±260ns and RPD within 6 dB can not be always guaranteed, therefore, the sync time is needed. For ATG inter-band CA, even through we consider RTD only including TAE 3µs, the PCell and SCell can not share the sync information due to 3µs is longer than CP/2. Therefore, the sync time in SCell activation delay requirement is always needed.

Hence, the delay enhancement proposed in proposal 1 and proposal 2 can not be applied in ATG scenario.

Proposal 5: Reuse the legacy SCell activation delay requirement for ATG.

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement

- Proposal 1:

  - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

    - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

In legacy FR2, Klayer1_measurement =1 when the L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured. This was introduced from R15, and only FR2-1 SCS up to 240kHz was considered at that time.

In our understanding, the location of SSB symbols and the RSSI symbols of intra-frequency neighbour cells are determined by serving cell timing and the configured SSB-ToMeasure and SS-RSSI-Measurement. Thus, 1 symbol margin is considered including the maximum propagation delay difference and sync margin between serving cell and neighbour cells. However, such understanding shall be firstly aligned among RAN4 group.

Proposal 6: Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement.

If serving cell timing is used as the reference timing, then this margin shall consider the maximum propagation delay difference and sync margin between serving cell and neighbour cells. For the ATG scenario, the maximum propagation delay difference is about 1ms if we consider 300km ISD.

If neighbour cell timing is used as the reference timing, then 1 symbol margin is enough to cover sync margin between cells.

Proposal 7: If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells.

Proposal 8: If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin.

Issue 1-3-5: Measurement enhancement for ATG UE with antenna array

Candidate options:

- UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control.

  - Option 1: this can already be achieved by the existing configuration.

  - Option 2: Consider enhancement for Rel-19 ATG CA

In last meeting, we listed several cases that the neighbour cell measurement on SCC and/or CC with deactivated SCell can be skipped.

- Case 1: Network could derive the coarse neighbour cell’s quality on the carrier frequency of SCC by neighbour cell’s quality on the carrier frequency of PCC.

- Case 2: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

- Case 3: UE is in the cell center area

We also analyzed the benefit of such neighbour cell measurement skipping, mainly about the throughput gain, due to following:

- Before SCell activation, when UE performs the neighbour cell measurement on CC with deactivated SCell, the scheduling restriction cased by antenna array will apply to PCell for Case#4 UE, throughput loss will be caused.

- After SCell activation, when UE performs the neighbour cell measurement on SCC (intra-frequency measurement), the scheduling restriction cased by antenna array will apply to SCell for UE with antenna array on SCC (Case#2, Case#4 and Case#5), and additional scheduling restriction will apply to PCell for Case#4 UE, throughput loss will be caused.

In existing specification, the only method to control neighbour cell measurement is to configure s-MeasureConfig, which is the threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. However, this methodology applies for all carrier frequencies of configured  MO.

2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 3> if a measurement gap configuration is setup, or 3> if the UE does not require measurement gaps to perform the concerned measurements: 4> if s-MeasureConfig is not configured, or 4> if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or 4> if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP: 5> if the measObject is associated to NR and the rsType is set to csi-rs: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3; 5> if the measObject is associated to NR and the rsType is set to ssb: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

Such existing configuration can not fulfill the ATG demand that neighbour cell measurement can be skipped only for SCC and/or CC with deactivated SCell. Therefore, enhancement for R19 ATG shall be considered.

Observation 1: The existing network configuration can not achieve the functionality that UE only skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell}.

We propose to achieve such feature by network configuration. If network identify neighbour cell measurement on SCC is not needed by deployment, or by the measurement report from PCell, then network can configure a signal which indicates UE is not required to measure the neighbour cell for the configured MO for SCC.

Proposal 9: Introduce a network signaling to indicate that the UE is not required to measure the neighbour cell on the configured MO for SCC. LS shall be sent to RAN2 for signaling design.

If neighbour cell measurement will be skipped, then the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed at all, the measurement delay requirement and scheduling restriction of omni-directional antenna UE can be reused. Throughput can be increased without UE consuming additional power.

Proposal 10: From requirement point of view, if UE skip the neighbour cell measurement of SCC and/or CC with deactivated SCell, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed, the scheduling restriction caused by antenna array will not be applied.

SCell dormancy impact on Active BWP switch delay

In Rel-18, the active BWP switch delay of single CC was introduced, but the SCell dormancy related requirements was precluded due to signal carrier scenario in Rel-18. Now, since CA was supported in R19, and the SCell dormancy function also supported by ATG, the related requirements shall be included in the active BWP switch delay of single CC requirements, the legacy TN requirement can be reused.

Proposal 11: Introduce SCell dormancy related impact for active BWP switch delay of single CC requirement. The legacy TN requirement can be reused.

- Conclusion

In this contribution, we further discuss the UE capability, Case#4 related issue and further enhancement of ATG measurement. The observation and proposals are:

Proposal 1: The report granularity shall be per BC.

Proposal 2: ‘No’ shall be indicated by UE for the field ‘Need of FDD/TDD differentiation’.

Proposal 3: Define the UE feature of Rx beam type as following

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_ATG_enh | 50-1 | common Rx beam | Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

Proposal 4: Regardless UE Rx beam type, if the active serving cell and the SCell being added/released or activated/de-activated are in a FR1 band pair, the interruption requirement for inter-band CA shall be used.

Proposal 5: Reuse the legacy SCell activation delay requirement for ATG.

Proposal 6: Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement.

Proposal 7: If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells.

Proposal 8: If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin.

Observation 1: The existing network configuration can not achieve the functionality that UE only skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell}.

Proposal 9: Introduce a network signaling to indicate that the UE is not required to measure the neighbour cell on the configured MO for SCC. LS shall be sent to RAN2 for signaling design.

Proposal 10: From requirement point of view, if UE skip the neighbour cell measurement of SCC and/or CC with deactivated SCell, the beam sweeping factor N1 and antenna array specific L1/L3 sharing factor is not needed, the scheduling restriction caused by antenna array will not be applied.

Proposal 11: Introduce SCell dormancy related impact for active BWP switch delay of single CC requirement. The legacy TN requirement can be reused.

- Reference

- R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC