3GPP TSG-RAN WG4 Meeting#115     		R4-2506420

# Malta, MT, 19th - 23rd May, 2025

Agenda Item:	7.17.4

Source: 	CMCC

Title: 			 Simulation results for BS interference suppressing with MMSE-IRC

Document for:	Discussion

- Introduction

Based on the simulation assumption approved in last meeting[1], we give our simulation results.

- Simulation results

PUSCH requirements with interference (CP-OFDM/FDD, 5MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 11.32 | 12.13 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 7.47 | 8.13 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 4.11 | 4.27 |

PUSCH requirements with interference (CP-OFDM/FDD, 20MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 11.04 | 12.07 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 7.39 | 8.21 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 4.09 | 4.32 |

PUSCH requirements with interference (CP-OFDM/TDD, 10MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 11.78 | 13.22 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 8.14 | 9.15 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 4.7 | 5.24 |

PUSCH requirements with interference (CP-OFDM/TDD, 100MHz)

|  |  |  |  | SNR for HomoNet | SNR for HetNet |
| --- | --- | --- | --- | --- | --- |
| 1T2R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 11.66 | 12.87 |
| 1T4R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 8.18 | 9.21 |
| 1T8R | MCS 16 | TDLA30-10 for HetNet TDLC300-100 for HomNet | MMSE-IRC | 4.7 | 5.27 |

- Simulation assumption

PUSCH requirements with interference

Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD, FDD |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  | TDD: 30kHz/10MHz, 100MHz FDD: 15kHz/5MHz, 20MHz |
| Transmit precoding |  | Disabled |
| Antenna configuration |  | 1x2 (1 interferes) 1x4 (2 interferes) 1x8 (2 interferes) |
| Propagation condition |  | For serving cell: TDLA30-10 HetNet and TDLC300-100 for HomNet For Interference cell: TDLC300-100 |
| MCS |  | 2R: MCS 16 4R: MCS 16 8R: MCS 16 |
| PUSCH configuration | Mapping type | Type A/B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | DM-RS sequence generation | nSCID =0 for all cells Different NID0 for serving and interference cells NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively For DFT-S-OFDM: group hopping and sequence hopping are disabled |
| Maximum HARQ transmissions |  | 4 |
| Redundancy version coding sequence |  | {0,2,3,1} |
| Test metric |  | SNR@70% max TP |

| Physical cell ID | Physical cell ID of 0 for the serving cell, and cell ID 1 for interferer 1, cell ID 2 for interferer 2 |
| --- | --- |
| INR values for Homogeneous deployment | INRs 5.35 dB in case of 1 interference cells INRs 7.28, -2.52 dB in case of 2 interference cells |
| INR values for Heterogeneous deployment | INRs 9.82 dB in case of 1 interference cells INRs 12.38, -0.97 dB in case of 2 interference cells |
| Modulation order of interfering PUSCH | 16QAM randomly modulated symbols |
| Interference cell modeling | Interfering cell has same channel model as for serving cell and it is independently generated for each cell. |

- Reference

[1] R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telecom