3GPP TSG-RAN WG4 Meeting #115				R4-2506437

Malta, MT, 19th – 23rd May, 2025

Agenda item:	8

Source: 	CMCC

Title: 	Rel-19 NR_ATG_enh input to UE feature list

Document for:	Discussion

- Discussion

After the discussion, Rel-19 NR_ATG_enh WI agreed to introduce the UE capability for the Beam Type when UE capable of antennaArrayType-r18 on both PCC and SCC in inter-band CA, the current progress is as follows [1]:

Issue 1-1-2: UE Beam Type capability signaling Issue 1-1-2-1: The description of capability definition Agreement: - Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. Issue 1-1-2-2: The Report granularity of UE capability FFS: - Per BC Issue 1-1-2-3: The Report feature Agreement: - Conditional mandatory - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band [of the supported Band combination] Issue 1-1-2-4: The Report functionalities between FDD and TDD Agreement: - No functionality difference between FDD and TDD - FFS how to reflect it on the spec. Issue 1-1-2-5: The Report functionalities between FR1 and FR2 Agreement: - FR1 only

Based on the current progress, the UE feature related shall be involved in the Rel-19 UE feature list, we provide the detail information in the following table:

- NR_ATG_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_ATG_enh | 50-1 | common Rx beam | Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | [Per BC] | [No] | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band [of the supported Band combination] |

To be noted, the UE feature reporting type, FDD/TDD differentiation, and the Note are still under discussion in ATG RRM core session. The table will be updated upon the final agreement related.

Proposal 1: Input the Rel-19 ATG UE feature ‘Common Rx beam’ to Rel-19 UE feature list.

- Conclusion

Proposal 1: Input the Rel-19 ATG UE feature ‘Common Rx beam’ to Rel-19 UE feature list.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_ATG_enh | 50-1 | common Rx beam | Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | [Per BC] | [No] | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

- Reference

[1] R4-2504898, WF on RRM requirements for NR_ATG_enh, CMCC