3GPP TSG-RAN WG4 Meeting #115	R4-2506438

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on UE CA Demodulation performance requirements for LessThan 5MHz

Agenda item:	7.3.5

Document for:	Discussion

Introduction

In RAN4#114bis it was discussed if performance requirements are to be introduced for Lessthan 5Mhz in CA configuration. It was not clarified if PDSCH CA requirements are to be introduced, however the configuration used for providing simulation results were agreed and captured in [1].

In the following we will discuss the remaining open issues.

Discussion

PDSCH CA demodulation requirements for 3MHz CHBW

Whether to define PDSCH CA demodulation requirements for 3MHz CHBW

In RAN4#114bis it was discussed weather to define PDSCH CA demodulation requirements for 3MHz or not (see [1])

Whether to define PDSCH CA demodulation requirements for 3MHz CHBW - Candidate options: - Option 1: Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements - Option 2: Do not introduce new PDSCH requirements for <5MHz channels - Way Forward: - Keep open and this issue will be concluded in the next meeting. - In parallel with this discussion, companies are encouraged to bring simulation results in the next meeting.

When 3MHz CBW was introduced, there was also introduced a specific band (n106) which only supports 3MHz. Due to this, it was discussed if a 3MHz only UE would exist as that would require specific requirements made for 3MHz.

In Rel-18 discussion however, it was assumed based on UE/chipset vendor feedback, that no UE supporting only 3MHz were expected in the future, hence no requirements were introduced as UEs will anyway tested using 10MHz CBW.

In the case a UE reports CA capability where the new n106 is possible to be selected, then it cannot anymore be assumed the UE will be tested with a higher CBW in CA.

It is therefore our view, that even if there were no PDSCH performance requirements introduced in Rel-18 for 3MHz CBW, the situation for CA is potentially different.

Also, since existing tables for CA requirements contain all possible CBW also 3MHz should be introduced for completeness’s sake.

In addition, we have run the simulations for the agreed configurations and have the following results:

2Rx:

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | R.PDSCH.1-10.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 10.8 |

4Rx:

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | R.PDSCH.1-10.5 FDD | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 5.3 |

Based on our simulation results, we see no issue with introducing PDSCH CA requirements and expect the final SNR values to be close to the 5MHz SNR values.

Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements

Test applicability for PDSCH CA requirements for 3MHz CHBW

In RAN4#114bis it was discussed if the existing applicability rule can be reused in case PDSCH CA requirements are introduced (see [1]):

Test applicability for PDSCH CA requirements for 3MHz CHBW (if introduced) - Proposals: - Option 1: The existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4 can be reused. - Other options are not precluded

In PDSCH requirements are agreed to be introduced, we see no issue re-using the existing test applicability rule for PDSCH CA requirements.

Reuse the existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4.

SDR requirements for 3MHz CHBW

Whether to define SDR requirements for 3MHz CHBW

In RAN4#114bis it was also discussed if there is a need to define SDR requirements for 3MHz CBW (see [1]):

Whether to define SDR requirements for 3MHz CHBW - Candidate options: - Option 1: RAN4 to discuss whether there are SDR requirement impact for UE supporting 3MHz channel bandwidth in CA. - Option 2: Do not define new SDR requirements for <5MHz channels.

In existing SDR requirements the common parameter section in Table 5.5A-1 contains the definition of number of PDCCH candidates and aggregation levels for different SCS/CBW, said table will need to be updated including also the configuration for 3MHz. If the used number of PDCCH candidates are to be kept, the configuration for 3MHz can be similar as to 5MHz, i.e., 2/AL2.

Add the following in TS38.101-4 Table 5.5A-1 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 3MHz”

CA CQI reporting requirements for 3MHz CHBW

Whether to define CA CQI reporting requirements for 3MHz CHBW

In RAN4#114bis it was discussed if updates are needed for CA CQI requirements for 3MHz CBW (see [1]):

Whether to define CA CQI reporting requirements for 3MHz CHBW - Candidate options: - Option 1: The CA CQI test parameters need to be updated for 3MHz CC (Ericsson) - E///: AL4 for PDCCH in Table 6.2A.3.1.1-1, and no subbandSize configuration in Table 6.2A.3.1.1-2. - Other options not precluded.

Since existing CA CQI requirements are defined for 1/AL8 which is not possible for 3MHz, there is a need to add 1/AL4 for 3MHz. In addition, as mentioned in RAN4#114bis there is no sub band size for 3MHz. To ensure it is possible for UE to be tested for CA CQI reporting requirements with 3MHz we see option 1 as needed to be introduced.

Add the following in TS38.101-4: AL4 for PDCCH in Table 6.2A.3.1.1-1, and no subbandSize configuration in Table 6.2A.3.1.1-2.

Conclusion

In this paper we have provided our view on the topic adding CA UE performance requirements for the topic of Lessthan 5MHz.

PDSCH CA demodulation requirements for 3MHz CHBW

Whether to define PDSCH CA demodulation requirements for 3MHz CHBW

- Based on our simulation results, we see no issue with introducing PDSCH CA requirements and expect the final SNR values to be close to the 5MHz SNR values.

- Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements

Test applicability for PDSCH CA requirements for 3MHz CHBW

Reuse the existing test applicability rule for PDSCH CA requirements in 5.1.1.7 in TS38.101-4.

SDR requirements for 3MHz CHBW

Whether to define SDR requirements for 3MHz CHBW

Add the following in TS38.101-4 Table 5.5A-1 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 3MHz”

CA CQI reporting requirements for 3MHz CHBW

Whether to define CA CQI reporting requirements for 3MHz CHBW

Add the following in TS38.101-4: AL4 for PDCCH in Table 6.2A.3.1.1-1, and no subbandSize configuration in Table 6.2A.3.1.1-2.

- References

- R4-2504754 - Way Forward for [114bis][326] NR_less_than_5MHz_Ph2_demod

- RP-241622 - Revised WID: NR channel BW less than 5MHz for FR1 Phase 2

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)