3GPP TSG-RAN WG4 Meeting #115	R4-2506440

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.17.3

Document for:	Discussion

Introduction

In RAN4#114bis the topic of intercell and intracell inter user interference for 8Rx configuration were further discussed with focus on down selecting the testcases as well as which applicability rules to introduce.

In the following we will discuss the remaining open issues and provide our view with observations and proposals to enable further progress on the topic

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE

In RAN4#114bis the definition of the MMSE-IRC receiver capability for 8Rx UEs were discussed (see [1]):

Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors.

Agreement was made to redefine the existing receiver definitions in 38.101-4 and 38.306. The actual new definition was also agreed with one open issue indicated by “[with support of]” in the agreement as there were proposal to change the wording. We see the current wording as without the text in [] as OK since if a receiver is able to “support up to x layers” it can also be assumed that it is able to “detect up to x layers”

We do not strictly see the need to change “[with support of]” to make it clearer, however we are open to further discuss.

We are fine with removing the [] from the agreement in RAN4#114bis but are also open to discuss alternative wording.

8Rx UE MMSE-IRC receiver coverage

In RAN4#114bis the receiver coverage was discussed (see [1]):

Agreement: - Down select the [limit] to one of 0.5dB or 1.0dB. Remaining proposals: - Option 1: Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases by the agreed criteria, if 2 sets of requirements are required for at least one test case, define two sets of PDSCH requirements for all cases, which are applicable for both baseline and simplified receivers.

In our view, using only 0.5dB as [limit] is too low and would in most cases result in two sets of requirements. In addition, in RAN4 the most used margin for defining requirements for a new feature has been if the new feature shows the average of companies’ simulation results to be more than 1.0dB of improvement compared to the average of the used reference.

Set the [limit] to 1.0dB

Regarding the remaining option 1, we agree that it might seem strange with some requirements for the same requirement type to have some cases with one and some cases with two sets of requirements. We do not however see an issue where for example two sets of requirements are defined for PDSCH Inter-cell interference and only one set of requirements are defined for PDSCH intra-cell inter-user interference.

In our view, it will make sense to define requirements for both baseline and simplified receivers for all testcases of a test-type if done for one requirement of said feature. This will only apply to cases where it is at all applicable to define two sets of requirements. A test-type could for example be “PDSCH with Inter-cell Interference” or “PDSCH with Intra-cell Inter-user interference”

Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases of a test-type by the agreed criteria, if 2 sets of requirements are required for at least one test case of said test-type, define two sets of PDSCH requirements for all cases with said test type, which are applicable for both baseline and simplified receivers.

PDSCH requirements with inter cell interference

Requirement definition

Based on the latest simulation results collection in [2] we see it feasible with enough simulation results to define requirements for all simulated testcases.

It is feasible to define requirements for all simulated inter-cell interference cases from RAN4#114bis. Requirements can be defined the usual way by outlier removal and adding impairments + 0.4dB margin added to the average of impaired results.

PDSCH requirements with intra-cell inter-user interference

Requirement definition

Based on the latest simulation results collection in [2] we see it feasible with enough simulation results to define requirements for all simulated testcases.

It is feasible to define requirements for all simulated inter-cell interference cases from RAN4#114bis. Requirements can be defined the usual way by outlier removal and adding impairments + 0.4dB margin added to the average of impaired results.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm and Receiver coverage

In RAN4#114bis it was decided to postpone to this meeting if 1 or 2 sets of requirements will be defined. In addition, companies were encouraged to provide suggestions on the methodology for deciding CQI reporting requirements (see [1])

CQI calculation algorithm and Receiver coverage Agreement: - Companies are encouraged to provide suggestions on methodology for deciding CQI reporting requirements in next meeting. - Decision to have one or two sets of requirements is postponed to next meeting when more simulation results are available.

If one set of requirements are to be used, the decision to have only one set of requirements will have to be done using the same SINR value for both. The issue is, how to select the SINR value to be used for the decision. If two sets of requirements are selected, then each set of requirements can have individual SINR values. Two options seems to be relevant to consider:

- Select the SINR based on TP gain for each receiver type individually. If the selected SINR value is the same for both receivers, then check the gamma value for each receiver type individually.

- Select the SINR based on TP gain for one of the receiver types and check the gamma value for each receiver type with said SINR value.

We believe that the first option will most likely not get to the two receiver types resulting in the same SINR based on TP gain, hence the option is not so useful. The second option will however always be useful, however the TP gain will not be the same for both receives in case only one set of requirements are defined. Still we see the second option as a viable way forward.

Select the SINR based on TP gain for one of the receiver types and check the gamma value for each receiver type with said SINR value.

If the difference in gamma is below 0.1 between simplified and baseline receiver, we expect only one set of requirements are needed. Based on the difference in gamma for the existing requirements for 2Rx and 4Rx in Rel-17, it is our expectation that this will be the case.

If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined.

Applicability rule and release independency

Test applicability to skip 2/4Rx requirements

In RAN4#114bis the test applicability to skip 2/4Rx requirements were discussed (see [1]):

Test applicability to skip 2/4Rx requirements Proposals - Option 1: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 4: Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test: - UE support both 8Rx and 2/4Rx - The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement. - Option 5: RAN4 shall have the following applicability rules: - UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx. - UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

Overall, we see it relevant to discuss the different use cases individually (i.e., Inter-cell interference, Intra-cell Inter-user interference).

Inter-cell Interference:

For inter-cell interference assuming all testcases which has been discussed for 8Rx are defined as requirements, rank 2 and 4 will be covered. Since rank 2 is covered we do not see a reason for 8Rx UE which has passed rank 2 to also pass requirements with rank 1 for 4Rx as rank 2 is harder for the UE to pass compared to rank 1.

UE that passes Rank2 ICI test case with 8Rx can skip Rank1 ICI requirements with 4Rx.

Intra-cell Inter-user interference:

For 4Rx UEs, requirements are defined with Rank 1+1 and 2+2. Since it has been agreed in RAN4#114bis to not include rank 1+1 nor rank 4+4 for 8Rx UEs, it means that if applicability rules are introduced to skip 2Rx/4Rx tests if 8Rx tests are passed, only one requirement will remain (i.e., rank 2+2), hence we see it relevant for 8Rx UEs to pass rank 1+1 requirements if the 8Rx UE also supports 2Rx/4Rx.

Do not introduce applicability rules for 8Rx UE to skip 4Rx Intra-cell Inter-user interference requirements.

Release independence

In RAN4#114bis the question to introduce release independency was discussed and it was agreed that the new Rel-19 requirements shall at least be release independent from Rel-18. It was however still kept open, if the requirements shall also be release independent from Rel-17 [1]:

Release independency Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18. Remaining proposals: - Option 1: The new Rel-19 requirements shall also be release independent from Rel-17. - Option 2: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

With relation to introducing release independence from Rel-17 we observe the following:

- The 2Rx and 4Rx with inter-cell and intra-cell inter-user interference requirements are defined in Rel-17, hence a Rel-17 UE is expected to have IRC receiver implemented.

- The 8Rx requirements defined in Rel-18 are release independent from Rel-17 (TS38.307 Table 5.4-1)

- No new signalling is being introduced for 8Rx requirements.

Since 8Rx SU-MIMO requirements are release independent from Rel-17 and requirements for 2Rx and 4Rx with inter-cell and intra-cell inter-user interference are introduced in Rel-17, the interference requirements defined for 8Rx can be release independent from Rel-17.

The new Rel-19 requirements for 8Rx shall also be release independent from Rel-17.

Conclusion

In this paper we have provided our view on the topic of UE demodulation performance requirements for 8Rx with MMSE-IRC.

The following Observations and Proposals were made:

General

Definition of MMSE-IRC receiver capability for 8Rx UE

- We do not strictly see the need to change “[with support of]” to make it clearer, however we are open to further discuss.

- We are fine with removing the [] from the agreement in RAN4#114bis but are also open to discuss alternative wording.

8Rx UE MMSE-IRC receiver coverage

Set the [limit] to 1.0dB

In our view, it will make sense to define requirements for both baseline and simplified receivers for all testcases of a test-type if done for one requirement of said feature. This will only apply to cases where it is at all applicable to define two sets of requirements. A test-type could for example be “PDSCH with Inter-cell Interference” or “PDSCH with Intra-cell Inter-user interference”

Introduce one more rule on the top of the agreed criteria: After reviewing the simulation results for all the test cases of a test-type by the agreed criteria, if 2 sets of requirements are required for at least one test case of said test-type, define two sets of PDSCH requirements for all cases with said test type, which are applicable for both baseline and simplified receivers.

PDSCH requirements with inter cell interference

Requirement definition

It is feasible to define requirements for all simulated inter-cell interference cases from RAN4#114bis. Requirements can be defined the usual way by outlier removal and adding impairments + 0.4dB margin added to the average of impaired results.

PDSCH requirements with intra-cell inter-user interference

Requirement definition

It is feasible to define requirements for all simulated inter-cell interference cases from RAN4#114bis. Requirements can be defined the usual way by outlier removal and adding impairments + 0.4dB margin added to the average of impaired results.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm and Receiver coverage

Select the SINR based on TP gain for one of the receiver types and check the gamma value for each receiver type with said SINR value.

If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined.

Applicability rule and release independency

Test applicability to skip 2/4Rx requirements

UE that passes Rank2 ICI test case with 8Rx can skip Rank1 ICI requirements with 4Rx.

Do not introduce applicability rules for 8Rx UE to skip 4Rx Intra-cell Inter-user interference requirements.

Release independence

Since 8Rx SU-MIMO requirements are release independent from Rel-17 and requirements for 2Rx and 4Rx with inter-cell and intra-cell inter-user interference are introduced in Rel-17, the interference requirements defined for 8Rx can be release independent from Rel-17.

The new Rel-19 requirements for 8Rx shall also be release independent from Rel-17. Further discuss if the requirements shall be mandatory for Rel-18 and optional for Rel-17

- References

- R4-2504694 - Way forward for [114bis][322] NR_demod_Ph5_Part2_UE

- R4-2503545 - SummarySimRes_8RX_ICI_MUMIMO_114bis