3GPP TSG-RAN WG4 Meeting #115	R4-2506442

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance and CSI reporting requirements for MIMO_Ph5

Agenda item:	7.24.6

Document for:	Discussion

Introduction

In the WID [1] for NR MIMO Phase 5 the following objectives are given for RAN1:

- Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

We see bullet 2. and 3. having relevance for UE demodulation performance and/or reporting requirements. The request in the WID for the objective of performance part WI is [1]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify necessary performance requirements for the enhancements listed above.

In the following contribution we will provide background information for the enhancements done with relation to “CSI support for up to 128 CSI-RS ports” and “UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul” as well as which changes we see relevant for RAN4 to further investigate for potential introduction of requirements as well as which type of requirements.

Discussion

CJT reporting requirements

Time offset (TO) reporting

The time offset (TO) across a set of TRPs is caused due to non-ideal/non-synchronized backhaul. In Rel. 18 CJT eType II the backhaul was assumed ideal and the existing delay profile difference across TRPs was only affected by the propagation delays. Rel. 18 CJT eType II accounted for the delay misalignment by means of the mode 1 which could pre-compensate these offset due to propagation delay during the frequency compression part. However, this feature was not really intended to tackle the time offset compensation, but it was rather a way to align the delay taps information to increase the compression efficiency by aligning taps to reduce the number of reported CJT eType II coefficients.

In addition, with Rel 18 CJT eType II mode 1 the maximum detectable delay was enabled by the frequency subband resolution of the CSI report equal to 1/∆fSB which is rather rough taking into account ∆fSB is a multiple of several PRBs. The latter is also connected with the role of the cyclic prefix (CP), which is there to help that transmitted data symbols to be properly recovered, eliminating the effect of the inter-symbol interference (ISI) when data is not correctly aligned in time. The CP can help to certain extent mitigate the misalignment however significant losses may appear if large TO values are not correctly compensated for multiple TRP carrying out coherent data transmission. The latter is illustrated below in Figure 1.

Figure 1. Illustration of the Time Offset for multiple TRPs and the effect on inter-symbol interference.

For that reason, the standalone procedure of measurement of the offsets was included in the 3GPP TS 38.214 v19 [3] as a way to better enable the time offset detection by means of exploiting the better capabilities from the CSI time tracking reference signals (TRS).

The reporting of the time offset is done based on providing the relative offset with respect to a selected reference TRP. A single wideband Time Offset (TO) quantity is reported for each of the TRPs, except the reference TRP. To indicate which TRP is chosen as a reference, UE includes TRS resource set index associated with this TRP. The reported TO quantities require a defined quantization range defined in term of the CP and given by AD = {0.5CP, CP}. The quantization range can have up MD = {32, 64,128, 256} different quantization steps. Notice that the last step is used to report an out-of-range indication.

After reporting the TO values to the NW side, the NW may compensate the PDSCH during data transmission using the knowledge of the TO reported values for the set of TRPs. The correct compensation of the PDSCH for data transmission is our motivation to propose new demodulation requirements for the new CJT calibration quantities, not only for TO but also for CFO and PO.

On the other hand, an important distinction of the TO reporting with other offsets is the fact that the standalone TO procedure can be linked with the CJT eType II reporting. First of all, TO can be calculated jointly or in a separated time occasion to the CJT eType II. In any of these cases, the estimated TO is used to adjust the received CSI-RS at the UE before the CJT eType II report is calculated. Later, we discuss the implications of this for other requirements like PMI and the CJT calibration performance requirements that we aim to propose.

The following example below is an SLS simulation for FDD with CJT eType II, 4 TRPs and 500m ISD, in which we compare different cases for CJT TO calibration with and without TO error, the latter corresponding to the ideal fully synchronized case. In these simulations, the normalized throughput performance of two-sided TO compensation versus case without compensation and case without TO error is analyzed. Two-sided TO compensation is applied at the NW side to calculate the (CJT) PDSCH precoder, and at the UE on the measured CSI-RS before CJT CSI calculation. The latter is just like in the linked case with TO compensation before Rel. 18 CJT eType II calculation that we mentioned before. Notice that losses between the non-compensated case (with TO) with respect to the two-sided compensated case and the ideal case are just 2% in this simulation. However, these figure numbers may increase for larger ISD values.

Figure 2. Simulation example of the CJT performance with/without TO compensation

Carrier frequency offset (CFO) reporting

Similarly to the TO case, the new Rel. 19 has also introduced standalone reporting of the carrier frequency offset (CFO). In the case of CFO, the impairment might occur due to small issues with the oscillators and other circuitry inside the set of TRPs creating different frequency drifts across the multi TRP transmission as illustrated in Figure 3. CFO introduces a time-variant complex exponential component into the properties of the transmission channel. This can be seen as a variable phase rotation introduced and the effect is not critical for single TRP transmission as the data reception can easily remove the effect from the constellation of received symbols. The problem appears when the coherent transmission is done with multiple TRPs and each of the TRP may have a different frequency drift. This makes it unfeasible to separate the effect of CFO from the transmitted data.

Figure 3. Illustration of the Carrier Frequency Offset for multiple TRPs.

Hence, CFO issues for CJT transmission also justified a standalone UE report in Rel. 19. In this case, the measurement is also supported on TRS supported on the good resolution capabilities in time domain. Similarly, like in TO, the anchor TRP is selected and the offsets are reported relative to this reference. A single wideband CFO quantity is reported for each of the TRPs, except the reference anchor TRP. The reference TRS resource set associated with the anchor TRP index is included in the report. The quantization range as per specification includes the values given by the options AFO = {0.1ppm, 0.2ppm} in which ppm indicates part per million of the central carrier frequency. The number of quantization steps includes the following options MFO = {16, 32, 256}. On the contrast to TO, the last quantization step is reserved for reporting an invalid measurement, which should not be confused with out-of-range. Currently, the determination of invalidity is left to implementation and interpretation of the UE side.

On the other hand, in contrast to TO, CFO reporting has no linkage determined in the specification to the calculation of CJT eType II PMI. Therefore, there is not the same ambiguity as in TO with relation to whether CFO could justify an extension of the PMI requirements. We will discuss this issue later.

Figure 4. Simulation example of the CJT performance with/without CFO compensation

Here we also provide exemplary CFO compensation SLS results for synthetically varying maximal CFO compensation error in Figure 4. SLS results for CJT CFO calibration reporting shows how mTRP CJT spectral efficiency (SE) normalized relative to single TRP SE. We see that all the performance gain from CJT may be lost if accurate compensation is not achieved. Also, what can be noted is that most gains are available for the cell-edge users UE experiences low but balanced RSRP from multiple TRPs.

Phase offset (PO) reporting

Finally, the phase offset (PO) reporting is a CJT calibration quantity that is intended only for TDD, in contrast to TO and CFO which make sense for both FDD and TDD. The goal here is to align the existing phase offsets between the reception of the uplink channel signals like SRS and the transmitting signals using the downlink channel. In other words, if a phase misalignment exists between the UL and DL that means the PDSCH could not correctly utilize the acquired channel due to the mismatch from SRS and performance losses are expected as a result. The phase mismatch in TRP is illustrated in Figure 4.

Figure 4. Illustration of the Phase Offset between UL/DL for multiple TRPs

In this case the PO reporting requires some support from the UL side to do the estimation. Two major steps are needed,  1) a triggered SRS measurement and after that 2) a CSI-RS measurement using the same UE ports used for the first measurement. By the combination of the two measurements the UL/DL phase offset can be determined. During RAN 1 studies, two types of implementations were discussed. One using non-precoded CSI-RS signals and a second using precoded CSI-RS signals based on the first SRS measurement. There are no major implications of using one or another in which respects to the impact of the 3GPP specification. As mentioned before, the fact of using the same UE ports for DL and UL is an important aspect of the accurate PO estimation. Based on RAN 1 discussions it was agreed to use a single SRS port and likewise the same UE port at the DL CSI-RS reception. Notice that only 1 single port CSI-RS per TRP can be configured for doing the estimation. And the reference CSI-RS resource set associated with the reference TRP index is included in the report.

Regarding the reporting aspects of the PO reporting the phase value is between  0 ≤ φ ≤ 2π and the number of quantization steps is given by Mφ ={16, 32, 256}. Similarly to CFO, the last quantization state is mapped to report an invalid condition. Another particularity of the PO reporting is that it supports wideband (basic feature) and subband reporting (extended feature). The subband PO reporting was justified by the possible frequency selectivity that PO may exhibit in some cases. The size of a subband is defined by the options {1, 2, 4, 8, 16} PRBs per subband.

In the following example of a SLS simulation in Figure 6 we provide a comparison of PO calibration for a case with TDD scheme, 4TRPs and ISD 200m. PO compensation was carried out with a single SRS port as specified in specification. It is important to note that the effect of SRS noise is not considered in these simulations to remove its effect from the consideration. Non-deal SRS may limit the ISD values as SRS quality may be quite degraded at long distances. In the results we notice that the case with phase offset misalignment (i.e. with compensation error) may result is significant losses with respect to the ideal case (i.e., no calibration error) leading to loss of all the gains from CJT operation.

Figure 6. performance results with/without Phase Offset calibration for TDD CJT.

RAN4 RRM work summary

Multiple companies brought simulation results for time and frequency offset CJT calibration reporting. Only two companies brought results for phase reporting. RAN4 has agreed to define RRM core requirements for time and frequency offset reporting based on the simulation results for CJT calibration [4]. RAN4 is still to define the number of samples required to achieve acceptable accuracy for time and frequency offsets reporting for CJT for RRM core requirements. On the other hand, work in RRM towards defining any phase offset reporting requirements is indeed behind the efforts to define core requirements for time and frequency offsets. Anyway, we consider that phase offset study is quite important and it should be considered into the RAN 4 demodulation performance agenda. In this document we propose new CJT calibration offset reporting requirements and this includes TO, CFO, and PO as part of the objectives.

Core requirements for CJT phase offset reporting might not be specified, what increases the importance of studying feasibility for CJT calibration phase offset demodulation requirements.

Demodulation performance requirements

The introduction of the new CJT calibration reporting feature has no impact on demodulation performance requirements, hence we see no need to introduce new demodulation performance requirements (i.e., PDSCH, PDCCH, PBCH, SDR) due to the introduction of the new CJT reporting requirements.

The introduction of the new CJT calibration reporting feature does not impact UE performance requirements (i.e., PDSCH, PDCCH, PBCH, etc.).

Do not introduce new demodulation performance requirements for PDSCH, PDCCH, PBCH or SDR.

CQI/RI reporting requirements

Although CJT offsets may indirectly influence CQI and RI values reported by UE, we do not see any changes required for CQI and RI reporting requirements. The main reason is that CQI/RI reporting capabilities can directly be tested without CJT calibration and/or CJT in general since CJT operation should be mostly transparent to UE.

CQI/RI reporting can be directly tested without CJT calibration and/or CJT in general, hence there is no need for additional reporting requirements for CQI or RI.

Do not introduce new CQI and RI reporting requirements.

PMI reporting requirements

PMI reporting may be relevant for the CJT calibration reporting and will be discussed in the next section. Regarding the PMI requirements themselves, at first glimpse, the linkage in specification of TO with CJT eType II could indicate some changes justifying new PMI requirements. However, as seen the PMI calculation as such is not changing per se, and the test is different to the existent PMI reporting requirement. The test would require calculating the PMI but also it would require the offsets compensation when PDSCH transmission is done. This corresponds indeed to a new performance requirement not existent in the specification.

Moreover, for CFO there is no linkage with the CJT eType II calculation and PO is intended for CJT with TDD (thus no PMI needed). Therefore, we do not propose new PMI requirements but instead propose to consider PMI as part of the new CSI reporting requirement namely CJT calibration offset reporting requirement  that we propose in the next section.

CJT calibration reporting and with PMI reporting are not directly interdependent and do not require changes to the existing PMI reporting requirements.

Do not introduce new dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements.

CJT calibration offsets reporting requirements

We propose to create a new section related to the recently introduced Rel. 19 CJT calibration offsets reporting requirements as part of the TS38 101-4 technical specifications. The correct estimations, reporting and compensation of the different offsets that may appear across a set of TRPs, namely the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) may have a significant impact on performance if not properly compensated during the CJT data transmission. In this subsection, we open the discussion on the necessary methodology to simulate and estimate the impact on performance based on the correct compensation of the impairments related to these offsets.

CJT calibration offsets reporting is a new type of CSI reporting that have a direct performance effect and requires new requirements to be defined.

Introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements.

Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

Proposed methodology to simulate the performance requirements

In this section we mention some ideas related to the proposed methodology for the performance requirements associated to the already described CJT calibration quantities. First of all, the goal consists of measuring the PDSCH data transmission performance after compensation with the TO, CFO and PO reported by the UE. This measurement will give a direct relation to how good the performance is after compensation.

In order to carry out the test, the testing equipment and/or a network in the simulation setup may need to consider at least the means to represent 2 TRPs transmitting toward a given UE. The methodology should consist of introducing offsets in the reference signals and the data transmission following some rules on how the offsets are generated. UE may measure the offsets and report them to the testing equipment /network side. PDSCH transmission can be compensated based on the reported offset by the UE. Then metrics of performance can be obtained to compare the compensated PDSCH case vs. a given baseline.

Two cases may be studied,

- TDD channel acquisition (for TO, CFO and PO)

- FDD CSI based channel acquisition (for TO and CFO).

In the case with FDD, it shall be considered if the PMI calculation are to be excluded from the sequence of steps, whereas for TDD the channel can be acquired from uplink. In all cases it shall be considered if the channel acquisition is needed for the reporting requirement test but independent of the standalone CJT calibration offset reporting carried out by the UE.

FDD case does not need to be considered for defining CJT calibration phase offset reporting requirements.

Further study how to define CJT calibration offset reporting requirements with focus on specific methodologies for FDD and TDD cases. Starting point shall be:	
- TDD channel acquisition (for TO, CFO and PO)	
- FDD CSI based channel acquisition (for TO and CFO).

For the case with FDD, as mentioned earlier, PMI calculation might be needed. In this case we may consider that this case is a special requirement that could be called joint PMI/CJT calibration offset reporting performance requirements. CJT calibration reporting requirements may require (for all or some of the offsets: TO, CFO, PO) defined with connection to PMI reporting due to the fact that noticeable throughput or BER changes with or without CJT calibration reports taken into account may not be measurable without Rel-18 CJT eType-II codebook PMI reporting for CJT precoding. One of the ways to find out whether CJT calibration reporting requirement can be defined without PMI Rel-18 CJT eType-II codebook is to provide LLS results for the following cases: 1) CJT with CJT PMI and with CJT calibration, 2) CJT with CJT PMI and without CJT calibration, 3) CJT with random PMI and with CJT calibration, 4) CJT with random PMI and without CJT calibration, 5) sTRP eType-II PMI.

For the FDD case performance requirements might need a joint PMI/CJT calibration performance requirement. We see simulations are needed to confirm.

Further study whether the PMI calculation is necessary for the CJT calibration performance requirements studies in the FDD case by simulating the following cases:	
1) CJT with CJT PMI and with CJT calibration	
2) CJT with CJT PMI and without CJT calibration	
3) CJT with random PMI and with CJT calibration	
4) CJT with random PMI and without CJT calibration	
5) sTRP eType-II PMI.

In order to evaluate the quality of the reports from the UE we need to evaluate, for instance, the throughput metric based on the compensation of PDSCH for data transmission when the testing equipment uses the estimated and reported offsets provided by the UE. Different baselines could be considered to determine the resulting γ metrics. For instance, we may consider:

- Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. random compensation of PDSCH.

- Comparison of the PDSCH performance based on the reported offsets measured by UE vs. the non-compensated PDSCH.

- Comparison of the PDSCH performance based on the reported offsets measured by UE vs. ideally compensated PDSCH.

Further study the metrics to be used in a CJT calibration offsets reporting requirements. Starting point shall be one of the following options:	
A) Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. random compensation of PDSCH.	
B) Comparison of the PDSCH performance based on the reported offsets measured by UE vs. the non-compensated PDSCH.	
C) Comparison of the PDSCH performance based on the reported offsets measured by UE vs. ideally compensated PDSCH.

CSI support for up to 128 CSI-RS ports

Background

During Rel-19, RAN1 has been extending CSI feedback support in NR to 128 gNB transmit ports according to the following work item objective [1]:

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

In this section of the contribution, we first provide background information on the developments in RAN1 and then discuss the RAN4 performance requirements to support the new included features.

Prior to Rel-19, the various NR codebooks for CSI feedback supported feeding back precoding matrix indices (PMIs) for up to 32 CSI-RS ports. In Rel-19, the following codebooks have been extended to support 48, 64, and 128 CSI-RS ports:

- Type I Single-Panel Codebook (TypeI SP)

- Type I Multi-Panel Codebook (TypeI MP)

- Enhanced Type II Codebook (eTypeII)

- Further Enhanced Type II Port Selection Codebook (feTypeII) – supporting 48 and 64 CSI-RS ports only

- Enhanced Type II Codebook for Predicted PMI (eTypeII Doppler)

The configuration of CSI-RS resources has not been fundamentally modified to support the increased CSI-RS port counts. The maximum number of ports which can be configured in a CSI-RS resource remains at 32 ports. Instead, 2, 3, or 4 CSI-RS resources are aggregated in a CSI-RS resource set to support the increased port counts. The supported CSI-RS port configurations and aggregation factors K are shown in Table 1 where (N1,N2) indicate the antenna array configuration per polarization in terms of the number of columns and rows, respectively.

Table 1. Supported CSI-RS Configurations with Aggregation Factors

| Number of CSI-RS antenna ports, PCSI-RS | (N1,N2) | Aggregation Factors, K |
| --- | --- | --- |
| 48 | (8,3) | 2, 3 |
|  | (6,4) |  |
| 64 | (16,2) | 2, 4 |
|  | (8.4) |  |
| 128 | (16,4) | 4 |
|  | (8,8) |  |

Two methods have been agreed for mapping the ports from the aggregated CSI-RS resources to the codebook ports. From a simplified viewpoint, the two methods correspond to dividing the array into resources horizontally or vertically.

The codebook extensions take several forms. The three Type II codebooks are extended to support higher port counts without structural changes to the codebooks, but by simply adding support for the additional array configurations given in Table 1. The TypeI SP codebooks are extended using two different schemes called Scheme A and Scheme B which modify the way the DFT beams in the codebooks are selected for the wideband W1 matrix and the options for co-phasing the beams across polarizations. In addition, as in the legacy codebooks, there are differences in the codebook structures between ranks 1-4 and ranks 5-8. Due to these changes in structure, the beam search methods for the extended TypeI SP codebooks will likely be modified in UE implementations due to the changes in the codebook structures. Finally, the TypeI MP codebook is extended in a way similar to the Type II codebooks – with minimal changes to the codebook structure – but with only one codebook mode, whereas the legacy codebook has two modes.

Structural changes to the codebooks relative to the legacy codebooks are limited to the Type I single panel codebooks.

The extended Type I single panel codebooks will likely require changes in the beam search methods used in codebook implementations at the UE.

Now we summarize the structures of the two TypeI SP codebook schemes. In Scheme A for ranks 1-4, beam selection is essentially identical to beam selection in codebookMode = 1 of the legacy codebooks, except that the P<16 ports approach is followed for ranks 3-4.  An anchor beam is selected, and additional beams are selected from a small orthogonal set relative to the anchor beam.  For ranks 5-8, beam selection is performed from a maximally sized orthogonal set relative to the selected anchor beam.  This method creates an increase in the feedback signaling overhead for the selected beams, but this overhead is wideband and only one instance is required.  For all ranks, co-phasing requires 1-2 bits per subband depending on the rank.

In the TypeI SP codebook Scheme B, arbitrary beam selection per layer is performed from a set of beams which are mutually orthogonal in both the horizontal and vertical directions.  For ranks 1-4, one beam is selected per layer and independently signaled for each layer.  For ranks 5-8, one beam is selected per layer pair and the selected set of beams is combinatorially signaled as in the legacy (Rel-15) Type II codebook.  For all ranks, co-phasing is 2 bits per layer per subband.  The CSI feedback overhead for Scheme B for per subband CSI feedback can be significantly higher than in Scheme A due to the larger number of co-phasing bits per subband and due to the unique co-phasing per subband.

The signaling overhead for Scheme B can be significantly higher than for Scheme A with per subband reporting due to unique co-phasing per layer as well as constant 2 bits per co-phasing factor.

PDSCH, PDSCH CA, PDCCH, PBCH, and SDR performance requirements

The extension of the CSI codebooks to support up to 128 CSI-RS ports primarily creates new codebooks in the NR standard.  Since CSI-RS resources are aggregated to support the larger number of CSI-RS ports, the specifications also define how to map the ports of the aggregated CSI-RS resources to the codebook ports.  Both of these aspects can be tested by PMI reporting requirements so that PDSCH, PDSCH CA, PDCCH, PBCH, and SDR requirements are not applicable to the extended codebooks.

We see that PDSCH, PDSCH CA, PDCCH, PBCH, and SDR requirements are not applicable to the extended codebooks.

Do not introduce new PDSCH, PDSCH CA, PDCCH, PBCH, or SDR performance requirements for the Rel-19 CSI codebook extensions to support up to 128 ports.

CQI and RI reporting requirements

Existing requirements for reporting the CQI and the RI address throughput gains when these parameters are properly reported.  The requirements are addressed using few transmit ports at the gNB and using the (Rel-15) TypeI SP codebook.  For CQI reporting, the requirements address conditions such as wideband and per-subband CQI reporting.  While the CQI is calculated conditioned on the codebook entry, new mechanisms for these calculations are not introduced by the codebook extensions to 128 ports.  RI is calculated as a wideband parameter and the introduction of the extended codebooks also does not affect the calculation of the RI.  Since the reporting of both CQI and RI have not changed in Rel-19, we see no need to define additional requirements for CQI nor RI.

Since the reporting of both CQI and RI have not changed in Rel-19, we see no need to define additional requirements for CQI nor RI.

Do not introduce new CQI or RI reporting requirements for the Rel-19 CSI reporting extensions to support up to 128 ports.

PMI reporting requirements

Of the codebooks which have been extended to support up to 128 ports, only the legacy TypeI SP, eTypeII, and eTypeII Doppler codebooks have existing PMI reporting requirements.  For the extended codebooks, defining PMI reporting requirements is most important for the TypeI SP codebooks due to the changes in the codebook structure which lead to changes in the codebook search algorithms at the UE.

Legacy PMI reporting requirements only exist for the TypeI SP, eTypeII, and eTypeII Doppler codebooks (of the codebooks which are extended to 128 ports), but the codebook structure has only been modified for the extended TypeI SP codebook. Since TypeI SP codebooks have the least feedback overhead, are the simplest, and have the most modification from legacy codebooks, hence requirements are prioritized for these codebooks.

For Rel-19 CSI reporting extensions, introduce PMI reporting requirements for the extended TypeI SP codebooks.

The codebook structure has not been modified for neither the extended eTypeII codebook nor the eTypeII Doppler codebook.  The extensions to these codebooks, as required by the work item objective, have been implemented by adding support for larger port counts without structural changes to the codebooks.  Therefore, the methods used by UEs to implement the extended Type II codebook search will likely be extended versions of algorithms used for the legacy codebooks.  Also, legacy codebooks are prerequisite features for the extended codebooks in the UE capabilities being developed for Rel-19.  Introducing additional requirements for the extended Type II codebooks would test extended search algorithms as well as extended prediction algorithms for the eTypeII Doppler codebook.  On the other hand, requirements would test the increased computational complexity due to the additional antenna ports or areas where the implementation is modified, rather than simply extended, to control the computational complexity.

The extended eTypeII and eTypeII Doppler codebooks have been implemented by adding support for larger port counts without structural changes to the codebooks.  UE implementations of these codebooks are likely to be extensions of the legacy codebook implementations.

While the structure of the eTypeII and eTypeII Doppler codebooks are similar (especially when N4=1 in the Doppler codebook), the Doppler codebook requires sufficiently frequency CSI-RS instances to maintain sufficient accuracy in its predictions.  We expect the Doppler codebook to require greater CSI-RS overhead to support its use and feel that the eTypeII codebook is more likely to be implemented.  Therefore, we propose to prioritize the eTypeII codebook over the eTypeII Doppler codebook.

The extended eTypeII Doppler codebook likely requires greater CSI-RS overhead to support its use.

RAN4 to further study whether PMI reporting requirements are needed for the eTypeII and eTypeII Doppler codebook extensions, with priority on eTypeII.

For each of the extended TypeI SP codebook schemes, the base feature is support for 64 CSI-RS ports.  Therefore, as a starting point, requirements should be defined at least for 64 ports and RAN4 can discuss whether requirements for 48 and 128 ports should be also introduced.  Reviewing the legacy requirements for the TypeI SP codebooks, we noticed that many of the key parameters are common across the requirements.  Therefore, as a starting point for the extended codebook requirements, we have selected a similar set of parameters as shown in Table 2.  We start by assuming requirements for both 2 and 4 UE Rx ports but we can consider more UE Rx ports for use cases such as fixed wireless terminals.  We propose to start by prioritizing requirements for Scheme A since it is the scheme with lower CSI feedback overhead.  For the channel model, we start with the TDLA30-5 NLoS channel because it is used in many of the legacy requirements.  Alternative channel models can also be considered.  If the TDL models are used the extended codebooks, antenna correlation matrices must be defined for more than 32 ports since the current TS 38.101-4 [2] does not specify matrices for more than 32 ports.  We address this issue in the next subsection.	
We start with high antenna correlation which is also used in many of the legacy requirements, though we are open to discussing medium antenna correlation.  A fixed rank of 2 is selected as a starting point as used in the legacy requirements.  Due to the way beam selection varies across ranks in the codebooks, other ranks can be considered, especially for ranks 5-8 since the beam selection methods vary between ranks 1-4 and ranks 5-8 in both schemes.  However, the richness of the channel transmission modes must be considered in order to have enough useful transmission modes.  In addition, the consequences of two transmitted codewords for ranks 5-8 must be considered when evaluating such a requirement.

Table 2:  Starting point for extended TypeI SP PMI reporting requirements

| Parameter | Values |
| --- | --- |
| UE Rx Ports | 2, 4 |
| CSI-RS Ports | 64 (8,4) |
| Aggregated CSI-RS Resources | 2 |
| Codebooks | TypeI SP, Scheme A (priority) |
| Channel model | TDLA30-5 |
| Antenna Correlation | High X-pol |
| Ranks | 2 |

Many of the key parameters are common across the legacy TypeI SP codebook requirements and can be used as a starting point for the extended TypeI SP codebook requirements.

Use the parameters in the following table as the starting point for discussing requirements for the extended TypeI SP codebook. Further discuss whether other ranks and number of CSI-RS Ports shall be included in the evaluation:

| Parameter | Values |
| --- | --- |
| UE Rx Ports | 2, 4 |
| CSI-RS Ports | 64 (8,4) |
| Aggregated CSI-RS Resources | 2 |
| Codebooks | TypeI SP, Scheme A (priority) |
| Channel model | TDLA30-5 |
| Antenna Correlation | High X-pol |
| Ranks | 2 |

Extension of Antenna Correlation Matrices for greater than 32 ports

In order to address and evaluate requirements for the extended codebooks, the use of a spatially correlated channel is desired.  While true spatial channel models (such as the CDL models) would make this straightforward, TDL models are used in legacy codebook PMI requirements and currently the only option available.  Currently in TS38.101-4 [2], correlation matrices for the TDL channels are specified for up to 8 transmit ports for uniform linear arrays (ULAs) and up to 32 ports, only in a (4,4,2) configuration, for cross-polarized arrays (X-pol).  Assuming that TDL models will continue to be used, support for correlation matrices for larger port counts are required in order to create correlated arrays for Rel-19 codebook requirements.  However, it has been found that TDL correlation matrix derivations that exceed the (initially measurement justified) 2 Tx/Rx branches, develop unintentionally abysmal correlation values [5]. This makes the TDL+correlation channel model extremely ill conditioned and extremely rank deficient starting from 4 ports in uni-pol and 8 ports in XP variants.

If it is found that extreme ill condition and rank deficiency is not a problem for the >32 port CB extension, the existing correlation matrices can be extended to support larger port counts and additional antenna configurations through an extension of the existing method for calculating these correlation matrices by recognizing the methods used to construct these matrices. Therefore, we propose such an extension of the existing specifications as the method for calculating spatial correlation matrices with greater than 32 ports. While the extended codebooks are designed for use with X-pol arrays, for completeness’ sake we show the extensions for correlation matrices for both X-pol arrays and ULAs.

The existing TDL correlation matrices defined in TS 38.101-4 can be extended to larger port counts and array configurations by expressing the matrices using formulas parameterized by the array dimensions and allowing additional values of the array dimensions to support up to at least 128 antenna ports, if extreme overestimated ill condition and rank deficiency of the channel is not an issue.

However, as the study on (new) spatial channel models in RAN4 demodulation requirements (FS_NR_demod_SCM) is still ongoing, we propose to try TDL correlation extension as a stop gap solution.  The correlation matrix details are described in Appendix A where the essential changes are provided in the following two proposals.

Extend the antenna correlation matrix models for X-pol arrays in TS 38.101-4 to support greater than 32 ports in the following manner:

- For the first dimension (N1), define the vector r1: - r₁(k) = αk - 1N1- 12, for k = 1, 2, ..., N₁ - RgNBDim,1 = Toeplitz(r₁) - where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1. - For the second dimension (N2), define the vector r2: - r₂(k) = αk - 1N2- 12, for k = 1, 2, ..., N₂ - RgNBDim,2 = Toeplitzr2 - If N2=1, RgNBDim,2=1. - Then the correlation matrix at the gNB is RgNB=RgNBDim,1⨂ RgNBDim,2 - The correlation matrix at the UE side assumes a 1D cross-polar array: - rue(k) = βk - 1N- 12, for k = 1, 2, ..., N - RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.

In addition, the antenna correlation matrix models for ULAs in TS 38.101-4 to support larger port counts in the following manner:

- The correlation matrix at the gNB side for a ULA have a Toeplitz structure: - r(k) = αk - 1N- 12, for k = 1, 2, ..., N - RgNB = Toeplitzr - where Toeplitz(r) constructs an N×N Toeplitz matrix from the vector r. - The spatial correlation matrix at the UE side follows the same logic but with α replaced by β as follows: - rue(k) = βk - 1M- 12, for k=1,2,…,M - RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.

Conclusion

In this paper we have provided our view on the topic of UE demodulation performance and CSI reporting requirements for MIMO_Ph5

The following Observations and Proposals were made:

CJT reporting requirements

RAN4 RRM work summary

- Core requirements for CJT phase offset reporting might not be specified, what increases the importance of studying feasibility for CJT calibration phase offset demodulation requirements.

Demodulation performance requirements

The introduction of the new CJT calibration reporting feature does not impact UE performance requirements (i.e., PDSCH, PDCCH, PBCH, etc.).

- Do not introduce new demodulation performance requirements for PDSCH, PDCCH, PBCH or SDR.

CQI/RI reporting requirements

CQI/RI reporting can be directly tested without CJT calibration and/or CJT in general, hence there is no need for additional reporting requirements for CQI or RI.

Do not introduce new CQI and RI reporting requirements.

PMI reporting requirements

CJT calibration reporting and with PMI reporting are not directly interdependent and do not require changes to the existing PMI reporting requirements.

Do not introduce new dedicated PMI reporting requirements but consider PMI calculation as part of the CJT calibration reporting requirements.

CJT calibration offsets reporting requirements

CJT calibration offsets reporting is a new type of CSI reporting that have a direct performance effect and requires new requirements to be defined.

Introduce new requirements for CJT calibration offset reporting for the time offset (TO), the carrier frequency offset (CFO) and the phase offset (PO) scenarios. Further study exact implementation of the new requirements.

Introduce a new subsection in TS 138 101-4 to cover introduction of CJT calibration offset reporting requirements.

Proposed methodology to simulate the performance requirements

FDD case does not need to be considered for defining CJT calibration phase offset reporting requirements.

Further study how to define CJT calibration offset reporting requirements with focus on specific methodologies for FDD and TDD cases. Starting point shall be:	
- TDD channel acquisition (for TO, CFO and PO)	
- FDD CSI based channel acquisition (for TO and CFO).

For the FDD case performance requirements might need a joint PMI/CJT calibration performance requirement. We see simulations are needed to confirm.

Further study whether the PMI calculation is necessary for the CJT calibration performance requirements studies in the FDD case by simulating the following cases:	
1) CJT with CJT PMI and with CJT calibration	
2) CJT with CJT PMI and without CJT calibration	
3) CJT with random PMI and with CJT calibration	
4) CJT with random PMI and without CJT calibration	
5) sTRP eType-II PMI.

Further study the metrics to be used in a CJT calibration offsets reporting requirements. Starting point shall be one of the following options:	
A) Comparison of the compensated PDSCH performance based on the reported offsets measured by UE vs. random compensation of PDSCH.	
B) Comparison of the PDSCH performance based on the reported offsets measured by UE vs. the non-compensated PDSCH.	
C) Comparison of the PDSCH performance based on the reported offsets measured by UE vs. ideally compensated PDSCH.

CSI support for up to 128 CSI-RS ports

Background

Structural changes to the codebooks relative to the legacy codebooks are limited to the Type I single panel codebooks.

The extended Type I single panel codebooks will likely require changes in the beam search methods used in codebook implementations at the UE.

The signaling overhead for Scheme B can be significantly higher than for Scheme A with per subband reporting due to unique co-phasing per layer as well as constant 2 bits per co-phasing factor.

PDSCH, PDSCH CA, PDCCH, PBCH, and SDR performance requirements

We see that PDSCH, PDSCH CA, PDCCH, PBCH, and SDR requirements are not applicable to the extended codebooks.

Do not introduce new PDSCH, PDSCH CA, PDCCH, PBCH, or SDR performance requirements for the Rel-19 CSI codebook extensions to support up to 128 ports.

CQI and RI reporting requirements

Since the reporting of both CQI and RI have not changed in Rel-19, we see no need to define additional requirements for CQI nor RI.

Do not introduce new CQI or RI reporting requirements for the Rel-19 CSI reporting extensions to support up to 128 ports.

PMI reporting requirements

Legacy PMI reporting requirements only exist for the TypeI SP, eTypeII, and eTypeII Doppler codebooks (of the codebooks which are extended to 128 ports), but the codebook structure has only been modified for the extended TypeI SP codebook. Since TypeI SP codebooks have the least feedback overhead, are the simplest, and have the most modification from legacy codebooks, hence requirements are prioritized for these codebooks.

For Rel-19 CSI reporting extensions, introduce PMI reporting requirements for the extended TypeI SP codebooks.

The extended eTypeII and eTypeII Doppler codebooks have been implemented by adding support for larger port counts without structural changes to the codebooks.  UE implementations of these codebooks are likely to be extensions of the legacy codebook implementations.

The extended eTypeII Doppler codebook likely requires greater CSI-RS overhead to support its use.

RAN4 to further study whether PMI reporting requirements are needed for the eTypeII and eTypeII Doppler codebook extensions, with priority on eTypeII.

Many of the key parameters are common across the legacy TypeI SP codebook requirements and can be used as a starting point for the extended TypeI SP codebook requirements.

Use the parameters in the following table as the starting point for discussing requirements for the extended TypeI SP codebook. Further discuss whether other ranks and number of CSI-RS Ports shall be included in the evaluation:

| Parameter | Values |
| --- | --- |
| UE Rx Ports | 2, 4 |
| CSI-RS Ports | 64 (8,4) |
| Aggregated CSI-RS Resources | 2 |
| Codebooks | TypeI SP, Scheme A (priority) |
| Channel model | TDLA30-5 |
| Antenna Correlation | High X-pol |
| Ranks | 2 |

Extension of Antenna Correlation Matrices for greater than 32 ports

The existing TDL correlation matrices defined in TS 38.101-4 can be extended to larger port counts and array configurations by expressing the matrices using formulas parameterized by the array dimensions and allowing additional values of the array dimensions to support up to at least 128 antenna ports, if extreme overestimated ill condition and rank deficiency of the channel is not an issue.

Extend the antenna correlation matrix models for X-pol arrays in TS 38.101-4 to support greater than 32 ports in the following manner:

- For the first dimension (N1), define the vector r1: - r₁(k) = αk - 1N1- 12, for k = 1, 2, ..., N₁ - RgNBDim,1 = Toeplitz(r₁) - where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1. - For the second dimension (N2), define the vector r2: - r₂(k) = αk - 1N2- 12, for k = 1, 2, ..., N₂ - RgNBDim,2 = Toeplitzr2 - If N2=1, RgNBDim,2=1. - Then the correlation matrix at the gNB is RgNB=RgNBDim,1⨂ RgNBDim,2 - The correlation matrix at the UE side assumes a 1D cross-polar array: - rue(k) = βk - 1N- 12, for k = 1, 2, ..., N - RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.

In addition, the antenna correlation matrix models for ULAs in TS 38.101-4 to support larger port counts in the following manner:

- The correlation matrix at the gNB side for a ULA have a Toeplitz structure: - r(k) = αk - 1N- 12, for k = 1, 2, ..., N - RgNB = Toeplitzr - where Toeplitz(r) constructs an N×N Toeplitz matrix from the vector r. - The spatial correlation matrix at the UE side follows the same logic but with α replaced by β as follows: - rue(k) = βk - 1M- 12, for k=1,2,…,M - RUE = Toeplitz(rue) - The calculation of Rspat, Rhigh, and Rmedium remains the same as currently specified in TS 38.101-4. - The values of α, β, and the scaling factor a for specific correlation levels and antenna configurations are FFS.

- References

- RP-242394, ”WID revision: NR MIMO Phase 5,” Samsung (Moderator), 3GPP TSG RAN Meeting #105, 9-12 September 2024.

- 3GPP TS 38.101-4, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18),” V18.7.0.

- TS 38.214 - NR; Physical layer procedures for data

- R4-2504903 - WF on RRM requirements for NR_MIMO_Ph5_Part2.

- R4-2411300, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#112.

- Appendix A: MIMO Correlation Matrices using cross polarized antennas

For X-pol arrays the spatial correlation matrix at the gNB is obtained using Toeplitz structures along the two dimensions and then combining them using the Kronecker product. The decay of correlation is governed by the parameter α ∈ (0, 1).  The following discussion indicates via formulas how the existing correlation matrices are specified so that extension of these methods to larger port counts is straightforward by parameterizing the array dimensions via N1 and N2.  These equations apply not only to the larger port counts but they will also reproduce the currently specified correlation matrices for up to 32 ports.

Let N₁, N₂ ∈ N be the number of antenna ports along the first and second dimensions, respectively, per polarization, assuming two polarizations.

For the first dimension (N1), define the vector r1:

r₁(k) = αk - 1N1- 12, for k = 1, 2, ..., N₁.

Then the correlation matrix of antenna ports in the first dimension with the same polarization is:

RgNBDim,1 = Toeplitz(r₁)

where Toeplitz(r₁) constructs an N1×N1 Toeplitz matrix from the vector r1.

Similarly, for the second dimension (N2), define the vector r2:

r₂(k) = αk - 1N2- 12, for k = 1, 2, ..., N₂.

Then the correlation matrix of antenna ports in the second dimension with the same polarization is:

RgNBDim,2 = Toeplitzr2

The full two-dimensional (2D) spatial correlation matrix at the gNB for a single polarization is then given by:

RgNB=RgNBDim,1⨂ RgNBDim,2

where RgNB is a N1N2×N1N2 matrix.

For a one-dimensional (1D) cross-polarized antenna array, where N2=1 and  N1≥2, the spatial correlation matrix at the gNB has a Toeplitz structure like the equations for the 2D cross-polarized antenna array but with RgNBDim,2=1 and therefore:

RgNB=RgNBDim,1⨂ RgNBDim,2=RgNBDim,1

The spatial correlation matrices at the UE side follow the same logic but with α replaced by β and assuming the 1D case as follows:

rue(k) = βk - 1N1- 12, for k = 1, 2, ..., N₁

RUE = Toeplitz(rue)

The values for the α and β parameters for cross-polarized MIMO correlation matrices for greater than 32 antenna ports at the gNB will need to be agreed upon by companies for the desired correlation levels.

These generalized equations are proposed to allow for the extension of the correlation matrices to port counts greater than 32 for the gNB and greater than 8 for the UE. These equations are backward compatible with the spatial correlation matrices already in the specification (38.101).

The channel spatial correlation matrix, as per specification 38.101-4 [2], is then defined by:

Rspat=PRgNB⊗Γ⊗RUEPT

where

- RUE and RgNB are defined above,

- Γ is the polarization correlation matrix defined in [2],

- P is the permutation matrix defined in [2]

- ∙T denotes the matrix transpose.

The values for Rspat are adjusted to ensure the correlation matrix is positive semi-definite after round-off to 4-digit precision. This is done using the equations in 38.101-4 [2]:

Rhigh=Rspat+aIn1+a  or Rmedium=Rspat+aIn1+a

where the value a is a scaling factor whose value varies with the correlation level such that the resulting correlation matrix is positive semi-definite.

MIMO Correlation Matrices using ULA antennas

The spatial correlation matrices at the gNB for a ULA with N antenna ports have a Toeplitz structure:

r(k) = αk - 1N- 12, for k = 1, 2, ..., N

RgNB = Toeplitzr

The spatial correlation matrices at the UE side follow the same logic but with α replaced by β  as follows:

rue(k) = βk - 1N- 12, for k = 1, 2, ..., N

RUE = Toeplitz(rue)

The channel spatial correlation matrix is then given by the Kronecker product:

Rspat=RgNB⨂ RUE

The values for high correlation cases are adjusted to ensure the correlation matrix is positive semi-definite after round-off to 4-digit precision.  As with the X-pol correlation matrices, this is done using the equation:

Rhigh=Rspat+aIn1+a

where a is a scaling factor whose value varies with the correlation level such that the resulting correlation matrix is positive semi-definite.