3GPP TSG-RAN WG4 Meeting #115	R4-2506443

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on UE Demodulation performance requirements for LPWUS

Agenda item:	7.28.7

Document for:	Discussion

Introduction

In the WID for NR_LPWUS [1] objectives for the UE performance and test cases part has been defined as:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify the UE demodulation performance and test cases for LP-WUS/WUR (RAN4). - Study testability of above requirements - Specify RRM measurement performance requirement if identified, and corresponding test cases for the LP-WUS/WUR (RAN4).

As can be seen from the objective there are two steps involved for the topic:

- Study testability of requirements

- If testable, specify the UE demodulation performance and test cases for LP-WUS/WUR

In the following we will provide our view on which requirements would make sense to define and how they can possible be tested.

Discussion

Background

The following agreement are taken from the WF provided in RAN4#111 [3]:

Issue 1-1-9: Side condition for LP-WUR requirements Agreements: - FFS RAN4 should consider both idle and connected mode conditions in the side conditions for the LPWUR requirements. Issue 1-1-11: Metric for LP_WUR requirements Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values. Issue 2-1-1: Center frequencies for LLS simulation Agreement: - For ACS and ASCS simulation, select 900MHz, 2.6GHz and 3.5GHz as example frequencies for FR1. - FFS on FR2 example band(s) Issue 2-1-3: Waveform for LLS simulation Agreement: - Use both OOK1 and OOK4 based on RAN1 agreements for link level simulations Issue 2-1-6: Frequency error assumption for LLS simulation Agreement - Frequency error assumption for LLS simulation - Up to 20ppm Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFMD-based receiver. Issue 2-3-3: Required number of guard RB for ASCS Agreement: - RAN4 further discuss required number of guard RB for ASCS based on LLS simulation outcome next meeting.

It has been decided in RANP to work on introducing a low power wake-up signalling, to enable the UE to go to deep sleep but still be able to be “woken up” by the network. The overall idea is to introduce a new receiver LP-WUR (Low Power – Wake Up Receiver) which will be able to detect a simple signal from the NW (LP-WUS / Low Power – Wake Up Signal). When the signal is detected and decoded to be relevant for the UE in question, the UE will initiate exit sleep mode and initiate normal paging procedure.

For performance requirements the focus is on the actual performance of the receiver, in this case the LP-WUR. As this is a new receiver design, it is imperative that it is tested for performance, so the UE receiver implementation is validated fully.

Due to it being a new receiver design it will likely not be possible to re-use existing test setup already available and used for PDSCH performance requirements in TS38.101-4 [2]. In addition, the standard period between LP-WUS will be too long for running a reliable test where a high number of LP-WUS has to be received. Due to this, a new test setup has to be developed/defined to support conducting performance tests for the LP-WUR.

Due to a new low power wake-up receiver being introduced (LP-WUR), a new test setup will be required for testing the UE LP-WUR performance.

Physical Layer Design

For the physical layer design, it has been decided to cater both On-Off Keying (OOK) as well as IQ receiver. The latter is to enable UEs to support LP-WUS also in the standard receiver in case it has a low power wakeup mode. Different coding configurations of OOK have been discussed and it was agreed to use both OOK-1 and OOK-4. OOK-4 is selected to increase the number of OOK bits withing an OFDM symbol.

To support IQ receivers (standard- or a sequence-based receiver), an OFDM overlay sequence can be added to each segment of the OOK.

Two modulation formats have been decided by RAN1:
- On-Off Keying (OOK) 
- OFDM based

In addition to the LP-WUS it has also been decided to introduce a Low Power Synchronization Signal (LP-SS). The LP-SS will be send periodically and will be used for time and frequency synchronization. As the frequency offset can be up to 20ppm and thereby the time offset as well – the LP-SS usage/ detection will be needed for synchronization too be able to detect the LP-WUS – the LP-SS can be assumed to be tested indirectly.

No separate test of LP-SS is expected as the LP-SS is prerequisite for synchronization in both frequency and time to be able to detect and decode the LP-WUS. However, we see it too soon to completely disregard the need for LP-SS performance requirements.

Receiver types

As previously indicated, for LP-WUR two types of modulation formats are defined:

- OOK receiver supporting OOK-1 and OOK-4

- OFDM receiver utilizing an OFDM overlay sequence.

In the RAN4 RF sessions it has been agreed to define two types of receivers [4]:

Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. - Requirement applicability should be clearly defined. - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

Since there are two types of receivers RAN4 shall study both receiver types and based on the outcome decide if performance requirements are to be defined for both receiver types, i.e., 2 sets of requirements, or if one set of requirements will be sufficiently independent on which receiver type is implemented in the DUT.

It has been agreed to define two LP-WUR types (Type 1 and Type 2) to match receivers capable of receiving the LP-WUS either with On-Off Keying (OOK) or OFDM.

RAN4 to study the performance for both receiver types (Type 1 and Type 2) and further discuss based on simulation results if one or two sets of requirements are to be defined.

If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

Test setup

For introducing a performance test, it will be required to have a way to measure the performance of the new LP-WUR. For existing performance requirements, it has been measurable using the ACK/NACK from the HARQ responses.

For defining UE performance requirements for LP-WUR, it can be considered to do it in a similar way as for PDSCH/PDCCH performance tests, where a continued stream of LP-WUS is transmitted through a pre-defined channel.

For performance tests, where it is purely the performance of the LP-WUR which is to be tested, we see it relevant to having a dedicated test mode enabling the UE to send ACK/NACK kind of responses for the reception of the LP-WUS transmissions. The TE can then compare the number of responses to the actual number of transmissions made and hereby calculate the resulting throughput. In addition, the test mode can enable the UE to always have the LP-WUR active to receive any LP-WUS transmitted from the TE.

Similar to the existing PDSCH testcases, the test can be setup for a specific SNR where the LP-WUR will have to perform to at least a certain percentage of the total number of LP-WUS send. The test could be verifying the MDR (Missed Detection Rate) and FAR (Failed Acceptance Rate) as specified in 3GPP 38.869.V18 sec 6.1.

It is assumed that the UE is tested by RRM tests for a full flow utilizing the PUCCH/PUSCH as response channel, hence it is not a requirement to have the same configuration for UE performance tests, hence a dedicated reporting link to the TE would suffice.

The test and verification of a LP-WUR in RRC_idle will in normal condition take several hours to obtain a valid result within 1% MDR whereas using a dedicated reporting link to the TE will enable a much faster test environment for demodulation performance testing.

In the current test environment, it is not possible to easily define a test setup for doing performance testing of the new LP-WUR, hence it might be required to introduce a specific test mode where a high number of LP-WUS can be transmitted over a short period of time with the possibility for the UE to report back directly when a LP-WUS is received.

The UE report should include if a valid reception of a LP-WUS is found to be dedicated to the UE or not to cover MDR (Missed Detection Ratio) and FAR (Failed Acceptance Ratio).

RAN4 to further discuss how to implement UE performance test to validate the LP-WUR capability of receiving LP-WUS. The test setup must be able to secure information about the number of received LP-WUS as well as how many of the received LP-WUS was dedicated to the DUT.

Performance requirements

LP-WUS/LP-WUR

Under the assumption that a suitable test setup can be agreed, and it will be possible for the UE to inform the TE about the reception of an LP-WUS, the following shall be considered:

- Number of correctly received LP-WUS dedicated to the DUT.

- Number of correctly received LP-WUS not dedicated to the DUT

The TE can then compare the actual transmitted LP-WUS with the UE reported reception.

For the test not to take excessive long time it will need to be configured so LP-WUS is transmitted with a much higher frequency compared to what is expected to be seen in actual deployment. This will of course mean the test does not fully conform to actual deployment, however since the test is for the receiver performance only, we do not see an issue.

An LP-WUR performance test shall include reporting both the total number of received LP-WUS as well as the number of LP-WUS found to be dedicated to the DUT to fully cover the LP-WUR performance.

Further discuss to introduce performance requirements for LP-WUS, if a feasible test setup can be found. Proposed starting point would be for the DUT to report
- Each time the DUT receives an LP-WUS or has received a certain number of LP-WUS.
- If a received LP-WUS is dedicated to the DUT.
Similar to existing PDSCH performance requirements, focus should be to define a “throughput” rate (i.e., MDR and a FAR) for a specific SNR. Note that the pass/ fail criteria defined in 3GPP 38.869 V18 Section 7.1.2 is 1%MDR and 1% FAR.

Due to the nature of the devices in question which are expected to be mostly static and when in sleep on a quite unstable clock generation we have the following proposal for initial analysis:

For test configuration use the following as starting point:
- Frequency offset: Up to 20ppm
- Channel model: TDL-C300, further discuss which doppler settings to be analyzed.

LP-SS (OOK receiver type only)

As we see the main focus to be on the LP-WUR/LP-WUS reception and that it is likely the LP-SS will be indirectly tested by the LP-WUS performance testes, studying possible requirement definition for LP-SS has a lower priority and can be further looked into later if time permits.

Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS.

Conclusion

In this paper we have provided our view on the topic adding of adding UE Demodulation performance requirements for the topic of LPWUS.

We have the following observations and proposals:

Background

- Due to a new low power wake-up receiver being introduced (LP-WUR), a new test setup will be required for testing the UE LP-WUR performance.

Physical Layer Design

Two modulation formats have been decided by RAN1:
- On-Off Keying (OOK) 
- OFDM based

No separate test of LP-SS is expected as the LP-SS is prerequisite for synchronization in both frequency and time to be able to detect and decode the LP-WUS. However, we see it too soon to completely disregard the need for LP-SS performance requirements.

Receiver types

It has been agreed to define two LP-WUR types (Type 1 and Type 2) to match receivers capable of receiving the LP-WUS either with On-Off Keying (OOK) or OFDM.

- RAN4 to study the performance for both receiver types (Type 1 and Type 2) and further discuss based on simulation results if one or two sets of requirements are to be defined.

If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

Test setup

In the current test environment, it is not possible to easily define a test setup for doing performance testing of the new LP-WUR, hence it might be required to introduce a specific test mode where a high number of LP-WUS can be transmitted over a short period of time with the possibility for the UE to report back directly when a LP-WUS is received.

The UE report should include if a valid reception of a LP-WUS is found to be dedicated to the UE or not to cover MDR (Missed Detection Ratio) and FAR (Failed Acceptance Ratio).

RAN4 to further discuss how to implement UE performance test to validate the LP-WUR capability of receiving LP-WUS. The test setup must be able to secure information about the number of received LP-WUS as well as how many of the received LP-WUS was dedicated to the DUT.

Performance requirements

An LP-WUR performance test shall include reporting both the total number of received LP-WUS as well as the number of LP-WUS found to be dedicated to the DUT to fully cover the LP-WUR performance.

Further discuss to introduce performance requirements for LP-WUS, if a feasible test setup can be found. Proposed starting point would be for the DUT to report
- Each time the DUT receives an LP-WUS or has received a certain number of LP-WUS.
- If a received LP-WUS is dedicated to the DUT.
Similar to existing PDSCH performance requirements, focus should be to define a “throughput” rate (i.e., MDR and a FAR) for a specific SNR. Note that the pass/ fail criteria defined in 3GPP 38.869 V18 Section 7.1.2 is 1%MDR and 1% FAR.

For test configuration use the following as starting point:
- Frequency offset: Up to 20ppm
- Channel model: TDL-C300, further discuss which doppler settings to be analyzed.

LP-SS (OOK receiver type only)

Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS.

- References

- RP-241824 - Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- R4-2410569 - WF for [111][136] NR_LPWUS_UERF

- R4-2505234 - WF for [114bis][131] NR_LPWUS_UERF