3GPP TSG-RAN WG4 Meeting #115	R4-2506444

Valletta, Malta, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on UE_BS demodulation performance requirements for 7 MHz CBW

Agenda item:	7.8.6

Document for:	Discussion

Introduction

The topic of adding 7MHz CBW to existing bands n26 and n5 has now reached the point that discussion is needed if performance requirements are to be defined for UE and BS side. The following objective is taken from the WID [1]:

### 4.2 Objective of Performance part WI Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations for irregular channel bandwidths, corresponding to the core requirements: - Specify necessary RRM performance requirements if needed (RAN4) - Specify, if necessary, UE demodulation performance and CSI reporting requirements for the 7 MHz channel bandwidth (RAN4) - Specify, if necessary, BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

As per the WID the focus is on n26 and n5 however the WID does not exclude additional bands having 7MHz CBW added at a later point in time:

NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

In the following we will provide Nokia’s view on the need for introducing UE/BS performance and/or reporting requirements for 7MHz CBW.

Discussion

Background

The addition of 7MHz CBW has been done to support operators request to cover other channel bandwidths including 6, 7, 11, 12 and 13MHz. As a starting point, the bands n5 and n26 has been selected to include 7MHz CBW. N5 and n26 are FDD bands only.

It can be noted that the currently selected bands for 7Mhz CBW both have also 10Mhz CBW included. In addition, there does not seem to be an immediate plan to introduce a band which supports 7MHz only, however it cannot be said for sure, if a 7MHz only band will exist in the future.

There is to our understanding no plans to introduce a 7MHz only band (similar to n106 for 3Mhz), however it cannot be said for sure if a 7MHz only band will exist in the future.

UE Demodulation performance requirements

In previous discussions for introducing 3Mhz CBW it was agreed to not introduce PDSCH performance requirements for 3MHz. The UE and chipset providers confirmed in the online sessions that there were no plans to introduce UEs which only supported 3MHz CBW (n106), hence no PDSCH performance requirements were introduced as also according to chipset and UE vendors there would be no difference in baseband receiver performance if tested with 3MHz CBW or 10MHz CBW

Chipset and UE vendors to confirm the same understanding as for 3MHz that in case a 7MHz only band is defined, no UE is expected to be available supporting the 7MHz band which does not also support 10MHz.

PDSCH Performance requirements

For UE PDSCH performance requirements with the introduction of 7MHz, we see no immediate difference in the test compared to the existing 10MHz CBW tests.

Assuming UEs supporting 7MHz CBW is expected to also support 10MHz CBW, no new PDSCH performance requirements for 7MHz CBW is to be introduced.

PDSCH CA Performance requirements

Here we see the need to update the requirements tables for PDSCH CA to include 7MHz CBW to ensure UE is tested if 7MHz is selected for the CA requirement.

According to the objective in the WID [1], only 15kHz SCS is considered for 7MHz CBW, hence requirements are only needed for 15kHz CBW:

### 4.1 Objective of SI or Core part WI or Testing part WI The core part of this WI includes: - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL

7MHz CBW needs to be included in the PDSCH CA requirements tables. Only 15kHz SCS is required.

Introduce PDSCH CA requirements for 7MHz CBW with 15kHz SCS only.

PDCCH / PBCH performance requirements

There have been no changes introduced from RAN 1 with relation to PDCCH and PBCH with the introduction of 7MHz CBW, hence if a UE is tested for PDCCH / PBCH with 10MHz CBW it would also cover 7MHz CBW performance.

Assuming UEs supporting 7MHz CBW is expected to also support 10MHz CBW, no new PDCCH / PBCH performance requirements for 7MHz CBW are to be introduced.

SDR Performance requirements

In existing SDR requirements the common parameter section in Table 5.5A-1 contains the definition of number of PDCCH candidates and aggregation levels for different SCS/CBW, said table will need to be updated including also the configuration for 7MHz. If the same number of PDCCH candidates are to be kept as for other CBW, the configuration for 7MHz can be similar as to 5MHz, i.e., 2/AL2.

Add the following in TS38.101-4 Table 5.5A-1 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 7MHz”

UE CSI Reporting requirements

CQI/PMI/RI reporting requirements

There have been no changes introduced from RAN1 with relation to CSI reporting with the introduction of 7MHz CBW, hence if a UE is tested with 10MHz CBW it would also cover 7MHz CBW performance.

Assuming UEs supporting 7MHz CBW is expected to also support 10MHz CBW, no new CQI/PMI/RI reporting requirements for 7MHz CBW are to be introduced

BS demodulation performance requirements

PUSCH Performance requirements

For BS PUSCH performance requirements with the introduction of 7MHz, we see no immediate difference in the test compared to the existing 5 or 10MHz CBW tests.

Since 7 MHz is going to be introduced for n5 and n26, which also support 5 and 10 MHz channel bandwidths, Nokia does not see an immediate need to introduce new performance requirements as the demodulation performance will be verified from 5 and 10 MHz.

PUCCH Performance requirements

In Rel-18 Less Than 5 MHz, new PUCCH formats were added due to the fact the reduced bandwidth for PUCCH hopping sets caused overlap and frequency diversity reduction in some channels. For 7MHz, this is not the case, so no performance difference is expected.

Since 7 MHz is going to be introduced for n5 and n26, which also support 5 and 10 MHz channel bandwidths, Nokia does not see an immediate need to introduce new performance requirements as the demodulation performance will be verified from 5 and 10 MHz.

PUCCH frequency hopping will not have reduced performance in 7 MHz.

PRACH Performance requirements

Since 7 MHz is going to be introduced for n5 and n26, which also support 5 and 10 MHz channel bandwidths, Nokia does not see an immediate need to introduce new performance requirements as the demodulation performance will be verified from 5 and 10 MHz.

BS conformance tests

As negligible demodulation performance impact is expected for BS demodulation requirements for 7MHz, it is expected that no new requirements will be needed. Therefore, RAN4 should reuse existing requirements, and introduce an applicability rule for BS Conformance testing for 7 MHz similar to that introduced for 3MHz BW.

RAN4 shall reuse prior BS conformance requirements.

RAN4 shall introduce an applicability rule for BS conformance testing for 7 MHz.

Conclusion

In this paper we have provided our view on the topic of UE and BS demodulation and CSI performance requirements for 7MHz CBW.

The following Observations and Proposals were made:

Background

- There is to our understanding no plans to introduce a 7MHz only band (similar to n106 for 3Mhz), however it cannot be said for sure if a 7MHz only band will exist in the future.

UE Demodulation performance requirements

- Chipset and UE vendors to confirm the same understanding as for 3MHz that in case a 7MHz only band is defined, no UE is expected to be available supporting the 7MHz band which does not also support 10MHz.

PDSCH Performance requirements

Assuming UEs supporting 7MHz CBW is expected to also support 10MHz CBW, no new PDSCH performance requirements for 7MHz CBW is to be introduced.

PDSCH CA Performance requirements

7MHz CBW needs to be included in the PDSCH CA requirements tables. Only 15kHz SCS is required.

Introduce PDSCH CA requirements for 7MHz CBW with 15kHz SCS only.

PDCCH / PBCH performance requirements

Assuming UEs supporting 7MHz CBW is expected to also support 10MHz CBW, no new PDCCH / PBCH performance requirements for 7MHz CBW are to be introduced

SDR Performance requirements

Add the following in TS38.101-4 Table 5.5A-1 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 7MHz”

UE CSI Reporting requirements

CQI/PMI/RI reporting requirements

Assuming UEs supporting 7MHz CBW is expected to also support 10MHz CBW, no new CQI/PMI/RI reporting requirements for 7MHz CBW are to be introduced

BS demodulation performance requirements

PUCCH Performance requirements

PUCCH frequency hopping will not have reduced performance in 7 MHz.

BS conformance tests

RAN4 shall reuse prior BS conformance requirements.

RAN4 shall introduce an applicability rule for BS conformance testing for 7 MHz.

- References

- RP-243323 - New WID on 7 MHz Channel Bandwidth for n26 and n5