3GPP TSG RAN WG4 Meeting #115	R4-2506454

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:			7.7.3

Source:	vivo

Title:	Further discussion on RRM requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the RAN4#114 meeting, RAN4 discussed RRM requirements for low band CA via switching. Following agreements were made and captured in the WF [2].

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirement for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

In the RAN1#120-bis meeting, following agreements for the WI were reached.

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact.

In the previous RAN4 meetings, we provided our views in [2, 3]. In this contribution, we further provide our views on RRM requirements impact due to low band carrier aggregation with switching.

# Discussion

## 2.1 General

SSB assumptions for R19 LB CA via switching

RAN1 made following agreements on switching pattern for low band CA via switching.

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms.

The periodicity of the switching pattern is 40ms. The configuration of duration on each carrier is very flexible, which can be one slot from signalling perspective. Furthermore, it seems there may be multiple switches during one period, which further discussion in RAN1 in needed.

In general, the flexibility of switching pattern configuration complicates the requirements for SDL SCell operation. It is unclear whether SSB/TRS can be available during one period or multiple periods. When SSB periodicity is smaller than switching pattern periodicity, it can be assumed that at least one SSB burst is available during 40ms. Otherwise, it can be assumed that one SSB burst is available in each SSB period. The same assumption can be made for TRS.

In Rel-18 NES WI, the scenario of SSB-less inter-band CA without TRS was deprioritized. TRS was used for inter-band SCell activation. A UE capability for supporting the SSB-less inter-band CA was introduced. Considering the similar assumption on SSB/TRS availability, it is better to support both SSB-based and SSB-less SDL SCell operation.

Proposal 1: Both SSB-based and SSB-less SDL SCell operation is supported.

Proposal 2: For defining RRM requirements for SDL SCell operation, it is assumed at least one SSB/TRS is available either during one switching pattern period or one SSB/TRS period depending on the periodicity of switching pattern and SSB/TRS.

MRTD assumptions for R19 LB CA via switching

- FFS: - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching.

In the RAN4#114 meeting, it was agreed that the receive timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session, where option 1 is 4.67us and option 2 is 3 us. In existing specifications, the TAE for inter-band CA is 3us. Since FDD PCell and SDL SCell is collocated, there will be no additional timing difference observed at UE.

In Rel-18 NES, the RTD is less than CP. It is mainly for 30KHz SCS where the CP is less than 3us. If RTD is assumed as 3us then there will be performance degradation. It means for 30KHz SCS, the alignment at BS side should be better than 3us. In general, the timing alignment at BS side is irrelevant to SCS. There would be no motivation to further relax for 15kHz SCS.

Considering the TAE requirements for inter-band CA and TDD system is 3us, it is straightforward to use 3us as MRTD for co-located inter-band CA.

Proposal 3: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Interruption requirements

RAN1 agreed to introduce RRC configured switching gap.

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA.

The switching gap duration is configured by the NW. The switching gap covers switching period and UE TA. The UE is not scheduled during the switching gap.

In our view, the switching gap should be in symbol level granularity. It serves the same purpose as interruption requirements. With this, there is no need to further define interruption requirements in RRM specification.

Proposal 4: No need to define interruption requirements for carrier switching between FDD CC and SDL CC.

## 2.2 SDL SCell requirements

For SDL carrier operation, L3 measurement for activated/deactivated SCell, SCell activation/deactivation delay, L1 related procedures, e.g., RLM, BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement need to be discussed from RRM requirements perspective.

SDL SCell activation

If the SDL SCell is known, depending on how long ago the SCell is measured, e.g., <2400ms or > 2400ms, the UE needs one or two SSBs for SCell activation. If the SDL SCell is unknown, then very long time is needed for the SCell activation where L1-RSRP measurement need to be conducted for acquisition of QCL and TCI state information.

Thus, to make the SCell activation more efficient, it would be better to assume certain QCL relationship between SSB on the PCell carrier and RS on the SDL SCell carrier. The PCell should be configured as reference cell for the SDL SCell. The RS on SDL carrier is supposed to be QCLed with SSBs on PCell carrier. Then, coarse timing of the SDL SCell always follows the PCell timing. With the assumption, the SDL SCell can always be considered as known from SCell activation perspective. UE needs one SSB or TRS for AGC and one SSB or TRS for fine timing tracking.

From requirements perspective, the SSB-based SDL SCell activation delay requirements can reuse the existing one when the SCell is known that 2 SSBs are needed for AGC adjustment and fine timing tracking. The SSB-less SDL SCell activation delay requirements can reuse the existing one for Rel-18 SSB-less inter-band CA that 2 TRSs are needed for AGC adjustment and fine timing tracking. The side conditions for Rel-18 SSB-less inter-band CA can be further reused in principle for both cases, e.g.,

- The RTD between the SDL SCell and the collocated reference serving cell (FDD PCell) is within 3ms, and

- The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between SSB symbol on the SDL SCell or TRS/A-TRS symbol on the SSB-less SDL SCell and SSB symbol on the reference serving cell, and

- The RS(s) of the SDL SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of the inter-band PCell.

Proposal 5: SDL SCell activation delay requirements for SSB-based SCell operation with FDD PCell as the reference cell is to reuse existing requirements for known SCell where two SSBs are needed.

Proposal 6: SDL SCell activation delay requirements for SSB-less SCell operation with FDD PCell as the reference cell is to reuse existing requirements for Rel-18 SSB-less inter-band CA where two TRSs are needed.

Proposal 7: SDL SCell activation delay requirements for SSB-less SCell operation consider both P-TRS and A-TRS.

Proposal 8: Side conditions for SSB-based SDL SCell activation are

- The RTD between the SDL SCell and the collocated reference serving cell (FDD PCell) is within 3ms, and

- The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between SSB symbol on the SDL SCell and SSB symbol on the reference serving cell

Proposal 9: Side conditions for SSB-less SDL SCell activation are

- The RTD between the SDL SCell and the collocated reference serving cell (FDD PCell) is within 3ms, and

- The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SDL SCell and SSB symbol on the reference serving cell, and

- The RS(s) of the SDL SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of the inter-band PCell.

By further considering the assumption that at least one SSB/TRS is available either during one switching pattern period or one SSB/TRS period depending on the periodicity of switching pattern and SSB/TRS, the RS period Trs in the SCell activation delay is maximum of switching pattern periodicity or RS (SSB/TRS) periodicity.

Proposal 10: The RS period Trs in the SCell activation delay is maximum of switching pattern periodicity and RS (SSB/TRS) periodicity.

L3 measurement on activated SDL SCell

Firstly, there is no mobility on SDL carrier. The FDD PCell and SDL SCell is collocated and both are low band so that the SCell is changed together with PCell. When there is handover from PCell, the SDL SCell is changed accordingly. Secondly, for SSB-less SCell there is no L3 measurements for activated SCell. It can rely on FDD PCell to acquire coarse timing. So, there is no strong motivation to have L3 measurements for SSB-based SCell either.

Proposal 11: No need to define L3 measurement requirements for activated SDL SCell and neighbor cell on the carrier.

L3 measurement on deactivated SDL SCell

Since mobility on SDL carrier is not expected, the L3 measurement on deactivated SDL SCell would be mainly for activation of SCell on SDL carrier, i.e., the SCell can be known for activation if the measurement is performed not long ago.

If the SDL SCell is known, depending on how long ago the SCell is measured, e.g., <2400ms or > 2400ms, the UE may not need to adjust AGC during SCell activation. The benefit is that SCell activation delay can be reduced by one SSB period or switching pattern period. However, the trade-off is UE needs to perform L3 measurement for the deactivated SDL SCell periodically by using gap or causing interruptions. Considering L3 measurements would be much more frequent than SCell activation/deactivation, there should be no gain to perform L3 measurements for the deactivated SCell.

Based on above discussions for SDL SCell activation, the FDD PCell can be used as reference cell for coarse timing acquisition, the SDL SCell can always be considered as known. The L3 measurements on deactivated SDL SCell is unnecessary from this point of view.

Proposal 12: No need to define L3 measurement requirements for deactivated SDL SCell and neighbor cell on the carrier.

L1 measurement for activated SDL SCell

For L1 related measurement and procedures, e.g., RLM, BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR measurement, the requirements may need to be updated to address the unavailability of L1-RSs due to switching between FDD PCell and SDL SCell.

Similar to proposal 2, for defining L1 measurement requirements for SDL SCell operation, it can be assumed at least one L1-RS is available either during one switching pattern period or one L1-RS period depending on the periodicity of switching pattern and SSB/TRS. In other words, during maximum of switching pattern period and L1-RS period, one L1-RS is assumed available for L1 measurements.

For example, when no DRX is configured the SSB-based RLM the out-of-sync evaluation period is Max (100, Ceil (5 x P) x TSSB). It can be updated to Max (100, Ceil (5 x P) x max (Tswitching_pattern_period, TSSB)). Since switching pattern periodicity is 40ms, it may be further simplified by comparing SSB periodicity and 40ms.

Proposal 13: For L1 measurement requirements for SDL SCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity and L1-RS periodicity and DRX cycle if configured.

For the L1 measurements in FR1 sub 1GHz, it may not be necessary to update all the requirements for L1 measurements and procedures. In our view, RLM, L1-RSRP measurement and TCI state switch delay need to be updated.

Proposal 14: L1 measurement requirements for RLM, L1-RSRP measurement and TCI state switch delay are updated for SDL SCell via switching.

## 2.3 FDD PCell requirements

For FDD PCell carrier related operation, all the L3 measurement requirements related to availability of SSB on the carrier need to be discussed. For example, intra-frequency measurement delay may need to be extended due to some measurement occasions are not available because the UE dwells on SDL SCell during the supposed measurement occasion. There are also many L1 measurements (e.g., RLM/BFD/L1-RSRP/L1-SINR)/L1 procedures ((e.g., TCI state switch, PL-RS switch delay etc.) that would be impact due to the switching pattern.

Similar discussions on SDL SCell operation, the requirements for FDD PCell also need to be updated to address the unavailability of SSB for L3 measurements and unavailability of L1-RSs for L1 measurements due to switching between FDD PCell and SDL SCell.

Proposal 15: For L1 measurement requirements for FDD PCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity and L1-RS periodicity and DRX cycle if configured.

Proposal 16: For L3 measurement requirements for FDD PCell operation, the measurement period is based on max of switching pattern periodicity and SSB periodicity and DRX cycle if configured.

Proposal 17: L1 measurement requirements for RLM, L1-RSRP measurement and TCI state switch delay should be updated for FDD PCell via switching.

Proposal 18: Intra-frequency measurement requirements without gap should be updated for FDD PCell via switching.

# Summary

In this contribution, we further provided views on RRM requirements impact due to low band carrier aggregation with switching. Following proposals are present.

Proposal 1: Both SSB-based and SSB-less SDL SCell operation is supported.

Proposal 2: For defining RRM requirements for SDL SCell operation, it is assumed at least one SSB/TRS is available either during one switching pattern period or one SSB/TRS period depending on the periodicity of switching pattern and SSB/TRS.

Proposal 3: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Proposal 4: No need to define interruption requirements for carrier switching between FDD CC and SDL CC.

Proposal 5: SDL SCell activation delay requirements for SSB-based SCell operation with FDD PCell as the reference cell is to reuse existing requirements for known SCell where two SSBs are needed.

Proposal 6: SDL SCell activation delay requirements for SSB-less SCell operation with FDD PCell as the reference cell is to reuse existing requirements for Rel-18 SSB-less inter-band CA where two TRSs are needed.

Proposal 7: SDL SCell activation delay requirements for SSB-less SCell operation consider both P-TRS and A-TRS.

Proposal 8: Side conditions for SSB-based SDL SCell activation are

- The RTD between the SDL SCell and the collocated reference serving cell (FDD PCell) is within 3ms, and

- The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between SSB symbol on the SDL SCell and SSB symbol on the reference serving cell

Proposal 9: Side conditions for SSB-less SDL SCell activation are

- The RTD between the SDL SCell and the collocated reference serving cell (FDD PCell) is within 3ms, and

- The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SDL SCell and SSB symbol on the reference serving cell, and

- The RS(s) of the SDL SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of the inter-band PCell.

Proposal 10: The RS period Trs in the SCell activation delay is maximum of switching pattern periodicity and RS (SSB/TRS) periodicity.

Proposal 11: No need to define L3 measurement requirements for activated SDL SCell and neighbor cell on the carrier.

Proposal 12: No need to define L3 measurement requirements for deactivated SDL SCell and neighbor cell on the carrier.

Proposal 13: For L1 measurement requirements for SDL SCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity and L1-RS periodicity and DRX cycle if configured.

Proposal 14: L1 measurement requirements for RLM, L1-RSRP measurement and TCI state switch delay are updated for SDL SCell via switching.

Proposal 15: For L1 measurement requirements for FDD PCell operation, the evaluation period or measurement delay is based on max of switching pattern periodicity and L1-RS periodicity and DRX cycle if configured.

Proposal 16: For L3 measurement requirements for FDD PCell operation, the measurement period is based on max of switching pattern periodicity and SSB periodicity and DRX cycle if configured.

Proposal 17: L1 measurement requirements for RLM, L1-RSRP measurement and TCI state switch delay should be updated for FDD PCell via switching.

Proposal 18: Intra-frequency measurement requirements without gap should be updated for FDD PCell via switching.

- References

- R4-2504895	WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2500828 Discussion on RRM requirements for low band carrier aggregation via switching, vivo

- R4-2504028 Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2504969	LS of RAN4 agreements on LB CA via switching, Apple