3GPP TSG RAN WG4 Meeting #115	R4-2506456

St Julian’s, Malta, 19th – 23rd May, 2025

Agenda item:			7.30.3

Source:	vivo

Title:	Further discussion on RRM requirements for XR

Document for:	Discussion

# Introduction

In the RAN4#114bis meeting, there were further discussions on RRM requirements impact for XR. Good progress was made during the meeting. Following agreements were captured in the WF [1].

< Agreement>: - Skipping requirements applies for NR-SA <Agreement> - Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping. <Agreement>: - Gaps which shall not be cancelled - MUSIM - Positioning - For other gap types: - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19 - The discussion in RAN4#115 starts with Type 2 MG, then other gap types based on incoming contributions if time allows. <Agreement>: - Not define additional condition for the optional capability of 3 ms. <Conclusion>: The following is the common understanding in RAN4 RRM session: - The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where - X=5ms as baseline and X=3ms as optional UE capability. - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. <Agreement>: Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting. - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [2] was sent to RAN4 to trigger further discussions in RAN4. In the last RAN4 meeting, RAN4 concluded the issue and send the reply LS in [5].

RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation. RAN4 replied the LS in [4] in the previous RAN4 meeting mentioning that RAN4 is discussing other solutions.

In [6, 7, 8, 9], we provided our views on the RRM requirements impact and issues raised in the RAN1 LSs. In this contribution, we further provide our views on RRM requirements impact for XR in Rel-19 and questions raised in the RAN1 LSs.

# RRM requirements impact

## 2.1 Scope

Measurement gap type

In the RAN4#113 meeting, following agreement was made.

<Agreement>: - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

In the last meeting, it was agreed that MUSIM gaps and positioning gaps are not considered for gap skipping. It needs a bit more discussion whether additional gap type is to be considered.

Following type gap types are under discussion.

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

Type 1 MG is the baseline from Rel-15 and it should be supported. It includes both per-UE gap and per-FR gap configured via GapConfig.

The Type-2 MG, however, is not so clear. Whether it is a single gap configured via GapConfig-r17 or it can be multiple gaps. If it is the latter one, then what would be the difference between type-2 MGs and concurrent gaps. If it is the former one, then it should be supported as there is no difference compared to Type-1 MG in terms of gap skipping.

The multiple gaps configure as Type-2 MG is concurrent gaps which are quite complicated. In principle, it should be unnecessary that NW configures concurrent gaps when XR service is enabled, where packet delivery delay is import for the feature. Moreover, there could be complication for NW indicating which gap is to be skipped. When UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than 4 ms.

However, due to time offset for DCI indication, the DCI should be sent 5ms before the gap. If the distance between the gaps occasion is just larger than 4ms, it would be complicated for NW to indicate skipping of the 2nd gap. Thus, there is not much benefit to consider concurrent gaps for gap skipping.

For NCSG, in general there would be no difference compare to Type 1 MG or Type 2 MG from scheme of triggering gap skipping perspective. However, the interruption ratio due to VIL is relatively small, which is 2ms in FR1 and 1.5ms in FR2 within one visible interruption repetition period. In typical measurement gap configuration, 6ms MGL with 40/80ms MGRP would be configured for the UE. The interruption rate due to measurement gap is up to 15%. If 20ms MGRP is considered in FR2, the interruption rate is up to 30%. However, for NSCG the interruption rate can only be up to 7.5% for 20ms MGRP in FR2. Considering the possibility of collision between XR traffic and NCSG, the impact to XR service is very small. Thus, there is no need to consider NCSG when gaps are to be skipped.

Pre-configured measurement gaps are similar to Type-2 MG when it is activated which can be triggered by UE autonomous control or network configuration. When it is not activated, it is the same as no measurement gaps are configured. However, the timeline relationship between pre-MG activation/deactivation and gap skipping indication should be carefully considered.

Proposal 1: Pre-configure measurement gap may be considered.

## 2.2 RRM requirements

In the last meeting, it was agreed to use NR-U-like approach to define requirements for gap skipping when extended measurement delay is needed.

- Follow NR-U-like approach for designing the formulas for measurement delay extension.

How the delay is extended has not been decided yet.

Issue 3-2: Formulas for measurement delay extension <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter Issue 3-3: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter

The difference between option 1 and option 2 is whether the number of skipped occasions should be multiplied by CSSF. In principle, the measurement delay is specified for each inter-frequency layer measured with gaps. With option 1, the same number of gap occasions in which the frequency layer can be measured are given. Thus, regardless of which frequency layer is supposed to be measured in the skipped occasions, same number of additional gap occasions can be used for the measurements so that UE can complete the measurements without compromising accuracy.

Proposal 2: Option 1 is used to define requirements for gap skipping.

For the DRX case, optimization can be considered. Since UE typically measures once in one DRX cycle, it can consider the number of skipped gaps is one per DRX cycle even if more numbers were indicated for skipping.

Proposal 3: Consider only one gap is skipped during one DRX cycle if there is indication for skipping.

Regrading conditions for measurement skipping, following was discussed.

<Way forward>: - Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

Since NR-U like requirements is defined for measurement gap skipping, it is necessary to limit the max number of gaps skipped during the measurement period. Otherwise, the mobility performance cannot be ensured.

Proposal 4: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

# UE assistance information

In the last RAN4 meeting, it was agreed to define following UAI.

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

  - R ∈ {0, 20, 40, 60}%

There are two aspects needing further consideration. One is the time period and another one is about gap types.

For the time period, there should be no problem if no DRX is configured. However, since UE is in general required to measure once during one DRX cycle when DRX is configured, it is ambiguous how to calculate the ratio of the gaps since DRX could be longer than 1s and many gaps are not actually used. One simple way is that the ratio in UAI is regardless of DRX configuration.

For the gap type, there could be ambiguous if per-FR gap is configured. If the ratio is per gap or is summary of the two gaps. Since per-FR gaps can be used for measurement independently, the ratio should be per gap.

Proposal 5: The UAI about ratio of gaps for skipping is per gap if per-FR gap is configured and is regardless of DRX.

Proposal 6: Send an LS to RAN2 if proposal 5 is agreeable.

# Solutions for enabling Tx/Rx in gaps/restrictions

Indication solution

RAN1 concluded solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions as working assumption and an LS [3] was sent to RAN4 for confirmation. RAN4 replied the LS in [4] in the last RAN4 meeting mentioning that RAN4 is discussing other solutions.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

In the RAN1#118 meeting, RAN1 concluded details of DCI-based explicit indication for gap skipping.

Agreement For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”. - FFS: Interpretation of bit value equal to “0”. Agreement A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

For explicit indication by DCI, there are two options how UE would interpret different values for a gap skipping indication. UE only needs to support one option. As the example below, it could mean completely different UE behavior even if the same value is indicated by the DCIs.

Fig 5. Example of explicit DCI indication options

For option 2, UE needs to decide if a gap can be skipped by the latest DCI indication at least 5ms ahead of the start of the gap. For option 1, UE needs to decide if a gap can be skipped by the DCI indication during the gap period and 5ms ahead of the start of the gap. If there is DCI indication with value ‘1’, then the gap shall be skipped. For the example in Fig 5, the two gaps shall be skipped if it is option 1 but it shall not be skipped if it is option 2.

For both options, UE needs to decide if a gap can be skipped by monitoring DCIs contiguously. This complicates UE implementation as the UE needs to re-schedule measurements after a gap skipping. More importantly, it leaves UE less possibility to optimize measurements based on measurement configurations which will degrade mobility performance.

Observation 1: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

However, if gap skipping is explicit indicated by RRC it could give UE flexibility to optimize UE measurement. For some cases, the measurements may not be further delayed depending on measurement configuration and UE implementation.

Multiple options for semi-static schemes were proposed in the last RAN4 meeting.

Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

Firstly, the agreement was to study if there is feasibility issue for RRC based mechanism when the DCI based solution was confirmed in terms of feasibility. It is well known that DCI-based solution complicates UE implementation and leaves no chance for measurement scheduling optimization. RRC based solution can on the other hand be beneficial of UE implementation and measurements optimization.

Secondly, the concern on RRC based mechanism is that it is difficult for NW to decide the gap pattern to be skipped in advance. It may be true. However, even for DCI based solution the NW also needs to indicate the skipped gap in advance due to 5ms time offset for the indication to be effective. It means predication of packet arrival for XR service is needed for DCI based solution. Further considering the periodicity of packet arrival for XR service, there should be no significant difference between the two predications.

Given the complexity of DCI triggering scheme and benefit of RRC based signaling, semi-static solutions should be introduced.

Proposal 7: RAN4 agrees to introduce RRC signaling indication for gap skipping.

Per-FR gap

Regarding skipping indication on frequency domain, following proposals were presented in the last meeting.

- Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

RAN1 made following agreements on DCI indication to skip a gap occasion.

Agreement When measurement gap/restriction is indicated to be skipped by explicit indication by DCI format 1_1/1_2 that does not schedule PDSCH and - if CIF field is present, the scheduled cell for the skipping indication is determined by CIF field. - if CIF field is not present, the scheduled cell for the skipping indication is the scheduling cell. Agreement For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

Based on whether PDSCH is scheduled or not, and whether CIF filed is present or not, the first gap occasion to be skipped is for what serving cells are clear. Eventually, frequency range is also clear based on the determined serving cells.

Proposal 8: RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

# Summary

In this contribution, we further provided views on RRM requirements impact for XR in Rel-19, UAI and triggering schemes. Following proposals and observations are present.

Proposal 1: Pre-configure measurement gap may be considered.

Proposal 2: Option 1 is used to define requirements for gap skipping.

Proposal 3: Consider only one gap is skipped during one DRX cycle if there is indication for skipping.

Proposal 4: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified.

Proposal 5: The UAI about ratio of gaps for skipping is per gap if per-FR gap is configured and is regardless of DRX.

Proposal 6: Send an LS to RAN2 if proposal 5 is agreeable.

Proposal 7: RAN4 agrees to introduce RRC signaling indication for gap skipping.

Proposal 8: RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

Observation 1: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 2: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

Observation 3: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Observation 4: There is UE complexity issue and low efficiency issue for dynamic indication via DCI is used for gap skipping and restriction skipping due to L3 measurements outside gaps.

- References

- R4-2504911	WF on RRM requirements for NR_XR_Ph3, Nokia

- R1-2405736	LS on UE assistance information, RAN1

- R1-2407561	LS on gaps/restrictions that are caused by RRM measurements, RAN1

- R4-2420198	Response LS on gaps/restrictions that are caused by RRM measurements, RAN4

- R4-2504972	Reply LS on UE assistance information, RAN4

- R4-2415198	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2418759	Further discussion on RRM impacts for XR and reply LS, vivo

- R4-2500827	Further discussion on RRM requirements for XR, vivo

- R4-2504027	Further discussion on RRM requirements for XR, vivo

- Appendix: Draft LS on indication solution for gap/restriction skipping

3GPP TSG RAN WG4 Meeting #115	R4-25x

Malta, Malta, 19th – 23rd May, 2025

Title:	LS on indication solution for gap/restriction skipping

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:

E-mail Address:

1. Overall Description:

RAN4 sent a reply LS R4-2420198 in the last RAN4 meeting with following statement.

Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

RAN4 further discussed semi-static indication solutions and made following agreement.

- RRC based indication solution is supported in addition to DCI-based indication solution. It is up to UE which solution is supported.

RAN4 would like to kindly ask RAN2 to introduce necessary RRC signaling. RAN4 will further study how the gap patterns are specified.

Additionally, RAN4 agreed to only support gap skipping in the WI.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to introduce necessary RRC signalling for gap skipping.

3. Reference:

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #115	 	   May 19 – May 23, 2025			Malta